| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Omitting Periodic Ranging for Fixed M2M Devices |  |
| Date Submitted | 2011-09-19 |  |
| Source(s) | Ping-Heng Kuo, Wei-Chieh Huang, Pang-An Ting ITRI Hsi-Pin Ma, Jen-Ming Wu, Yuan-Hao Huang, Yeong-Luh Ueng, Shih-Yu Chang NTHU | E-mail: pinghengkuo@itri.org.tw |
| Re: | Call for contributions on the IEEE P802.16p REV3 AWD |  |
| Abstract | Periodic ranging for fixed devices may be omitted, in order to further optimize the network efficiency of M2M applications. |  |
| Purpose | To be discussed and adopted by 802.16p WG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Omitting Periodic Ranging for Fixed M2M Devices

Ping-Heng Kuo, Wei-Chieh Huang, Pang-An Ting

ITRI

Hsi-Pin Ma, Jen-Ming Wu, Yuan-Hao Huang, Yeong-Luh Ueng, Shih-Yu Chang

NTHU

# Introduction

Many devices in the M2M network are not mobile, their positions are fixed since they are installed. It is anticipatable that the round trip delays (RTD) of these fixed devices are approximately constant over time. Based on such characteristic, we suggest that periodic ranging for fixed devices can be omitted, in order to further optimize the network efficiency of M2M applications.

- Text proposal for inclusion in IEEE 802.16p-11/0021 (802.16Rev3)

--------------------------------- Start of Proposed Text Part 1 ---------------------------------

6.3.X Periodic Ranging

Periodic ranging may be omitted for fixed M2M devices.

--------------------------------- End of Proposed Text Part 1 ---------------------------------