| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Access class for network entry/re-entry of a large number of M2M devices |  |
| Date Submitted | 2011-09-09 |  |
| Source(s) | Lei Zhou,Kaushik Josiam, Hai Wang, Xufeng Zheng, Hyunjeong Kang, Chiwoo Lim, Jaeweon Cho, Rakesh Taori Samsung Electronics | E-mail: l.zhou @samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for contributions from IEEE802.16p WG |  |
| Abstract | This contribution proposes to the scheme on access class restriction from network entry/reentry of a large number of M2M devices |  |
| Purpose | To be discussed 802.16p WG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Priority access for network entry/re-entry of a large number of M2M devices

Lei Zhou,Kaushik Josiam, Hai Wang, Xufeng Zheng, Hyunjeong Kang, Chiwoo Lim, Jaeweon Cho, Rakesh Taori
Samsung Electronics

# Introduction

This contribution proposes to modify mapping and schedule method of access class restriction based on Periodicity of the NS-RCH allocations in IEEE 802.16m.

------------------------------------------------------ Start of Proposed Text -----------------------------------------------------

------------------------------------------------------ Start of Proposed Text1 ----------------------------------------------------

[Remedy1: Add proposed text from line 59 on Page 27 in  IEEE 802.16p-11/0022 with the followings:]

16.2.15. 7 Access class of M2M devices

The ABS may restrict ranging of M2M devices by setting the access class to 1 in the AAI-SCD message. Access class set to 1 restricts M2M devices from performing ranging for network re-entry. The M2M devices may perform network entry/reentry after the BS sets the M2M access class to 0.

Access class restriction in AAI-SCD message may be used for configuring 4 access classes and every access classes can span 4 continuous superframes based on NS-RCH configuration. Access class relative ID of superframe R is expressed as follows:

R=mod（superframe number, N）                              (xxx)

N= M*W is the total number of superframes which all of access class include and is set for 16, M is the total number of access class and is set to 4, W is the number of superframes in one access class and is set to 4.

Allocation of access class can refer to table xxx each of the 4 frames in a superfame individually. The ABS can decide the frame(s) in a superframe which access class that the M2M devices can use for ranging during network re-entry.

Table xxx allocation of access class

| Field | Condition |
| --- | --- |
| Access class (i) | 0≤R≤3 |
| Access class (i+1) | 4≤R≤7 |
| Access class (i+2) | 8≤R≤11 |
| Access class (i+3) | 12≤R ≤15 |

------------------------------------------------------ End of Proposed Text 1-----------------------------------------------------

------------------------------------------------------ Start of Proposed Text 2----------------------------------------------------

[Remedy 2: Add the followings to the Table 708 from line# 47 Page 18 in80216p-10_0018r2 with the followings:]

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ... | ... | ... | ... |
| Access class restriction of frame Access class (i) | 1 | INTEGER (0..1) | Optional |
| Access class restriction of frame Access class (i+1) | 1 | INTEGER (0..1) | Optional |
| Access class restriction of frame Access class (i+2) | 1 | INTEGER (0..1) | Optional |
| Access class restriction of frame Access class (i+3) | 1 | INTEGER (0..1) | Optional |
| ... | ... | ... | ... |

------------------------------------------------------ End of Proposed Text -----------------------------------------------------