| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MGID reassignment procedure in idle mode |  |
| Date Submitted | 2011-09-09 |  |
| Source(s) | Namgi Kim Kyonggi University Soojung Jung, Jaesun Cha, Chulsik Yoon, Kwangjae Lim ETRI | Voice: +82-31-249-9662 E-mail: ngkim@kyonggi.ac.kr Voice: +82-42-860-5456 E-mail: sjjung@etri.re.kr |
| Re: | Comments on IEEE 802.16p-11/0021 AWD [802.16Rev3-based] content |  |
| Abstract | This contribution provides a text for MGID reassignment procedure in IEEE 802.16p-11/0021. |  |
| Purpose | To discuss and adopt the proposed text in IEEE 802.16p-11/0021. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MGID reassignment procedure in idle mode

Namgi Kim

Kyonggi University

Soojung Jung, Jaesun Cha, Chulsik Yoon, Kwangjae Lim

ETRI

# Proposal

According to the sections 6.3.1 in the AWD [1], the IEEE 802.16p system supports M2M Group Identifier (MGID). However, the reassignment procedure of MGID in idle mode is TBD. Therefore, we propose a MGID reassignment procedure in idle mode by location update or network reentry procedure through the RNG-RSP message.

# References

[1] IEEE 802.16p-10/0021, “IEEE 802.16p Machine to Machine (M2M) Amendment Working Document (AWD) [802.16Rev3-based],” Aug. 2011.

# Proposed Text

We differentiated any texts modified by this contribution by using a different color as follows.

- Texts added by this contribution: blue underlined texts.

- Texts deleted by this contribution: red strikeout texts.

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

[Remedy 1: Modify section 6.3.1 as follows;]

### 6.3.1 Addressing and connections

Insert the following texts at the end of subclause 6.3.1

A 15-bit M2M group ID (MGID) uniquely identifies an M2M group in the domain of the network entity that assigns MGID.

An MGID is assigned to a service flow of an M2M device by a network entity after initial network entry through DSA procedure and released during an explicit network exit (e.g., power down location update). The assigned MGID shall be retained by an M2M device even in idle statemode unless the M2M device exits from the network. The MGID can be re-assigned during normal mode and idle mode. During normal mode, the MGID may be changed by DSC procedure. Reassignment procedure during idle mode is TBD. During idle mode, the MGID may be changed by location update or network reentry procedure through the RNG-RSP message.

[Remedy 2: Add the following new texts to the end of section 6.3.2.3.6;]

The following TLV parameter may be included in the RNG-RSP message when the MGID is changed:

MGID Changing Information

Indicates the mapping information of current MGID, current CID, new MGID, and new CID.

[Remedy 3: Modify section 11.6 as follows;]

### 11.6 RNG-RSP management message encodings

Change the contents of Table 688 as indicated

Table 688 – RNG-RSP message encodings

| Name | Type(1byte) | Length | Value | PHY scope |
| --- | --- | --- | --- | --- |
| … | ... | … | ... | ... |
| M2M SMS Response | 45 | 1 | Bits 0-1: accept or reject SMS request 0b0: reject 0b1: accept If (reject) { Bits 2-3: action code 0b00: network re-entry 0b01-0b11: reserved } else { Reserved } Bits 4-7: Reserved | OFDMA |
| Temp CID Timer | 46 | 1 | Life time duration for the Basic CID assigned by BS | OFDMA |
| M2M SMS Confirmation | 47 | 1 | Bit 0: SMS confirmation b0 - NACK b1 -ACK Bits 1-7: Reserved | OFDMA |
| M2M SMS | 48 | Variable | M2M SMS message content up to 140 bytes Padding bits to align boundary of byte. | OFDMA |
| MGID Changing Information | 49 | 8 | Bits 0-14: current MGID Bit 15: padding, will be set to 0 Bits 16-31: current CID Bits 32-46: new MGID Bit 47: padding, will be set to 0 Bits 48-63: new CID | OFDMA |

----------------- End of the text proposal ---------------------------------------------------------------------------------------