| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Authenticated Validity for M2M devices |  |
| Date Submitted | 2011-09-09 |  |
| Source(s) | Eldad Zeira Interdigital Communications LLC | E-mail: eldad.zeira@interdigital.com |
| Re: | Call for Comments for 802.16p AWD: IEEE 802.16p-11/0021 and IEEE 802.16p-11/0022 |  |
| Abstract | The contribution discusses and proposes text to fulfill some of the security related requirements for 802.16p. See also associated slides in S802.16p-11/0251 or its revision. |  |
| Purpose | To be discussed and adopted by TGp M2M |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Authenticated Validity for M2M devices

Eldad Zeira

InterDigital

This contribution is identical to C802.16p-11/0251. It has been uploaded in order that S802.16p-11/0251r1 can be uploaded.

# Introduction

This contribution proposes text to fulfill some of the security requirements for M2M networks in 802.16p. Specifically, the SRD requires that (6.4):

The 802.16p system shall support integrity and authentication of M2M devices, as well as integrity and privacy of M2M application traffic which requires a secure connection.

6.4.1	The 802.16p system shall support a device validity check between the device and the network.

6.4.2	The 802.16p system shall enable a flexible security suite that can be adjusted per the security requirements of the M2M application

# Text Proposals

<<<<<Remedy 1: Text proposal for IEEE 802.16p-11/0022 >>>>>>>>>>>

## Add in 6.2.15 as follows:

f) Perform registration and setup default service flows.

An M2M Device that is capable of self validity check and has failed that test shall not attempt to enter the network.

In network entry, if the AMS cannot attach to the preferred cell, the AMS may choose to perform a network entry without any preference of the BS type, even though the AMS has preference of the BS type in general.

## Add at the bottom of 6.2.15:

- Device validity testing capability information.

<<<<<<<<<<<<<

## Modify 6.2.15.6 as follows:

In the AAI-REG-REQ, the AMS informs the ABS of its capability parameters to be negotiated except those capabilities already negotiated with the ABS through AAI-SBC-REQ/RSP messages. In AAI-REG-RSP, the ABS responds with the accepted capability parameters and with validity acceptance confirmation if validity has been accepted by the network. If the AMS omits some capability parameters in the AAI-REG-REQ, the ABS considers the AMS follows the default values for those parameters and, if acceptable, the ABS may omit those parameters applying default value in its AAI-REG-RSP. If validity has not been accepted, the M2M Device shall exit the network without any further signaling.

## Add the following entries to AAI-REG-REQ (table ?)

| Field | Size (bits) | Value (description) | Condition |
| --- | --- | --- | --- |
| Validity information | TBD | Implementation specific information indicating device validity testing capabilities and results of such test | - |

## Add the following entry to AAI-REG-RSP (table ?)

| Field | Size (bits) | Value (description) | Condition |
| --- | --- | --- | --- |
| Validity acceptance confirmation | 1 | (=1) Indicates that device validity has been accepted by the network | - |

<<<<<Remedy 2: Text proposal for IEEE 802.16p-11/0021 >>>>>>>>>>>

## Add in 6.3.9:

Implementation of phase b), h), i), j) are optional. The MR-BS may instruct the RS to omit phases e), f), g), h), i), j), o) by the RS network entry optimization TLV in the RNG-RSP message.

An M2M Device that is capable of self validity check and has failed that test shall not attempt to enter the network.

## Add at the bottom of 6.3.9:

- Device validity testing capability information (implementation dependent)

## Modify 6.3.9.9:

Registration is the process by which the SS is allowed entry into the network and a managed SS receives its Secondary Management CID and thus becomes manageable. To register with a BS, the SS shall send a REG-REQ message to the BS. The M2M Device includes its device validity information. The BS shall respond with a REG-RSP message including validity acceptance confirmation if validity has been accepted by the network. For an SS that has indicated being a managed SS in the REG-REQ message, the REG-RSP message shall include the Secondary Management CID.

If validity hasn’t been accepted the M2M Device shall exit the network without any further signaling. Figure 79 shows the procedure that shall be followed by the SS.

## Insert in 6.3.2.3.7:

An M2M Device attempting initial network entry shall also include in the RNG-REQ message the device validity information.

## Insert in 11.7.8:

11.7.8.12 Validity information

Validity information is implementation specific information indicating device validity testing capabilities and optionally results of such test.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| TBD | TBD | Implementation specific | RNG-REQ |

## Insert in 6.3.2.3.8:

When responding to M2M Device initial network entry the BS shall include in the RNG-RSP message a validity acceptance field.

## Insert in 11.7.8:

11.7.8.13 Validity acceptance confirmation

Validity acceptance confirmation signifies the acceptance by the network of the validity information provided in RNG-REQ.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| TBD | 1 bit | (=1) Indicates that device validity has been accepted by the network | REG-RSP |