| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | M2M group control signaling scheme in 802.16m |  |
| Date Submitted | 2011-09-09 |  |
| Source(s) | Rui Huang, Honggang Li, Shantidev Mohanty Intel Corporation | E-mail: rui.huang@intel.com |
| Re: | RE: Call for comments on the 802.16p AWD |  |
| Abstract | This contribution proposes the M2M group control signaling based on the 802.16m. |  |
| Purpose | For review and adoption into 802.16p AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

M2M group control signaling scheme in 802.16m

Rui Huang, Honggang Li, Shantidev Mohanty

Intel Corporation

# Introduction

In [1], several typical M2M group identifier (MGID) functionalities for IEEE 802.16m system are discussed. In this contribution, we will address one of typical usages of MGID:  M2M group control signaling.

# M2M group control signaling in 802.16m

As one of typical usage of MGID, M2M group control signaling could benefit to reduce the massive control signaling due to a large number of M2M devices.

In Figure 1, the detail M2M group control signaling method is shown. These control signaling include two parts:

{Common messages for all M2M devices in a group, Dedicated messages for the individual M2M devices}

Figure 1. M2M Group control signaling in 802.16m

## Common messages part in M2M Group Control signaling

These common control messages could be shared by all M2M devices in a same M2M group. It is a most simple and efficient way to reduce the control signaling overhead. For example, ideally the overhead for a M2M group including K active M2M devices will be reduced as 1/ K.

And these common messages are depended on the M2M grouping criteria. For example, if we grouped M2M devices according to their SDU size and approximated CQI, these messages could be MCS and power control.

## Dedicated messages part in M2M Group Control signaling

Dedicated messages could be used for the resource allocation mainly. There are several resource allocation mechanisms available in the current 802.16m system: dynamic scheduling and persistent scheduling.

### Dynamic scheduling

With this approach, each data packets will be scheduled by MAC/PHY control signaling, e.g. Assignment A-AMAP in 802.16m by which the system could schedule the data every subframe. It is most flexible and could achieve maximum scheduling gain thanks to frequency and time domain diversity but with too high signaling overhead. If this method is applied to M2M devices’ traffic, the overhead is too large to be acceptable as shown in the table below.

| DL A-AMAP size | 33bits |  |
| --- | --- | --- |
| M2M data packet size | Home meter: 8*8 | Home security: 30*8 |
| Control singling overhead | 34% | 12% |

### Persistent scheduling

Persistent allocation is a technique used to reduce assignment overhead for connections with periodic traffic pattern and with relatively fixed payload size.

However, for M2M device, its data traffic is so small that could be transmitted in one duration and the data packets from same users will not be happened in the next interval periodly. That will result in some resource wastage also.

Figure 2. Persistent scheduling for M2M data packets

### M2M group scheduling

As mentioned above, the current scheduling mechanism is not optimized for M2M data. Fortunately, since M2M services have their unique characters in comparison with the human interaction services, e.g.

- Small data transmission

- Infrequent transmission

- Time controlled: the data could be transmitted in a predefined interval

Therefore, it is possible to design an efficient method to reduce the overhead of M2M device resource allocation. For example, the multiple M2M devices data burst could be bundled based on this M2M grouping mechanism, the control signaling overhead (e.g. the resource allocation) for M2M devices could be significantly reduced. At same time because the bundled packet size may be too large to be scheduled in a subframe which is the basic scheduling period, we need predefine a scheduling pattern in multiple subframes or frames to reduce the further overhead as well as the persistent scheduling in 16m.

Figure 3. Scheduling Pattern for M2M group

Step1.Multiple M2M devices will be grouped as “M2M group” firstly.  That is the M2M group MGi is composed of the multiple M2M devicesM2M1,M2M2,...,M2MK, where K is the number of M2M devices in a group.

Step2. BS will allocate a series of resource to a specific M2M group persistently. Actually BS could schedule these resources only for part of devices inMGi. The scheduled devices list in the duration when this group scheduling message is valid is M2Mm1,M2Mm2,...,M2MmQ indeed.

And the scheduling activated duration could be denoted as:

Lschedulingactive=P∗Irepeat

Where P=ceil(totalpacketsizepacketsizeperperiod),P<=K

Irepeat is the scheduling interval, e.g. L frames/subframes in Figure 2.

The scheduled devices index could be indicated by M2MID_Scheduling message . The scheduler will manage this M2MID_Scheduling message also.

Figure 4. Scheduled devices index mapping to M2M_ID

The detail resource allocation will be up to the packet size of the scheduled devices. We will give several  solutions for the specific resource index mapping below.

- Option A

And if the data packet size of each M2M devices is different, they will be given explicitly as shown in Figure5a.

Figure 5a Resource index mapping scheme

- Option B

And if the data packet size of each M2M devices is similar, they will be given by the differential way as shown in Figure5b. This method will reduce the signaling overhead without any limitation on the flexibility.

Figure 5b Resource index mapping scheme

- Option C

And if the data packet size of each M2M devices is exactly same, they will be represented by one as shown in Figure5c. This method will reduce the signaling overhead with some drawback on the flexibility.

Figure 5c Resource index mapping scheme

In order to reduce the system complexity we propose use option c) as the data resource scheduling mechanism for M2M devices in a M2M group. And in order to support the more flexible scheduling algorithm in the system, the dynamic scheduling mechanism will override this persistent scheduling message also.

Step3.The M2M devices in a M2M group will transmit or receive their data packets one by one according to the resource allocation message defined in Step2.

# Conclusion

In this contribution, we proposed an efficient control signaling mechanism for M2M group which will reduce the signaling overhead significantly.

# Text Proposal

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

Proposal #1 : Insert the following text in Section6.2.  in the latest 16p AWD

6.2.x  M2M Group Control Message

M2M group control message mechanism will be used to convey two types control messages by Broadcast Assignment A-MAP IE:

1. Common message for all devices in a same group, e.g.MCS, power control

2. UE specific message , e.g. resource allocation for multiple M2M devices in a M2M group

Proposal #2 : Revised the following text in Section16.3.5.5.2.4  in the latest 16p AWD

6.3.5.5.2.4.13 Broadcast Assignment A-MAP IE

….

Table 866 – Broadcast Assignment A-MAP IE

| Field | Size(bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Broadcast_Assignment_A-MAP_IE(). |  |  |  |
| … |  |  |  |
| Else{//Function_Index == 0b11 |  |  |  |
| MGID | 15 |  |  |
| Number_Scheduling_M2M | 4 | Number of the schdueling M2M devices in this group |  |
| Scheduling_M2MID | 2^4*TBD |  |  |
| Burst_Size | 8 | The burst size per devices in bytes |  |
| Resource_start_index | 11 |  |  |
| Allocation Period | 2 | Period of group persistent scheduling in frames |  |
|  |  |  | . |
| Reserved | 1 |  |  |
| } |  |  |  |

---------------------------End Text Proposal----------------------------------

# References

- IEEE C802.16p-11/0152

- IEEE C80216p-11/0155

- IEEE 802.16e2009

- IEEE 802.16mD12