| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for M2M adaptive initial backoff window scheme in 802.16m |  |
| Date Submitted | 2011-09-12 |  |
| Source(s) | Rui Huang, Honggang Li, Shantidev Mohanty Intel Corporation | E-mail: rui.huang@intel.com |
| Re: | RE: Call for comments on the 802.16p AWD |  |
| Abstract | This contribution proposes the M2M group control signaling based on the 802.16m. |  |
| Purpose | For review and adoption into 802.16p AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for M2M adaptive initial backoff window in 802.16m

Rui Huang, Honggang Li, Shantidev Mohanty

Intel Corporation

# Introduction

In the previous sessions, some ranging optimizations to reduce the ranging collision probability due to the large number of M2M devices are discussed [1][2]. In these contributions the ranging opportunities should be extended in the time domain. However, several key issues of such schemes are not addressed yet, for instance, how about the standard impaction due to these new mechanisms and how to adjust the appropriate ranging distribution window.

In this contribution, we will propose an adaptive initial backoff window scheme which could improve the successful access probability for M2M device with the minimum standard impaction. The other similar scheme for 802.16e is presented in [3] also.

# Adaptive initial backoff window scheme for M2M

The mechanism which enlarges the access window is the most effective way to reduce the random access collision. Therefore, such similar mechanisms ,which extend the devices accessing window, were proposed in 802.16p to avoid congestion from large number of devices accessing the network in a short interval of time[1][2].

However, there are several critical issues for these schemes open.

- Is the new configuration parameters needed or not to be defined in the current spec, e.g. Minmum access window[1] and Ranging Waiting Timer[2]?

- How to specify an appropriate time window size without huge the access delay performance degradation?

In the following subsections, we will present our adaptive initial backoff window scheme regarding to these issues above.

## Standard Impaction

According to [1][2], the new parameters should be added to the current spec to support the proposed mechanisms. But the backoff mechanism, which is most effective way to reduce the random access collision [4], could also be applied when there are too many M2M devices.

In Fig1, we give the simulation results when the initial backoff window size increasing based on EMD[5]

- Number of devices = 12000 in 60s with poissson distribution,

- 1 ranging opportunity per frame

- Initial backoff window size = 2 to 2^10

- Max backoff window size = 65536

- Max number of retries = 8

- T3 timer set to 60 ms

- 0% misdetection and false alarm probability

The results shows that

- When initial backoff window size S = 2, the successful access ratio r will be <0.05.

- When S=2^10,  r>0.99.

Figure 1: Success ratio when different initial backoff window size

Proposal #1: the initial backoff window scheme in current spec[4] could effectively reduce the collision probability without any additional new parameters definition.

## How to estimate an appropriate initial backoff window size

Theoretically the access collision probability is highly related to the number of access devices in a distributed duration. It means that if there are more M2M devices which attempt to access the network, the larger initial backoff window needed to reduce the collision probability. However, BS could not know the exact devices number because some of them will transmit a same ranging code. As result, it is impossible for BS to specify this initial backoff window by the number of access devices.

On the other hand, BS could estimate the ranging channel usage percentage easily. So we can define this ranging channel using percentage as the accesses loading estimation metricλ :

λ(t)=α∗M(t)N(t)

Where M(t)is the total number of ranging channels be using in an access window t.

N(t)is the total number of available ranging channels in an access window t.

α is the weight coefficient which is related to some other long term access statistics aspects, e.g. the successful preamble detection capability.

And the average accesses loading estimation metricλavg :

λavg=mean(λ(t))t∈[t0+1,...,t0+T]

In Fig2, we give the successful access ratio simulation results when λavg is vary from 0 to >1.

Figure 2. successful access ratio simulation results

The detail working flow of this optimized method is given below.

Figure 3. Adaptive Initial backoff window scheme procedure

Step1. BS will broadcast the random access resource configuration, e.g. random access code set, initial backoff  window size which is based on the previous loading estimation, and so on.

Step 2. BS estimates the instant access loading in the current access window : λ(t)=α∗M(t)N(t). And such estimation works will be implemented by BS only. BS needed not any feedback from the accessing devices.

Step 3. BS collects the access loading in the duration Tk to get the average access loading during T

λavg(Tk)=mean(λ(t)),t∈[t0,...,t0+Tk],k=0,1,...,

Step4. BS will adjust the random access parameters for Tk+1based on λavg(Tk) and the expected successful access ratio rtarget(Tk+1)

For example, base on 16p access EMD simulation results in Fig2, if λavg(Tk)=0.28 ,rtarget(Tk+1)=80% and the current inibackoff window size s(k)=1,the proposed initial backoff window size for next access window s(k+1)should be 64 for the next access duration.

Figure4. initial backoff window size selection example

Step 5. BS will indicate the modified random access parameters in Step4 in the corresponding messages.

According to the procedure described above, we can adaptively modify the random access resource based on the network access traffic loading semi-statically. That will be benefit to improve the overall successful access ratio when the loading is high. And at same time it could guarantee the lower access delay.

# Conclusion

In this contribution, we present a feasible solution to resolve the high random access collision probability problems raised for M2M device: an adaptive initial backoff window scheme. And the detail procedures, e.g. how to configure the appropriate size are also described.

# Text Proposal

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

Proposal #1 : Revised the following text in Section6.2.15  in the latest 16p AWD[6]

6.2.15.3 Contention-based initial ranging and automatic adjustments

....

When the ranging channel are used for M2M device and non-M2M devices, the inital backoff window size should be modified according the previous ranging loading estimation. The subsequent ranging could acheive higher successful probability with the adaptive inital backoff window.

Proposal #2 : Insert the following text in Section6.2.3  in the latest 16p AWD

6.2.3.23 AAI-PAG-ADV (paging advertisement) message

Table 706—AAI-PAG-ADV message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ... | ... | ... |  |
| For(i=0;i<Num_MGID;i++){ |  |  |  |
| MGID |  |  |  |
| .... |  |  |  |
| If (Action Code== 0b00 ) { |  |  |  |
| ... |  |  |  |
| Initial backoff window size | 4 | the new initial backoff window size | the initial backoff window given by BS according to: - ranging loading estimation or - paging type: M2M group paging enabled[7] |
| } |  |  |  |
| } |  |  |  |
| ... | ... | ... |  |

6.2.3.31 AAI-System Configuration Descriptor (SCD) Message

Table 708 - AAI-SCD Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ... | ... | ... |  |
| If ((M2M ranging indicator == 0b01 ) or (M2M ranging indicator == 0b10 ) { |  |  |  |
| ... |  |  |  |
| Initial backoff window size | 4 | the new initial backoff window size | the initial backoff window given by BS according to: - ranging loading estimation - paging type: M2M group paging enabled[7] |
| } |  |  |  |
| ... | ... | ... |  |

…

---------------------------End Text Proposal----------------------------------

# References

- IEEE C802.16p-11/0146, Huawei

- IEEE C802.16p-11/0190, LGE

- IEEE C80216p-11/0254r1, Intel

- IEEE 802.16mD12

- IEEE C802.16p-11/0126 “Evaluation Guideline for Comparison of Network Entry Solutions”, produced by the 802.16 WG, M2M TG, Network Entry ad-hoc group

- IEEE C802.16p-11/0022, IEEE 802.16.1/D0-based

- IEEE C80216p-11/0255r1, Intel