| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for M2M initial backoff window scheme in 802.16e |  |
| Date Submitted | 2011-09-11 |  |
| Source(s) | Rui Huang, Honggang Li, Shantidev Mohanty Intel Corporation | E-mail: rui.huang@intel.com |
| Re: | RE: Call for comments on the 802.16p AWD |  |
| Abstract | This contribution proposes the M2M group control signaling based on the 802.16m. |  |
| Purpose | For review and adoption into 802.16p AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for M2M adaptive initial backoff window in 802.16e

Rui Huang, Honggang Li, Shantidev Mohanty

Intel Corporation

# Introduction

In [1] we present an adaptive initial backoff window scheme which could improve the successful access probability for M2M device and at same time minimize the access delay in 802.16m. The similar scheme could be applied in 802.16e also.

Figure 1. Adaptive Initial backoff window scheme procedure

# Text Proposal

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

Proposal #1 : Insert the following text in Section6.3.9.5.1  in the latest 16p AWD[2]

6.3.9.5.1 Contention-based initial ranging and automatic adjustments

....

When the ranging channel are used for M2M device and non-M2M devices, the inital backoff window size should be modified according the previous ranging loading estimation. The subsequent ranging could acheive higher successful probability with the adaptive inital backoff window.

Proposal #2 : Insert the following text in Section16.2.3  in the latest 16p AWD[2]

6.3.2.3.51 MOB_PAG-ADV (BS broadcast paging) message

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MOB_PAG-ADV_Message_format() { | — | — |
| Management Message Type = 61 | 8 | — |
| Num_Paging_Group_IDs | 8 | Number of Paging Group IDs in this message |
| for (i = 0; i < Num_Paging_Group_IDs; i++) { | — | — |
| Paging Group ID | 16 | — |
| } | — | — |
| Num_MACs | 8 | Number of MS MAC addresses |
| For (j = 0; j < Num_MACs; j++) { | — | — |
| MS MAC Address hash | 24 | The hash is obtained by computing a CRC24 on the MS 48-bit MAC address. The polynomial for the calculation is 0x1864CFB |
| Action Code | 2 | Paging action instruction to MS 0b00 = No action required 0b01 = Perform ranging to establish location and acknowledge message 0b10 = Enter network 0b11 = Reserved |
| … |  |  |
| } |  |  |
| Padding | variable | Padding bits to ensure octet aligned |
| TLV Encoded Information | variable | TLV-specific |
| } | — | — |

….

For OFDMA PHY, the following TLV may be included in the MOB_PAG-ADV management message. If a CDMA code assignment TLV is included, the Page-Response window TLV shall be included. There shall be no more than one occurrence of the Page-Response window TLV.

…..

CDMA code and transmission opportunity assignment for M2M

The CDMA Code and Transmission Opportunity Assignment field indicates the assigned code and transmission opportunity for an M2M device that is paged to use over dedicated CDMA ranging region

Proposal #3 : Insert the following text in Section11.17.3  in the latest 16p AWD[2]

11.17.3 CDMA code and transmission opportunity assignment for M2M

| Type | Length(bits) | Value | Scope |
| --- | --- | --- | --- |
| 153 | Variable | Bits 0-7: Num_group, Number of M2M groups for (i = 0; i < Num_group; i++) { MGID; Initial backoff window size .... } | OFDMA |

---------------------------End Text Proposal----------------------------------

# References

- IEEE C802.16p-11/0253r1, Intel

- IEEE C802.16p-11/0021, IEEE 802.16Rev3/D0-based