| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for M2M group paging in 802.16m |  |
| Date Submitted | 2011-09-11 |  |
| Source(s) | Rui Huang, Honggang Li, Shantidev Mohanty Intel Corporation | E-mail: rui.huang@intel.com |
| Re: | RE: Call for comments on the 802.16p AWD |  |
| Abstract | This contribution proposes the M2M group control signaling based on the 802.16m. |  |
| Purpose | For review and adoption into 802.16p AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for M2M group paging in 802.16m

Rui Huang, Honggang Li, Shantidev Mohanty

Intel Corporation

# Introduction

In current 16p AWD[1], group paging may be used for M2M devices in order to reduce the paging message overhead. But on the other hand, this mechanism will lead some other issues, e.g. the large number of simultaneous ranging after a group of M2M devices are paged to prepare to access the network.

Therefore, in this contribution we will propose an optimized scheme which could reduce the collision probability when M2M group paging happened.

# Proposed network entry scheme for M2M group paging

As described in Fig 1 below, we can find one of main issues for this M2M group paging method is:

- When the devices in a same group which was paged by M2M group paging message are initiated to access the network in a short interval, it will increase the access collision probability.

Figure 1: Key issue of group paging mechanism

One of straightforward way to resolve this issue is reducing the devices number of a M2M group. But this method is inconsistent with the goal of group paging mechanism, which is control signaling optimization.

Regarding to minimum standard impaction and optimize the access delay performance for other non-M2M group devices further, we can propose one optimized method by which a dedicated initial backoff window will be provided for the devices in the paged M2M group only. The detail working flow of this optimized method is given below.

Figure 2. Dedicated Initial backoff window for M2M group paging devices

Step1.  BS will broadcast the random access resource configuration for non-M2M group paging devices, e.g. random access code set, initial backoff window size(s1), and so on.

Then the non-M2M group devices will transmit their random access request to BS in a specified initial backoff windom(s1).

Step 2. BS will broadcast other random access configuration for M2M group paging devices if there are some group paging happened.

Then  MSs with M2M group paging will transmit their random access request to BS in an specified initial backoff windom(s2, S2 is larger than S1).

According to the procedure described above, we can adaptively modify the random access resource based on M2M group paging. That will be benefit to improve the overall successful access ratio when there is M2M group paging. And at same time it could guarantee the lower access delay for the non-M2M group paging devices also.

# Text Proposal

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

Proposal #1 : Insert the following text in Section6.2.15  in the latest 16p AWD[6]

6.2.15.3 Contention-based initial ranging and automatic adjustments

....

When the ranging channel are used for M2M device and non-M2M devices, the specific inital backoff window size should be updated for M2M devices immediately  when M2M group paging happened.

Proposal #2 : Insert the following text in Section6.2.3  in the latest 16p AWD

6.2.3.23 AAI-PAG-ADV (paging advertisement) message

Table 706—AAI-PAG-ADV message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ... | ... | ... |  |
| For(i=0;i<Num_MGID;i++){ |  |  |  |
| MGID |  |  |  |
| .... |  |  |  |
| If (Action Code== 0b00 ) { |  |  |  |
| ... |  |  |  |
| Initial backoff window size | 4 | the new initial backoff window size | the initial backoff window given by BS according to: - ranging loading estimation or - paging type: M2M group paging enabled[7] |
| } |  |  |  |
| } |  |  |  |
| ... | ... | ... |  |

6.2.3.31 AAI-System Configuration Descriptor (SCD) Message

Table 708 - AAI-SCD Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ... | ... | ... |  |
| If ((M2M ranging indicator == 0b01 ) or (M2M ranging indicator == 0b10 ) { |  |  |  |
| ... |  |  |  |
| Initial backoff window size | 4 | the new initial backoff window size | the initial backoff window given by BS according to: - ranging loading estimation - paging type: M2M group paging enabled |
| } |  |  |  |
| ... | ... | ... |  |

…

---------------------------End Text Proposal----------------------------------

# References

- IEEE C802.16p-11/0022, IEEE 802.16.1/D0-based

- IEEE 802.16e2009

- IEEE 802.16mD12