| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for M2M group paging in 802.16e |  |
| Date Submitted | 2011-09-11 |  |
| Source(s) | Rui Huang, Honggang Li, Shantidev Mohanty Intel Corporation | E-mail: rui.huang@intel.com |
| Re: | RE: Call for comments on the 802.16p AWD |  |
| Abstract | This contribution proposes the M2M group control signaling based on the 802.16m. |  |
| Purpose | For review and adoption into 802.16p AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for M2M group paging in 802.16e

Rui Huang, Honggang Li, Shantidev Mohanty

Intel Corporation

# Introduction

In [1] we present an optimized initial backoff window scheme to resolve the issues raised by the group paging in 802.16m. In this contribution, we will propose the similar scheme for M2M devices in 16e also.

# Text Proposal

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

Proposal #1 : Revised the following text in Section6.2.15  in the latest 16p AWD[2]

6.2.15.3 Contention-based initial ranging and automatic adjustments

....

When the ranging channel are used for M2M device and non-M2M devices, the inital backoff window size should be used for M2M group paging devices also.

.

---------------------------End Text Proposal----------------------------------

# References

- IEEE C802.16p-11/0255

- IEEE 802.16p-11/0021

- IEEE 802.16e2009

- IEEE 802.16mD12