| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | M2M fast network re-entry in 16m |  |
| Date Submitted | 2011-09-10 |  |
| Source(s) | Honggang Li, Rui Huang Shantidev Mohanty Intel | Email: Honggang.li@intel.com |
| Re: | 802.16p amendment texts |  |
| Abstract |  |  |
| Purpose | Discuss and adopt proposed texts |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

M2M fast network re-entry in 16m

Hongang Li, Rui Huang, Shantidev Mohanty

Intel

# Introduction

The network re-entry process is used for MS back to the connected mode from the idle mode, after network re-entry, the MS can request the transport channel for data transmission. For M2M application, small packet transmission is very usual and upon sending it out, the device will back to sleep state immediately to save power, so the normal process of network re-entry and bandwidth request is no longer an efficient way.

A fast network reentry procedure is proposed to optimize the network reentry and bandwidth request process for M2M short packet transmission.

# Proposal

# Proposed texts

3.1 Proposed Text #1

[Add the following text after line 12 in page 30.]

----------------------------------------------  Text Start -----------------------------------------------

6.2.18.7.2 Network re-entry from idle mode for M2M devices

----------------------------------------------  Text End -----------------------------------------------

3.2 Proposed Text #2

[Add the following text in table 678 in page 6.]

----------------------------------------------  Text Start -----------------------------------------------

Table 684—AAI-RNG-REQ message Field Description

----------------------------------------------  Text End -----------------------------------------------

3.3 Proposed Text #3

[Add the following text in table 685 in page 8.]

----------------------------------------------  Text Start -----------------------------------------------

Table 685—AAI-RNG-RSP message Field Description

----------------------------------------------  Text End -----------------------------------------------