| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | M2M network re-entry combined with bandwidth request in 16m |  |
| Date Submitted | 2011-09-10 |  |
| Source(s) | Honggang Li, Rui Huang Shantidev Mohanty Intel Jaesun Cha ETRI Hyunjeong Kang Samsung | Email: Honggang.li@intel.com |
| Re: | 802.16p amendment texts |  |
| Abstract |  |  |
| Purpose | Discuss and adopt proposed texts |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

M2M network re-entry combined with bandwidth request in 16m

Hongang Li, Rui Huang, Shantidev Mohanty

Intel

Jaesun Cha

ETRI

Hyunjeong Kang

Samsung

# Introduction

The network re-entry process is used for MS back to the connected mode from the idle mode, after network re-entry, the MS can request the transport channel for data transmission. For M2M application, small packet transmission is very usual and upon sending it out, the device will back to sleep state immediately to save power, so the normal process of network re-entry and bandwidth request is no longer an efficient way.

A network reentry procedure combined with bandwidth request is proposed to optimize the network reentry and bandwidth request process for M2M short packet transmission, by embedding the bandwidth request in the AAI-RNG-REQ message.

# Proposal

In the optimized procedure as shown in fig 1, the M2M device can carry the bandwidth request in AAI-RNG-REQ message and the BS can acknowledge if the bandwidth request is accepted in AAI-RNG-RSP message and if accepted, allocate the channel in UL AMAP message according to the request from the M2M device, which can save the two random access procedures (one for network reentry and one for bandwidth request) to one procedure. The saved random channel resource can be used for more contention-based channel access by M2M devices.

Fig 1, M2M network reentry with bandwidth request

# Proposed texts

3.1 Proposed Text #1

[Add the following text after line 12 in page 30.]

----------------------------------------------  Text Start -----------------------------------------------

6.2.18.7.2 Network re-entry from idle mode for M2M devices

…

During network reentry, the M2M device may request UL BW without a contention-based bandwidth request by including Bandwidth Request in an AAI-RNG-REQ message. If an ABS receives the AAI-RNG-REQ message with M2M Bandwidth Request Indicator set to 1 and bandwidth request size, the ABS may accept or reject the bandwidth request in AAI-RNG-RSP message. If accept the bandwidth request, the ABS will allocate UL bandwidth for UL transmission after the M2M device is back to the connected mode.

----------------------------------------------  Text End -----------------------------------------------

3.2 Proposed Text #2

[Add the following text in table 678 in page 6.]

----------------------------------------------  Text Start -----------------------------------------------

Table 684—AAI-RNG-REQ message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ... | ... | ... |  |
| M2M BR Indicator | 1 | 0: No bandwidth request 1: bandwidth request | Optional |
| If(M2M BR Indicator==1){ |  |  |  |
| BR size | 19 | Amount of bandwidth requested in bytes |  |
| } |  |  |  |
| ... | ... | ... |  |

----------------------------------------------  Text End -----------------------------------------------

3.3 Proposed Text #3

[Add the following text in table 685 in page 8.]

----------------------------------------------  Text Start -----------------------------------------------

Table 685—AAI-RNG-RSP message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ... | ... | ... |  |
| M2M BR ACK | 1 | 0b0: accept bandwidth request 0b1: reject bandwith request | Shall be included if M2M BR indicator is included in AAI-RNG-REQ when M2M device is attempting network reentry from idle mode |
| ... | ... | ... |  |

----------------------------------------------  Text End -----------------------------------------------