| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Long cycle persistent scheduling for periodic M2M transmissions |  |
| Date Submitted | 2011-09-10 |  |
| Source(s) | Honggang Li, Rui Huang Shantidev Mohanty Intel | Email: Honggang.li@intel.com |
| Re: | Call for comments on the 16p AWD |  |
| Abstract | This contribution proposes texts about UL MIMO feedback for fixed M2M devices |  |
| Purpose | For discussion in 802.16p TG and adoption in to the 802.16p AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Long cycle persistent scheduling for periodic M2M transmissions

Hongang Li, Rui Huang, Shantidev Mohanty

Intel

# Introduction

For M2M applications, there are lots of cases that the M2M devices are fixed deployed and have small packets to send periodically, e.g. the electricity meters, temperature sensors, medical monitors/sensors etc.

For such kind of fixed M2M devices with periodic small packets transmission, the transmission pattern is very much alike periodic VoIP stream but in a much longer interval between two consecutive packets, so a long-cycle persistent scheduling for M2M transmission is proposed to meet such a requirement.

# Proposal

The long-cycle persistent scheduling process as follows:

- When an M2M device starts to work in connected mode after network entry and registration, it will notify the service flow characteristics to the BS.

- If the BS gets to know there will be periodic traffic pattern, she will pre-allocate the channel to the M2M device in UL persistent allocation A-MAP IE, with the channel allocation information for the data traffic and/or UL synchronization, and the allocation period of validity.

- The M2M device can switch to the idle mode for power saving

- When there is M2M data to be sent, if UL synchronization is lost, the M2M device will go through ranging process or other UL synchronization process to capture the UL synchronization and transmission paramters using pre-allocated UL synchronization channel, then

- Send the data in pre-allocated data channel, and back to the idle mode when ACK from the BS is received or other termination condition is met.

- If there is a bursty M2M data to be sent where there is no pre-allocated channel for it, the M2M device can use the random channel to send the data or back to the connected mode for data transmission.

The BS can de-allocate or reallocate the persistent scheduling for M2M service by UL persistent allocation A-MAP IE, if M2M device in idle mode, the BS can page it back to the connected mode and listen to the UL persistent allocation A-MAP IE for the de-allocation/re-allocation.

Figure 1, long cycle persistent scheduling for periodic M2M transmissions

# Text proposal

---- Start of proposed text ----

[#1: Add the Section 6.2.7 and texts in C802.16p-11/0022 after line 25 page 27 as follows.]

6.2.7 Persistent scheduling in the Advanced Air Interface

Persistent allocation is a technique used to reduce assignment overhead for connections with periodic traffic pattern and with relatively fixed payload size. To allocate resources persistently to an AMS, the ABS shall transmit the DL Persistent Allocation A-MAP IE for DL allocations and the UL Persistent Allocation MAP IE for UL allocations. For an UL persistent allocation, the AMS should give the intended service flow higher priority to carry data on the allocated resource, if the necessary flow information is available, e.g., through the HARQ channel mapping scheme as specified in subsection 16.2.12.12.7. The configuration parameters of the persistently allocated resource shall be maintained by the ABS and AMS until the persistent assignment is de-allocated, changed, or an error event occurs. Persistent scheduling does not include special arrangements for HARQ retransmission of data initially transmitted using persistently allocated resources. Resource for each DL retransmission shall be allocated using a DL Basic Assignment A-MAP IE. Resource for an UL retransmission shall be allocated using a using a UL Basic Assignment A-MAP IE only when control information for the retransmission changes.

Long-cycle persistent allocation is a technique used to reduce both assignment overhead for M2M connections with periodic traffic

pattern and with relatively fixed payload size, and the power consumption of M2M devices. To allocate resources persistently to an M2M device, the ABS shall transmit the UL Persistent Allocation MAP IE for UL allocations using M2M indicator = 0b1 and longer allocation period.

[#2: add the Section 6.3.5.5.2.4.9 and texts in C802.16p-11/0022 after line 19 page 35 as follows.]

6.3.5.5.2.4.9 UL Persistent allocation A-MAP IE

The UL persistent allocation A-MAP IE is specified in Table 855.

Table 855—UL Persistent Allocation A-MAP IE

| Syntax | Size (bits) | Value/Description |
| --- | --- | --- |
| UL M2M Persistent Allocation A-MAP_IE() { | - | - |
| A-AP IE Type | 4 | UL Persistent Allocation A-MAP_IE |
| M2M Indicator | 1 | 0b0: short cycle persistent scheduling 0b1: long cycle persistent scheduling |
| If(M2M Indicator==0b0){ |  |  |
| Allocation Period | 2 | Period of persistent allocation If (Allocation Period==0b00), it indicates the deallocation of persistent resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 6 frames |
| } else { |  |  |
| Allocation Period | TBD | Period of persistent allocation If (Allocation Period==0b00), it indicates the deallocation of persistent resource. 0b00: deallocation 0b01: 0b10: 0b11: |
| } |  |  |
| if (Allocation Period==0b00){ | - | - |
| Resource Index | 11 | Confirmation of the resource index for a previously assigned persistent resource that has been deallocated 512 FFT size: 0 in first 2 MSB bits + 9 bits for resource index 1024 FFT size: 11 bits for resource index 2048 FFT size: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 DL AAI subframes for FDD or all DL AAI subframes for TDD |
| HFA | 6 | Explicit Index for HARQ Feedback Allocation to acknowledge receipt of deallocation A-MAP IE |
| Reserved | 16 | Reserved bits |
| } else if (Allocation Period != 0b00){ | - | - |
| ISizeOffset | 5 | Offset used to compute burst size index |
| Mt | 1 | Number of streams in transmission Mt<=Nt up to 2 streams per AMS supported Nt : Number of transmit antennas at the AMS 0b0: 1 stream 0b1: 2 streams |
| TNS | 2 | Total number of streams in the LRU for CSM 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| if(TNS >Mt){ | - | - |
| SI | 2 | First pilot index for CSM with TNS = 2 streams: 0b00, 0b01 First pilot index for CSM with TNS = 3,4 streams: 0b00, 0b01, 0b10, 0b11 |
| } |  |  |
| else if (TNS == Mt) { |  |  |
| MEF | 1 | MIMO encoder format 0b0: SFBC 0b1: Vertical encoding |
| Reserved | 1 | Reserved bit |
| } | - | - |
| PF | 1 | Precoding Flag 0b0: non adaptive precoding 0b1: adaptive precoding using the precoder of rank Mt of the AMS.’s choice |

---- End of proposed text ----