| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Long cycle persistent scheduling for periodic M2M transmissions |  |
| Date Submitted | 2011-09-10 |  |
| Source(s) | Honggang Li, Rui Huang Shantidev Mohanty Intel Ming-Hung Tao ITRI Yi-Ting Lin Institute for Information Industry (III) | Email: Honggang.li@intel.com |
| Re: | Call for comments on the 16p AWD |  |
| Abstract | This contribution proposes texts about UL MIMO feedback for fixed M2M devices |  |
| Purpose | For discussion in 802.16p TG and adoption in to the 802.16p AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Long cycle persistent scheduling for periodic M2M transmissions

Hongang Li, Rui Huang, Shantidev Mohanty

Intel

Ming-Hung Tao

ITRI

Yi-Ting Lin

Institute for Information Industry (III)

# Introduction

For M2M applications, there are lots of cases that the M2M devices are fixed deployed and have small packets to send periodically, e.g. the electricity meters, temperature sensors, medical monitors/sensors etc.

For such kind of fixed M2M devices with periodic small packets transmission, the transmission pattern is very much alike periodic VoIP stream but in a much longer interval between two consecutive packets, so a long-cycle persistent scheduling for M2M transmission is proposed to meet such a requirement.

# Proposal

The long-cycle persistent scheduling process as follows:

- When an M2M device starts to work in connected mode after network entry and registration, it will notify the service flow characteristics to the BS.

- If the BS gets to know there will be periodic traffic pattern, she will pre-allocate the channel to the M2M device in UL M2M persistent allocation A-MAP IE, with the channel allocation information for the data traffic and/or UL synchronization, and the allocation period of validity.

- The M2M device can switch to the sleep state for power saving in connected mode

- When there is M2M data to be sent, it will wake up from the sleep state and send the data in pre-allocated data channel, the M2M device will switch to the sleep state when ACK from the BS is received or other termination condition is met.

The BS can de-allocate or reallocate the persistent scheduling for M2M service by UL persistent allocation A-MAP IE.

Figure 1, long cycle persistent scheduling for periodic M2M transmissions

# Text proposal

---- Start of proposed text ----

[#1: Add the Section 6.2.7 and texts in C802.16p-11/0022 after line 25 page 27 as follows.]

6.2.7 Persistent scheduling in the Advanced Air Interface

…

Long-cycle persistent allocation is used to reduce both assignment overhead for M2M connections with periodic traffic

pattern and with relatively fixed payload size, and the power consumption of M2M devices. To allocate resources persistently to an M2M device, the ABS may transmit the UL M2M Persistent Allocation MAP IE for UL allocations with longer allocation period. Only one spatial stream is supported for persistent scheduling for M2M.

[#2: add the Section 6.3.5.5.2.4.9 and texts in C802.16p-11/0022 after line 19 page 35 as follows.]

6.3.5.5.2.4.9 UL M2M Persistent allocation A-MAP IE

The UL M2M persistent allocation A-MAP IE is specified in Table 855.

Table 855—UL M2M Persistent Allocation A-MAP IE

| Syntax | Size (bits) | Value/Description |
| --- | --- | --- |
| UL M2M Persistent Allocation A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | UL M2M Persistent Allocation A-MAP_IE |
| Allocation Period | 3 | Period of persistent allocation for M2M: 0b000: deallocation 0b001: 5 superframes 0b010: 10 superframes 0b011: 25 superframes 0b100: 50 superframes 0b101~0b111: reserved |
| if (Allocation Period==0b00){ | - | - |
| Resource Index | 11 | Confirmation of the resource index for a previously assigned persistent resource that has been deallocated 512 FFT size: 0 in first 2 MSB bits + 9 bits for resource index 1024 FFT size: 11 bits for resource index 2048 FFT size: 11 bits for resource index Resource index includes location and allocation size |
| TTI and Relevance | 2 | Indicates the TTI type and the location of UL subframe relevant to this A-MAP. 0b00: long TTI 0b01: default TTI, the first UL subframe relevant to this A-MAP 0b10: default TTI, the second UL subframe relevant to this A-MAP 0b11: default TTI, the third UL subframe relevant to this A-MAP |
| HFA | 6 | Explicit Index for HARQ Feedback Allocation to acknowledge receipt of deallocation A-MAP IE |
| Reserved | 14 | Reserved bits |
| } else if (Allocation Period != 0b00){ | - | - |
| IsizeOffset | 5 | Offset used to compute burst size index |
| Resource Index | 11 | Confirmation of the resource index for a previously assigned persistent resource that has been deallocated 512 FFT size: 0 in first 2 MSB bits + 9 bits for resource index 1024 FFT size: 11 bits for resource index 2048 FFT size: 11 bits for resource index Resource index includes location and allocation size |
| TTI and Relevance | 2 | Indicates the TTI type and the location of UL subframe relevant to this A-MAP. 0b00: long TTI 0b01: default TTI, the first UL subframe relevant to this A-MAP 0b10: default TTI, the second UL subframe relevant to this A-MAP 0b11: default TTI, the third UL subframe relevant to this A-MAP |
| HFA | 6 | Explicit Index for HARQ Feedback Allocation to acknowledge receipt of deallocation A-MAP IE |
| N_ACID | 2 | Number of ACIDs for implicit cycling of HARQ channel identifier. 0b00: 1 0b01: 2 0b10: 3 0b11: 4 |
| Initial_ACID | 4 | Initial value of HARQ channel identifier for implicit cycling of HARQ channel identifiers. |
| Reserved | 5 | Reserved bits |
| } |  |  |
| Reserved | 1 | Reserved bit |

---- End of proposed text ----