| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Parameters clarification in AAI-PAG-ADV message |  |
| Date Submitted | 2011-09-10 |  |
| Source(s) | Honggang Li, Rui Huang Shantidev Mohanty Intel | Email: Honggang.li@intel.com |
| Re: | 802.16p amendment texts |  |
| Abstract |  |  |
| Purpose | Discuss and adopt proposed texts |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Parameters clarification in AAI-PAG-ADV message

Hongang Li, Rui Huang, Shantidev Mohanty

Intel

# Texts Proposal

----------------------------------------------  Text Start -----------------------------------------------

[Modify Table706 in page 17 as follows]

6.2.3.23  AAI-PAG-ADV (paging advertisement) Message

Table 706—AAI-PAG-ADV Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| For (j=0; j<Num_MGID; j++) { |  | Num_MGID indicates the number of MGIDs included in this paging message [0..63] | Shall be included if the ABS sends DL multicast data for M2M after transmission of the AAI-PAG-ADV message. |
| MGID | 15 | M2M Group ID |  |
| Action code | 2 | 0b00: Performing netwok re entry 0b01: Performing location update 0b10: Receiving multicast traffic 0b11: reserved |  |
| M2M report code | 1 | Indicate the opportunity for the M2M device to send the uplink report 0b0: No action required 0b1: Send uplink report | Present if M2M is supported |
| If (Action Code == 0b10){ |  |  |  |
| Multicast transmission start time (MTST) | 8 | Least significant 8 bits of the frame number in which the ABS starts sending DL multicast data. | Shall be present when the MTST needs to be included in this message. |
| } |  |  |  |
| If(Action code == 0b00){ |  |  |  |
| M2M network re-entry type | 3 | Indicate the network re-entry scheme for M2M device 0b000: dedicated channel allocation for AAI-RNG-REQ 0b001: dedicated ranging channel allocation, S-RCH 0b010: dedicated ranging channel allocation, NS-RCH 0b011-0b111: reserved |  |
| If( M2M network re-entry type==0b000){ | - |  |  |
| Dedicated channel allocation | TBD | If M2M network re-entry type = 0, to indicate Resource Index for AAI-RNG-REQ; |  |
| } else { M2M network re-entry type==0b001 or 0b010} |  |  |  |
| Dedicated channel allocation | TBD | If M2M network re-entry type = 1,2 to indicate dedicated ranging channel allocation; |  |
| } |  |  |  |
| Dedicated channel allocation timer | TBD |  |  |
| } |  |  |  |
| … |  |  |  |
| } |  |  |  |

----------------------------------------------  Text End -----------------------------------------------