| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Methods for MGID Change Detection |  |
| Date Submitted | 2011-09-11 |  |
| Source(s) | Marta Martinez Tarradell, Shantidev Mohanty, Honggang Li, Rui Huang Intel | E-mail: marta.m.tarradell@intel.com |
| Re: | 802.16p amendment working document (IEEE 802.16p-11/0021) |  |
| Abstract | Proposes a method to improve the way the MGID is re-assigned in a group of M2M devices |  |
| Purpose | Discuss and adopt proposed texts |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Methods for MGID Change Detection

Marta Martinez Tarradell, Shantidev Mohanty, Honggang Li, Rui Huang

Intel Corporation

# Introduction

# It is specified in Section 6.3.1 that MGID of a M2M Group may change. When the MGID changes the M2M devices in connected mode and idle mode use different procedures to update their MGID. However, it is not clear how the M2M devices detect this change of MGID.  This contribution proposes methods using which the M2M devices of a M2M Group can learn about the MGID change.

# Proposal

# When the BS determines that the MGID of a M2M group has changed, it informs the M2M devices regarding this change. The BS can use various message to indicate this change to the M2M devices. In this contribution we propose to use the DCD message to indicate this change.

# Proposed texts

## Proposed Text #1

----------------------------------------------  Proposed Text Start -----------------------------------------------

[Editor’s note] Add the following section after line 21 in page 4 of Section 6.3.1of IEEE 802.16p-11/0021.

# When the BS determines that the MGID of a M2M Group has changed, it indicates this change to the M2M devices belonging to that group using DCD message. Upon learning about the MGID change from DCD message, the M2M devices uses the procedures defined in Section 6.3.1 to update their MGID.

----------------------------------------------  Proposed Text End -----------------------------------------------