| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Improvement procedure to update of MGID |  |
| Date Submitted | 2011-09-20 |  |
| Source(s) | Marta Martinez Tarradell, Shantidev Mohanty, Honggang Li, Rui Huang Intel Eunjong Lee LG Electronics Hyunjeong Kang Samsung Electronics | E-mail: marta.m.tarradell@intel.com |
| Re: | 802.16p amendment working document (IEEE 802.16p11/0021) |  |
| Abstract | Proposes a method to improve the way the MGID is re-assigned in a group of M2M devices |  |
| Purpose | Discuss and adopt proposed texts |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Improvement Procedure to Update MGID

Marta Martinez Tarradell, Shantidev Mohanty, Honggang Li, Rui Huang

Intel

Eunjong Lee

LG Electronics

Hyunjeong Kang

Samsung Electronics

# Introduction

One requirement in 16p systems emphasizes the need of supporting efficient way to manage large number of M2M devices (see 6.2.1 in [1]).

For 16p systems, M2M Group Identifier (MGID) concept is created to recognize a group of M2M devices in the domain network entity. M2M Technical Report ([2]) shows different cases which M2M devices will be in idle most of their time. This proposal propone an improvement for updating MGID value to a whole group.

# Main Idea of this proposal

This contribution proposes a more efficient mechanism to update MGID in case a whole group or most of the M2M devices of a group need to take this action. It is proposed the usage of paging message to indicate the update or re-assignment of MGID (Figure 1).

When an idle mode M2M device that belongs to the M2M group (indentified by its MGID) receives a paging message directed to its MGID and with the M2M Update Indicator enable, it will look for the new MGID value in the MOB-PAG-ADV.

After receiving the updated MGID value, the M2M device must send an acknowledge message to the network to indicate that received the new MGID successfully (Figure 2). This acknowledge message will be carried in the RNG-REQ message.

If the BS does not receive the acknowledgement message from any of the M2M devices belonging to that M2M group which MGID is updated, it assumes that those M2M devices missed the MGID update information and will send them a unicast message with the new MGID value. To do that, the BS will wait until next paging cycle to request those M2M devices to do a RNG-REQ (Figure 3) and in the RNG-RSP (Figure 4) the BS will add the information related to the new MGID.

Figure 1. First Step : Send MOB-PAG-ADV to update all MGID to all MS’s in a M2M group

Figure 2. Second Step: M2M MS’s acknowledge the reception of the MGID update

Figure 3. Third Step: MS sends a RNG-REQ to BS

Figure 4. 4th Step: BS sends the new MGID value through RNG-RSP

# Proposed texts

## Proposed Text #1

----------------------------------------------  Proposed Text Start -----------------------------------------------

[Editor’s note] Add the following section after line 22 in page 4 of IEEE 802.16p-11/0021.

A 15-bit M2M group ID (MGID) uniquely identifies an M2M group in the domain of the network entity that assigns MGID that one or more devices belong to.

An MGID is assigned to a service flow of an M2M device by a network entity after initial network entry through DSA procedure and released during an explicit network exit (e.g., power down location update). The assigned MGID shall be retained by an M2M device even in idle mode unless the M2M device exits from the network or the network explicitly deletes the service flow associated with the MGID. The MGID can be re-assigned during normal mode and idle mode. During normal mode, the MGID may be changed, and delete by DSC, and DSD procedures procedure respectively. Reassignment procedure during idle mode is TBD.

During the idle mode, the MGID may be changed through location update (i.e. BS-initiated location update) or network reentry. When the BS updates the MGID through the BS-initiated location update, the BS can trigger the group location update as well as individual location update. When the BS changes the MGID of all M2M devices within the multicast group, the BS can trigger the group location update via paging message.

When the M2M device performs the timer based update, if the BS needs to update the MGID of M2M device, the RNG-RSP message with new MGID is sent by the BS in response to the RNG-REQ message. After receiving the updated MGID value, the M2M device shall send an acknowledgement (ACK) message to the BS. This ACK message is carried in the RNG-REQ message.

A BS may use MOB-PAG-ADV to indicate the update of MGID and its new value to all the M2M devices in a group. When an idle mode M2M device that belongs to the M2M group (indentified by its MGID) receives a paging message directed to its MGID TLV and the Action Code TLV is set to 0b11, this M2M device shall update its MGID based on the value indicated by MGID Re-assignment TLV (see section 11.17.5)

After receiving the updated MGID value, the M2M device shall send an acknowledgement (ACK) message to the BS. This ACK message is carried in the RNG-REQ message.

If the BS does not receive the acknowledgement message from any of the M2M devices belonging to that M2M group which MGID has been updated, it assumes that those M2M devices missed the MGID update information. In the next paging cycle BS may ask those M2M devices to perform location update and may send them a unicast message with the new MGID value (RNG-RSP). After receiving the updated MGID value, the M2M device shall send an acknowledgement (ACK) message to the BS. This ACK message is carried in the RNG-REQ message.

----------------------------------------------  Proposed Text End -----------------------------------------------

## Proposed Text #2

----------------------------------------------  Proposed Text Start -----------------------------------------------

[Editor’s note] Add the following section 6.3.2.3.5 RNG-REQ after line 58 in page 4 of IEEE 802.16p-11/0021.

The following TLV parameters may be included in an RNG-REQ message when MGID is updated in a M2M device.

MGID Update Acknowledgement Indicator TLV

One bit to 1 to indicate that received a MGID indicator.

[Editor’s note] Add the following section 11.5 in page 20 of IEEE 802.16p-11/0021.

| Name | Type | Length | Value | PHY Scope |
| --- | --- | --- | --- | --- |
| MGID Update Acknowledgement Indicator | 42 | 1 | Bit 0: set to 1 to indicate that new MGID was received Bit 1-7: Reserved | OFDMA |

----------------------------------------------  Proposed Text End -----------------------------------------------

## Proposed Text #3

----------------------------------------------  Proposed Text Start -----------------------------------------------

[Editor’s note] Add the following section 6.3.2.3.6 RNG-RSP after line 20 in page 5 of IEEE 802.16p-11/0021.

The following TLV parameters may be included in an RNG-RSP message to update MGID in a M2M device.

MGID Update TLV

Bits 0-15: current MGID value

Bits 16-31: new MGID value

[Editor’s note] Add the following section 11.6 in page 21 of IEEE 802.16p-11/0021.

| Name | Type | Length | Value | PHY Scope |
| --- | --- | --- | --- | --- |
| MGID Update | 49 | 4 | Bits 0-15: current MGID value Bits 16-31: new MGID value | OFDMA |

----------------------------------------------  Proposed Text End -----------------------------------------------

## Proposed Text #4

----------------------------------------------  Proposed Text Start -----------------------------------------------

Insert the following text in Section11.17.5 page 24 of IEEE 802.16p-11/0021.

| Name | Type | Length | Value |
| --- | --- | --- | --- |
| MGID | ... | ... | ... |
| Action Code | 156.2 | 1 | Bits 0-1: Indicates Action code for the M2M Group ID 0b00 - Performing network reentry 0b01- Performing location update 0b10 - Receiving multicast traffic 0b11- Reserved Re-assignment of MGID Bits 2-7: Padding, Will be set to 0. |

The following TLV element shall appear in each M2M group paging parameter TLV if Action Code is set to 0b11 (which identifies Re-assignment of MGID).

| Name | Type | Length | Value |
| --- | --- | --- | --- |
| MGID Re-assignment for M2M | 156.4 | 2 | New MGID (15 bits) |

----------------------------------------------  Proposed Text End -----------------------------------------------

# References

- IEEE 80216ppc-10_0004r2, “IEEE 802.16p Machine to Machine (M2M) System Requirements Document (SRD)”.

- IEEE 80216p-10_0005, Machine to Machine (M2M) Communications Technical Report

- IEEE P802.16-2009

- P802.16m/D12.