| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | M2M Group MAC Control Message (MGMC) |  |
| Date Submitted | 2011-09-14 |  |
| Source(s) | Marta Martinez Tarradell, Shantidev Mohanty, Honggang Li, Rui Huang Intel | E-mail: marta.m.tarradell@intel.com |
| Re: | 802.16p amendment working document (IEEE 802.16-11/021) |  |
| Abstract | Proposes a new MAC management message to send a single message to all the M2M devices that belong to a M2M group |  |
| Purpose | Discuss and adopt proposed texts |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

M2M Group MAC Control Message (MGMC)

Marta Martinez Tarradell, Shantidev Mohanty, Honggang Li, Rui Huang

Intel

# Introduction

One requirement in16p systems emphasizes the need of supporting efficient way to manage large number of M2M devices (see 6.2.1 in [1]).

For 16p systems, M2M Group Identifier (MGID) concept is created to recognize a group of M2M devices in the domain network entity. The MGID may be used to convey the same actions to a group of M2M devices that may share common functionality or characteristics. Current IEEE 802.16 standard ([3] and [4]) does not have any management message type that allows only communicating with a particular group of M2M devices.

# Main Idea of this proposal

This contribution proposes a mechanism to send same/similar information to multiple M2M devices simultaneously without sending separate messages to each M2M devices. A new MAC management message is proposed to convey the same/similar action to a group of M2M devices identified by MGID.

The actions that this message can transmit are not the focus of this contribution. Their definition would depend on the needs of the specific M2M application to support. This new MAC management message is referred to as M2M Group MAC Control (MGMC) message.

M2M Technical Report ([2]) exposes some examples of possible actions that may want to be prioritized and included. For example in the Security Access and Surveillance category (see 2.1 in [2]), there might be actions that want to be taken in all M2M devices if an intrusion is detected; in Public Safety (see 2.3 in [2]) different actions might be taken under particular alarms; in Healthcare (see 2.5 in [2]) might use this message to update/change specific parameters in the M2M devices for each M2M group based on their own categorization and progress; etc.

If a single M2M device needs to be conveyed with one of these actions, the same message might be used for a single M2M device instead of a group.

Current proposal focus its effort in the definition of this new messages and some initial actions but leaves most of the possible actions as TBD looking forward to deeper study about the detailed needs of M2M applications.

# Proposed texts

## Proposed Text #1

----------------------------------------------  Proposed Text Start -----------------------------------------------

[Editor’s note] Add the following section after line 22 in page 4 of IEEE 802.16p-11/021.

# Instead of sending separate messages to each M2M device with similar information, the BS may use the M2M Group message with the MGID to send that information to multiple M2M devices simultaneously.

----------------------------------------------  Proposed Text End -----------------------------------------------

## Proposed Text #1

----------------------------------------------  Proposed Text Start -----------------------------------------------

[Editor’s note] Add the following section 6.3.2.3 after line 24 in page 4 of IEEE 802.16p-11/021.

Table 38—MAC management messages (continued)

| Type | Message Name | Message Description | Connection |
| --- | --- | --- | --- |
| 70 | MGMC | M2M Group MAC Control Message | -- |
| 71-255 | -- | Reserved | -- |

----------------------------------------------  Proposed Text End -----------------------------------------------

## Proposed Text #2

----------------------------------------------  Proposed Text Start -----------------------------------------------

[Editor’s note] Add the following section 6.3.2.3.x after line 16 in page 9 of IEEE 802.16p-11/021

6.3.2.3.x M2M Group MAC Control Message (MGMC)

The MGMC message may be sent to a group of M2M devices that belong to the same M2M group (defined by an MGID) to communicate parameters and/or instructions. The MGMC message shall be sent to the M2M devices when the devices are in connected state or idle state. When a MGMC message is sent to M2M devices in idle state, it shall be transmitted during the paging listening interval of those devices.

Table XXX – MGMC message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MGMC_Message_format() { | -- | -- |
| Management Message Type = 70 | 8 | -- |
| MGID | 15 |  |
| Action Code | 4 | Use to indentify the purpose if this message 0x0: update/re-assignment of MGID value 0x1: update of parameters 0x2 – 0x15 : Reserved |
| If (Action Code == 0x00) { |  |  |
| New MGID | 15 | New MGID value to be assigned |
| } elseif (Action Code == 0x01) { |  |  |
| TBD |  | Update M2M specific parameters |
| } |  |  |
| } |  |  |

----------------------------------------------  Proposed Text End -----------------------------------------------

# References

- IEEE 80216ppc-10_0004r2, “IEEE 802.16p Machine to Machine (M2M) System Requirements Document (SRD)”.

- IEEE 80216p-10_0005, Machine to Machine (M2M) Communications Technical Report

- IEEE P802.16-2009

- P802.16m/D12.