| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Reply contribution on comment #12 |  |
| Date Submitted | 2011-09-16 |  |
| Source(s) | Giwon Park LG Electronics | E-mail: giwon.park@lge.com |
| Re: | Call for comments on IEEE 802.16p AWD (based 802.16.1/D0) |  |
| Abstract | This contribution proposes texts for localized idle mode |  |
| Purpose | To discuss and adopt the proposed text in the 802.16p amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Reply contribution on comment #12

Giwon Park

LG Electronics

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

Modify texts in page 5, line 4 as follows

6.2.1.3.2 Fixed M2M Deregistration ID (FMDID)

A 16-bit value that uniquely identifies a fixed M2M device in the base station that assigns FMDID domain of the ABS. An FMDID is assigned to a fixed M2M device by a base station ABS during idle mode entry and released during an explicit network exit (e.g., power down location update) or when the M2M device performs the network reentry. The ABS may assign a new FMDID to a fixed M2M device during location update procedure.

Modify the contents of Table 706 as follows

Table 706 – AAI-PAG-ADV message field description

| Field | Size (bits) | Value/Description | condition |
| --- | --- | --- | --- |
| Paging_Group_IDs bitmap | L | Indicate that the paging information for the corresponding paging group is included in AAI-PAG-ADV message. The length of Paging_Group_IDs bitmap is same as Num_PGIDs in PGID-Info message. 0: The paging information for the corresponding PGID is not included. 1:The paging information for the corresponding PGID is included | L equals the Num_PGIDs in PGID-Info message |
| For(i=0;i<M;i++){ |  |  | M equals the number of bits in Paging_Group_IDs bitmap whose bit is set to 1 |
| For(j=0; j< Num_AMSs ;j++){ |  | Num_AMSs indicates the number of paged AMSs in a corresponding paging group 1…32 |  |
| Deregistration Identifier | 18 | Used to indicate Deregistration ID for the AMS to be paged(Deregistration Identifier and Paging Cycle are used to identify each paged AMS) 0….. 218 -1 | Present if the S-SFH Network Configuration bit==0b0 |
| MAC Address Hash | 24 | Used to identify the AMS to be paged | Present if the S-SFH Network Configuration bit==0b1 |
| Paging Cycle | 4 | Used to indicate Paging cycle for the AMS to be paged 0x00: 4 Superframes 0x01: 8 Superframes 0x02: 16 Superframes 0x03: 32 Superframes 0x04: 64 Superframes 0x05: 128 Superframes 0x06: 256 Superframes 0x07: 512 Superframes 0x08: 32786 Superframes 0x09: 262144 Superframes 0x10: 4194304 Superframes 0x1108-0x15:Reserved | Present if the S-SFH Network Configuration bit=0b0 Values 0x08 – 0x10 shall be applied to M2M devices only. |
| Action Code | 1 | Used to indicate the purpose of the AAI-PAG-ADV message. 0b0:perform network reentry 0b1:perform ranging for location update |  |
| M2M Report code | 1 | Indication for the M2M device to send the uplink report 0b0:reserved 0b1:Send uplink report | Present if M2M is supported |
| } |  |  |  |
| } |  |  |  |
| For(i=0;i<Num_MGID;i++){ |  | Num_MGID indicates the number of MGIDs included in this paging message [0…63] | Shall be included if the ABS sends DL multicast data for M2M after transmission of the AAI-PAG-ADV message. |
| MGID | 15 | M2M Group ID |  |
| Action code | 2 | 0b00:Performing network reentry 0b01:Performing location update 0b10:Receiving multicast traffic 0b11:Reserved |  |
| If(Action code == 0b00){ |  |  |  |
| M2M network re-entry type | 3 | Indicate the network reentry scheme for M2M device 0b000:dedicated channel allocation for AAI-RNG-REQ 0b001:dedicated ranging channel allocation, S-RCH 0b010:dedicated ranging channel allocation,NS-RCH 0b011-0b111: Reserved |  |
| Dedicated channel allocation | TBD | If M2M network re-entry type =0, to indicated Resource Index for AAI-RNG-REQ If M2M network re-entry type =1,2, to indicated dedicated ranging channel allocation |  |
| Dedicated channel allocation timer | TBD |  |  |
| } |  |  |  |
| If(Action code ==0b10){ |  |  |  |
| Multicast transmission start time(MTST) |  |  |  |
| } |  |  |  |
| } |  |  |  |
| For(j=0; j<Num_FMDID;j++){ |  | Num_FMDID indicates the number of FMDIDs included in this paging message[1…32] | Shall be included when the ABS pages the fixed M2M devices |
| Fixed M2M Deregistration ID (FMDID) | 16 | Fixed M2M Deregistation ID |  |
| Action code | 1 | 0: Performing network reentry 1: Performing location update |  |
| M2M report code |  |  |  |
| } |  |  |  |
| } |  |  |  |
| For(i=0;i<Num_FMDID;i++){ |  | Num_FMDID indicates the number of FMDIDs included in this paging message[1…32] | Shall be included when the ABS pages the fixed M2M devices in localized idle mode |
| Fixed M2M Deregistration ID (FMDID) | 16 | Fixed M2M Deregistation ID |  |
| Action code | 1 | 0: Performing network reentry 1: Performing location update |  |
| M2M report code | 1 | Indication for the M2M device to send the uplink report 0b1: send uplink report | Present if needed |
| } |  |  |  |
| Extension Flag | 1 | Used to indicate the remaining part of the AAI-PAG-ADV message exists 0b0: This is the last fragment of the AAI-PAG-ADV message 0b1: This is not the last fragment of the AAI-PAG-ADV message; the remaining fragments of the message will be transmitted in the subsequent subframes or frames. |  |
| Emergency Alert Indication | 1 | Used to indicate the presence of emergency information 0b0: Reserved 0b1:There is emergency information | Optional Present if there is emergency information |

Modify the contents of Table 704 as follows

Table 704. AAI-DREG-REQ Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Localized_Idle_Mode_Accepted flag | 1 | 0: The device enters the normal idle mode. 1: The device enters the localized idle mode. | This parameter shall be presented when the fixed M2M device enters the idle mode. |
| Deregistration_ Request_Code | 3 |  |  |
| If (Deregistration_Request_Cod e == 0x01) { |  |  |  |
| Localized_Idle_Mode_Request flag | 1 | 0: The device enters the normal idle mode. 1: The device enters the localized idle mode. | This parameter shall be presented when the fixed M2M device enters the idle mode and Localized idle mode is supported by the AMS. |
| … | … | … | … |
| If (Deregistration_Request_Cod e == 0x02) { |  |  |  |
| Localized_Idle_Mode_Accept flag | 1 | 0: The device enters the normal idle mode. 1: The device enters the localized idle mode. | This parameter shall be presented when the fixed M2M device enters the idle mode and Localized idle mode is supported by the AMS. |
| … | … | … | … |

Modify the contents of Table 705 as follows

Table 705 – AAI-DREG-RSP message format

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Localized_Idle_Mode_Accepted_flag | 1 | 0: the M2M device enters the normal idle mode 1:the M2M device enters the localized idle mode | This parameter shall be presented when the fixed M2M device enters the idle mode |
| ….. |  |  |  |
| If (Action code ==0x05){ |  |  |  |
| ….. |  |  |  |
| Localized_Idle_Mode_Request flag | 1 | 0: The device enters the normal idle mode. 1: The device enters the localized idle mode. | This parameter shall be presented when the fixed M2M device enters the idle mode or when Localized idle mode is supported by the ABS. |
| Paging controller ID | 48 | Used to indicate paging controller that manages and retains the AMS’s idle mode information 0..248-1 | For fixed M2M devices, this parameter is presented only when the Localized_Idle_Mode_AcceptedRequest_Flag ==0b0 |
| Paging group ID | 16 | Used to indicate paging group that the AMS is located in 0..216-1 | For fixed M2M devices, this parameter is presented only when the Localized_Idle_Mode_AcceptedRequest_Flag ==0b0 |
| Deregistation ID | 18 | Used to indicate Deregiatration ID used to identify the AMS in idle mode 0..218-1 | Presnet when the S-SFH Network configuration bit ==0b0 For fixed M2M devices, this parameter is presented only when the Localized_Idle_Mode_AcceptedRequest_Flag ==0b0 |
| Fixed M2M Deregistration ID (FMDID) | 16 | Fixed M2M Deregistation ID | For fixed M2M devices, this parameter is presented only when the Localized_Idle_Mode_AcceptedRequest_Flag ==0b1 |
| ……. |  |  |  |
| If(Action code== 0x07){ |  |  |  |
| ……. |  |  |  |
| Localized_Idle_Mode_Accept flag | 1 | 0: The device enters the normal idle mode. 1: The device enters the localized idle mode. | This parameter shall be presented when the fixed M2M device enters the idle mode or when Localized idle mode is supported by the ABS. |
| Paging controller ID | 48 | Used to indicate paging controller that manages and retains the AMS’s idle mode information 0..248-1 | For fixed M2M devices,this parameter is presented only when the Localized_Idle_Mode_Accepted_Flag ==0b0 |
| Paging group ID | 16 | Used to indicate paging group that the AMS is located in 0..216-1 | For fixed M2M devices, this parameter is presented only when the Localized_Idle_Mode_Accepted_Flag ==0b0 |
| Deregistation ID | 18 | Used to indicate Deregiatration ID used to identify the AMS in idle mode 0..218-1 | Presnet when the S-SFH Network configuration bit ==0b0 For fixed M2M devices, this parameter is presented only when the Localized_Idle_Mode_Accepted_Flag ==0b0 |
| Fixed M2M Deregistration ID (FMDID) | 16 | Fixed M2M Deregistation ID | For fixed M2M devices, This parameter shall be presented when the Localized_Idle_Mode_Accepted_Flag ==0b1 |
| …. |  |  |  |
| } |  |  |  |

Modify the fourth column of row 1 of Table 684 (page 7) as follows:

Only present if the Localized_Idle_Mode_Request/Accepted flag is set to 1 in AAI-DREG-REQ/RSP message.

Modify the fourth column of row in line 16 of Table 685 (page 9) as follows:

Only present if the Localized_Idle_Mode_Request/Accepted flag is set to 1 in AAI-DREG-REQ/RSP message.

Modify the texts in the subclause 6.2.18.7.3

Localized idle mode operation for idle mode M2M devices: A fixed M2M device in idle mode do not need not to perform the paging group based update paging operation and location update operation. To eliminate the need for paging group based update and allocating the unnecessary paging information (i.e., Paging Group ID, Paging Controller ID), a fixed M2M device may include Localized_Idle_Mode_Accepted_flag set to 1 in the AAI-DREG-REQ message a fixed M2M device may enter Localized idle mode. The localized idle mode entry procedure is the same as the idle mode entry procedure as defined in 6.2.18.1.

Localized idle mode for the fixed M2M device is initiated either by the M2M devices or by its ABS.

In case of M2M device-initiated localized idle mode entry, a fixed M2M device may include Localized_Idle_Mode_AcceptedRequest_flag set to 1 in the AAI-DREG-REQ message. The fixed M2M device may request the ABS to retain specific M2M device service and operational information for idle mode management proposes through inclusion of the Idle Mode Retain information element in the AAI-DREG-REQ message.

When a ABS receives a AAI-DREG-REQ with Localized_Idle_Mode_AcceptedRequest_flag set to 1 and accepted M2M device’s request, it does not inform the paging Controller that the M2M device enters idle mode.

Then the ABS sends AAI-DREG-RSP with Localized_Idle_Mode_Accepted_flag set to 1 or 0 . Localized_Idle_Mode_Accepted_flag set to 1 indicates that ABS accepted M2M device’s request. Then the M2M device transitions to idle mode and does not perform paging group based update paging operation and location update operation.

Localized_Idle_Mode_Accepted_flag set to 1, the M2M device enters the normal idle mode.

Using ABS-initiated localized idle mode entry, an ABS may signal for a fixed M2M device to begin localized idle mode by sending an AAI-DREG-RSP message in unsolicited manner. This unsolicited AAI-DREG-RSP may include Localized_Idle_Mode_AcceptedRequest_flag set to 1. When a fixed M2M device receives an unsolicited AAI-DREG-RSP with Localized_Idle_Mode_ AcceptedRequest _flag set to 1, the fixed M2M device shall immediately start the idle mode initiation procedures by sending AAI-DREG-REQ message with Localized_Idle_Mode_Accepted_flag set to 1 or 0 in response to the unsolicited AAI-DREG-RSP message.

Modify texts in page 30, line 34 as follows

6.2.18.7.3.1 Idle mode operations for fixed M2M devices

When the fixed M2M device enters the Localized idle mode, a Fixed M2M Deregistation ID(FMDID) is assigned to the fixed M2M device and Paging Controller ID, Paging Group ID and Deregistration ID is not required to be assigned to the fixed M2M device.

----------------- End of the text proposal ----------------------------------------------------------------------------------------