| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MGID Harmonization |  |
| Date Submitted | 2011-11-09 |  |
| Source(s) | Anshuman Nigam Samsung Electronics | E-mail: anshun@samsung.com, |
| Re: | IEEE 802.16-11/0028, IEEE 802.16 Working Group Letter Ballot #34 |  |
| Abstract | This contribution proposes harmonized result of MGID Adhoc |  |
| Purpose | To be adopted in the updated version of IEEE 802.16p-11/0033 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MGID Adhoc Harmonization

Anshuman Nigam

Samsung, LGE, ETRI, ITRI, Huawei, Intel…

# Introduction

# Discussion

The solution proposed by MGID Adhoc is as follows.

  - Proposed harmonized solution:

    - MGID which as independent space from the STID but the length remains the same as STID

    - Use MGID in new MAP IE as the length remains the same as STID

    - Masking Code

      - 1 (bit) +  3 (Type bits) + 12 bits MGID

      - A reserved type is used to create a new 12 bit space for MGIDs

    - ABS broadcasts the Zone IDs

    - M2M device derives the Zone Index corresponding to Zone ID based on the implicit ordering of the Zone IDs in the broadcasted message.

    - Use overlapping Zones. An ABS can be a part of more than one M2M Group Zone.

      - Zone ID is 12 bits

      - Zone Index size is max number of overlapping zones a BS can have. We fix it to 4.

    - An M2M Device belongs to only one M2M Group Zone

    - Addressing

- Zone Index is carried inside the MAP

- Paging message includes MGID and Zone Index

    - The FID field in PDU is set to a default value of ….

    - Update MGID on Zone change

The above harmonized conceptual text was uploaded to temp folder as MGID_Adhoc.doc on 08/11/2011.

The next section contains text proposal for the above harmonized solution.

# Proposals

Change 1

Section 6.2.1.3.1, page 4, line 47

A 15-bit value that uniquely identifies an M2M device group in the domain of the network entity that assigns MGID that one or more M2M devices belong to. An M2M Group Zone is a logical zone comprising of multiple ABSs. The domain of the network entity An M2M Group Zone is identified by M2M GROUP ZONE ID. M2M GROUP ZONE ID may be broadcasted in the AAI-SCD message. MGID is a 12 bit value that uniquely identifies a downlink multicast service flow shared by a group of M2M devices within an M2M Group Zone. An M2M device derives the Zone Index corresponding to an M2M GROUP ZONE ID based on the implicit ordering of the M2M GROUP ZONE IDs in the broadcasted message where the Zone Index of the first included M2M GROUP ZONE_ID is assigned to be 0 and increases sequentially till (MAX_M2M_Group_Zone – 1) where Maximum number of M2M_Group_Zones supported by an ABS is defined to be 4. Hence when the ABS is part of only one M2M Group Zone, it broadcasts only one M2M GROUP ZONE_ID and the M2M device derives the corresponding M2M Group Zone Index as 0b00. An M2M device group is addressed using the MGID and the corresponding M2M Group Zone Index. All MGIDs that are assigned to an M2M device belongs to the same M2M Group Zone.

Change 2

Section 6.2.18.7.1, page 35, line 22

Group paging may be used for M2M devices. For this, M2M Group Identifier (MGID) defined in 6.2.1.3.1along with the Zone Index of the associated M2M group zone may be included in a paging message instead of an individual identifier to identify the group of M2M devices.

Change 3

Table 706 – AAI_PAG-ADV message field description, page 19, line 12

| Field | Size | Value/Description | Condition |
| --- | --- | --- | --- |
| For (i=0; i<Num_MGID; i++) {. |  | Num_MGID indicates the number of MGIDs included in this paging message [0..63]. | Shall be included if the ABS sends DL multicast data for M2M after transmission of the AAI-PAG-ADV message.. |
| MGID. | 15. | M2M Group ID. |  |
| Zone Index | 2 | Zone Index corresponding to an M2M GROUP ZONE ID based on the implicit ordering of the M2M GROUP ZONE IDs in the broadcasted message | Present if ABS is part of more than one M2M group zone. |
| Action Code. | 2. | 0b00: Performing network reentry 0b01: Performing location update 0b10: Receiving multicast traffic without requiring network reentry 0b11: MGID re-assignment. |  |
| If (Action Code == 0b00) {. |  |  |  |

Change 4

Table 849 – Description of CRC mask, page 44, line 52

| Masking prefix | Remaining 15 LSBs |  |
| --- | --- | --- |
| 0b0 | Type Indicator | Masking Code |
|  | 0b000 | 12 bit STID or TSTID. |
|  | 0b001 | Refer to Table 850. |
|  | 0b010 | Refer to Table 851. |
|  | 0b011 | LSB 12 bits of FMDID or LSB 12 bits of DID. |
|  | 0b100 | 12 bit MGID |
| 0b1 | 15-bit RA-ID: The RA-ID is derived from the AMS’s random access attributes [i.e., superframe number (LSB 5bits), frame_index (2 bits), preamble code index for ranging or BR (6 bits) and opportunity index for ranging or BR (2 bits)] as defined below: RA-ID = (LSB 5 bits of superframe number ¦ frame_index ¦ preamble_code_index ¦ opportunity_index). |

Change 5

Table 851 - Description of the masking code for type indicator 010, Page 45, line 20

| Decimal Value | Description. |
| --- | --- |
| 4094 | Used to mask Broadcast Assignment A-MAP IE for multicast assignment for M2M application (i.e., Function Index = 0b11).. |
| 4095 | Used to mask Broadcast Assignment A-MAP IE for multicast assignment (i.e., Function Index = 0b10). |
| Others | Reserved |

Change 6

Table 866—Broadcast Assignment A-MAP IE*, Page 46, line 1

| Syntax | Size | Notes |
| --- | --- | --- |
| Broadcast_Assignment_A-MAP_IE() {. |  |  |
| A-MAP IE Type. | 4 | Broadcast Assignment A-MAP IE. |
| Function Index. | 2 | 0b00: This IE carries broadcast assignment information 0b01: This IE carries handover ranging channel allocation information 0b10: This IE carries multicast assignment information 0b11: Reserved This IE carries multicast assignment information for M2M application or ranging channel allocation information for M2M devices Reserved. |
| …. |  | … |
| MGID | 15 |  |

Change 7

M2M Group Assignment A-MAP IE, Add this table on page 51, line 40

6.3.5.5.2.4.17 M2M Group Assignment A-MAP IE

| Syntax | Size | Notes |
| --- | --- | --- |
| M2M_A-MAP_IE() {. |  |  |
| A-MAP IE Type. | 4 | M2M A-MAP IE. |
| Function Index. | 2 | 0b00: This IE carries M2M assignment information 0b01-0b11: Reserved. |
| If(Function Index == 0b0) {. |  | … |
| Burst Size. | 6 |  |
| Resource Index. | 11 |  |
| Long_TTI_Indicator. | 1 |  |
| Zone Index | 2 |  |
| } |  |  |

Change 8

Assignment A-MAP IE types, Add this table in page 51, line 40 after change 7

| A-MAP IE Type | Usage | Property |
| --- | --- | --- |
| …. |  |  |
| 0b1100 | Broadcast Assignment A-MAP IE | Broadcast/Multicast |
| 0b1101 | Reserved M2M Group Assignment A-MAP IE | N/A |
| … |  |  |

Change 9

Modify the sections 6.2.28.4 and 6.2.28.4.1 as shown below

6.2.28.4 Multicast operation for machine to machine (M2M) applications

Multicast Service for M2M applications provides concurrent transport of DL data common to M2M devices belonging to an M2M device group using an MGID in an ABS. Multicast service is associated with an ABS and is offered in the downlink only. Each multicast connection is associated with a service flow provisioned with the QoS and traffic parameters for that service flow. Service flows to carry multicast data are instantiated on individual M2M devices participating in the service while in Connected State. During such instantiation, the M2M device learns the parameters that identify the service and associated service flows.

The ABS shall use a combination of MGID and FID to provide the multicast service. The same MGID and FID is assigned to a group of M2M devices that participate in the same multicast service and is assigned by a network during DSA procedure.

To access the multicast service, the M2M device that is assigned an MGID shall apply the 16-bit CRC mask with masking prefix = 0b0, message type indicator = 0b010, and decimal value = 4094 to decode the assignment A-MAP IE. If the MGID is included in the Broadcast Assignment A-MAP IE, the M2M device shall obtain the multicast burst according to the instruction in the Broadcast Assignment A-MAP IE. The FID field in the MAC header of the MAC PDU (in multicast burst) carrying the MAC SDUs for the multicast service flow is set to ‘0100’.

Change 10

Table 740 Modify line 55-60; Page 27 as follows

| If(Multicast service flow for M2M device){ |  |  |  |
| --- | --- | --- | --- |
| SFID | 32 | Service flow identifier | Shall be present if this service flow is related with M2M multicast service and when an 802.16p ABS initiates AAI-DSA-REQ. |
| MGID | 152 | MGID to be added | Shall be present if this service flow is related with M2M multicast service and when an 802.16p ABS initiates AAI-DSA-REQ. |
| M2M_Group Zone Index | 2 | M2M Group Zone Index of the corresponding M2M Group Zone ID that the MGID belongs to | Present if ABS is part of more than one M2M group zone. |
| MGSS | 64 | MGSS (M2M service Group Security Seed) for an M2M device group | May be present when an ABS initiates AAI-DSA-REQ for this service flow that is related with |

Change 11

Table 741 Modify line 44; Page 28

| If (Confirmation Code == 0 && AMS-initiated AAI-DSA-REQ) If(Multicast service flow for M2M device) |  |  |  |
| --- | --- | --- | --- |
| { |  |  |  |
| FID | 4 | An identifier of a service flow. |  |
| MGID | 152 | MGID to be added | Shall be present if this service flow is related with M2M multicast service and when an M2M device initiates AAI-DSA-REQ. |
| M2M_Group Zone Index | 2 | M2M Group Zone Index of the corresponding M2M Group Zone ID that the MGID belongs to | Present if ABS is part of more than one M2M group zone. |
| } |  |  |  |

Change 12

Table 743 Modify line 65; Page 28

| MGID | 152 | MGID to be added | Shall be present if this service flow is related with M2M multicast service and when an M2M device initiates AAI-DSA-REQ. |
| --- | --- | --- | --- |
| M2M_Group Zone Index | 2 | M2M Group Zone Index of the corresponding M2M Group Zone ID that the MGID belongs to | Present if ABS is part of more than one M2M group zone. |

Change 13

Delete table 763a on page 29

Change 14

Modify Table 763b, section 6.2.3.65

| If (Action Code == 0x00) {. |  |  |  |
| --- | --- | --- | --- |
| Current MGID. | 15. | Current MGID value to be assigned. |  |
| New MGID. | 15. | New MGID value to be assigned. |  |
| Current FID. | 4. | Current FID value to be assigned. |  |
| New FID. | 4. | New FID value to be assigned. |  |

Change 15

Table 685 Modify line 36; Page 9

| Current MGID | 152 |  |  |
| --- | --- | --- | --- |
| Current FID | 4 |  |  |
| New MGID | 152 | New MGID |  |
| M2M_Group Zone Index | 2 | M2M Group Zone Index of the corresponding M2M Group Zone ID that the MGID belongs to | Present if ABS is part of more than one M2M group zone. |
| New MGSS | 64 |  | Shall be included when the MGID is updated. |

Change 16

Table 706 Modify line 60; Page 20

| New MGID | 152 | New MGID |  |
| --- | --- | --- | --- |
| M2M_Group Zone Index | 2 | M2M Group Zone Index of the corresponding M2M Group Zone ID that the MGID belongs to | Present if ABS is part of more than one M2M group zone. |
| Current FID | 4 |  |  |
| New FID | 4 |  |  |

# References

[1] 80216p/11_0033: Enhancements to Support M2M Application for WirelessMAN Advanced System

[2] 802161_D2: WirelessMAN-Advanced Air Interface for Broadband Wireless Access Systems