| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Abnormal power down reporting in WirelessMAN-Advanced Air Interface for Broadband Wireless Access Systems |  |
| Date Submitted | 2011-11-06 |  |
| Source(s) | Heejeong Cho, Youngsoo Yuk, Inuk Jung and Jin Sam Kwak LG Electronics | E-mail: heejeong.cho@lge.com |
| Re: | IEEE 802.16-11/0028, IEEE 802.16 Working Group Letter Ballot #34 |  |
| Abstract | This contribution provides the text for abnormal power down reporting. |  |
| Purpose | For discussion in 802.16p TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Abnormal power down reporting in WirelessMAN-Advanced Air Interface
for Broadband Wireless Access Systems

Heejeong Cho, Youngsoo Yuk, Inuk Jung and Jin Sam Kwak

LG Electronics Inc.

# Introduction

There are ambiguous descriptions of the abnormal power down reporting in the current draft document [2]. In this contribution, the text for clarifications is suggested. Also, we propose an alternate mechanism to reduce the steps required to send the abnormal power down indication compared to existing scheme (e.g., AAI-RNG-REQ/RSP).

# Modification/Rationale

- Suggestion 1 : Modify the sentence as suggested in remedy-1

  - Use the unified terminology (MS  M2M device).

  - Remove descriptions for the specific mechanism (e.g., AAI-RNG-REQ) of the abnormal power down reporting in the clause 6.2.29.

    - There is another mechanism (e.g., quick access message including predefined BR index) to report the abnormal power down event.

- Suggestion 2 for the M2M device in connected state : Modify the sentence as suggested in remedy-2, remedy-3, remedy-4 and remedy-5

  - Use signaling header instead of AAI-RNG-REQ/RSP MAC control message.

    - The steps for the abnormal power down report need to be reduced in order to send the abnormal power down indication within the limited time.

  - Define the timer to wait for ABS’s response following the transmission of the abnormal power down report.

    - There are no descriptions about how long the M2M device waits for ABS’s response.

  - Retain M2M device’s context if possible.

    - The context needs to be kept in a network entity in order to expedite M2M device’s network reentry.

- Suggestion 3 for the M2M device in idle state : Modify the sentence as suggested in remedy-6, remedy-7 and remedy-8

  - Use the unified terminology (ranging code  ranging preamble code).

  - Define the timer to wait for ABS’s response following the transmission of the abnormal power down report.

    - There are no descriptions about how long the M2M device waits for ABS’s response.

    - T3 can be reused for this purpose.

  - Include parameter (e.g., DID or FMDID) used to identify the M2M device in the AAI-RNG-REQ.

    - The ABS needs to know which M2M device sends the AAI-RNG-REQ which indicates that an abnormal or involuntary power down has occurred.

  - Retain M2M device’s context if possible.

    - The context needs to be kept in a network entity in order to expedite M2M device’s network reentry.

# References

[1] IEEE P802.16.1™/D2, “WirelessMAN-Advanced Air Interface for Broadband Wireless Access Systems”

[2] IEEE P80216p-11/0033, “Enhancements to Support Machine-to-Machine Applications”

# Proposed Text Change

Adopt the following Remedies in IEEE 80216p-11_0033.

[Remedy 1: Modify the following text on page 42, line 13 as follows:]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

6.2.29 Abnormal power down reporting

When a MS detects an abnormal power down event, it tries to send an AAI-RNG-REQ message with the Ranging Purpose Indication indicating that an abnormal or involuntary power down has occurred (value 0b1110). When an abnormal or involuntary power down has occurred, an M2M device tries to report the abnormal power down event.

----------------------------------------------------- End of Proposed Text -----------------------------------------------------

[Remedy 2: Modify the following text on page 42, line 20 as follows:]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

6.2.29.1 Abnormal power down reporting in connected state

If the MS M2M device is in connected state with uplink bandwidth already allocated and available, then it may use the available bandwidth to send this AAI-RNG-REQ message containing the Ranging Purpose Indication with value 0b1110 an Abnormal Power Down Report signaling header (as defined in 6.2.2.1.3.10).

If the MS is in connected state but M2M device does not have available UL bandwidth, then it may use the procedure defined in 6.3.6 6.2.11 to request bandwidth. Upon receiving bandwidth allocation it may send the AAI-RNG-REQ message containing the Ranging Purpose Indication with value 0b1110. In case of using a three-step random access-based BR procedure, If the MS is in connected state but does not have available UL bandwidth, then it may use the quick access procedure defined in 6.2.11 to report the abnormal power down event. Tthe Predefined BR index may be used to indicate that an abnormal or involuntary power down has occurred. In other cases (e.g., a five-step random access-based BR, piggybacked BR or BR using P-FBCH), the M2M device may send the Abnormal Power Down Report signaling header upon receiving bandwidth allocation.

After transmitting the Abnormal Power Down Report signaling header or the predefined BR index indicating the abnormal power down event, the M2M device shall start its Abnormal Power Down Confirmation timer. The abnormal power down reporting procedure is over after receiving the Abnormal Power Down Confirmation signaling header. If the M2M device has not received the Abnormal Power Down Confirmation signaling header until the Abnormal Power Down Confirmation timer expires, it may restart the abnormal power down reporting procedure.

When the M2M device tries to (re)entry a network, the network (re)entry procedure is different depending on whether the M2M device's context is kept in a network entity. If the M2M device’s context is kept, the M2M device shall perform the same network reentry procedure as the M2M device in DCR mode. Otherwise, the M2M device shall perform an initial network entry procedure.

----------------------------------------------------- End of Proposed Text -----------------------------------------------------

[Remedy 3: Modify the following text on page 5, line 60 as follows:]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

| Type field (5-bits) | MAC signaling header |
| --- | --- |
| 01000 | M2M Bandwidth request (BR) with STID header |
| 01001 | M2M abnormal power down report |
| 01010 | M2M abnormal power down confirmation |
| 010000100101011-11111 | Reserved |

----------------------------------------------------- End of Proposed Text -----------------------------------------------------

[Remedy 4: Add the following text on page 6, line 38 as follows:]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

6.2.2.1.3.10 M2M abnormal power down report header

When an M2M device in connected state detects an abnormal power down event, it sends an M2M abnormal power down report signaling header indicating that an abnormal or involuntary power down has occurred. The M2M abnormal power down report signaling header is defined in Table 670b. The M2M device may request a network to retain specific M2M device service and operational information, by setting the corresponding bit in the Context Retain Information element to ‘1’.

Table 670b – M2M abnormal power down report header format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| M2M Abnormal Power Report Down Report () { |  |  |
| FID | 4 | Flow Identifier. Set to 0b0010 |
| Type | 5 | MAC Signaling header type = 0b01001 |
| Length | 3 | Indicates the length of the signaling header in bytes |
| STID | 12 | Indicates STID of the M2M device that transmits this M2M abnormal power down report signaling header. |
| STID_Valid_Offset | 3 | Indicates STID_Valid_Offset of the M2M device that sends this M2M abnormal power down report signaling header. If the assigned STID is not shared with other M2M devices, M2M device shall set this field to zero. |
| Emergency Type | 3 | 0b000: power outage 0b001~0b111: Reserved |
| Context Retain Information element | 5 | For each bit location, a value of 0 indicates the information for the associated network reentry control messages shall not be retained and managed; a value of 1 indicates the information for the associated network reentry control message shall be retained and managed. Bit 0: Retain M2M device service and operational information associated with AAI-SBC-REQ/RSP messages. Bit 1: Retain M2M device service and operational information associated with AAI-PKM-REQ/RSP messages. Bit 2: Retain M2M device service and operational information associated with AAI-REG-REQ/RSP messages. Bit 3: Retain M2M device service and operational information associated with network address. Bit 4: Retain M2M device state information. The information retained by setting bit 4 includes configuration of all Service Flows in the M2M device as set by successful AAI-DSA and AAI-DSC transactions. In particular it includes FIDs and related description (QoS descriptors and CS classifier information). |
| Reserved | 13 | Reserved. This field shall be set to Zero. |
| } |  |  |

6.2.2.1.3.11 M2M abnormal power down confirmation header

An M2M abnormal power down confirmation signaling header shall be transmitted by the ABS in response to a received abnormal power down report. The M2M abnormal power down confirmation signaling header is defined in Table 670c. When the ABS decides to retain M2M device’s context, it shall set the corresponding bit of the Context Retain Information element to ‘1’. The corresponding context is kept in a network entity in order to expedite M2M device’s network reentry.

Table 670c – M2M abnormal power down confirmation header format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| M2M Abnormal Power Down Confirmation () { |  |  |
| FID | 4 | Flow Identifier. Set to 0b0010 |
| Type | 5 | MAC Signaling header type = 0b01010 |
| Length | 3 | Indicates the length of the signaling header in bytes |
| STID | 12 | Indicates STID of the M2M device in the received M2M abnormal power down report signaling header. |
| STID_Valid_Offset | 3 | Indicates STID_Valid_Offset of the M2M device in the received M2M abnormal power down report signaling header. |
| Context Retain Information element | 5 | For each bit location, a value of 0 indicates the information for the associated network reentry control messages shall not be retained and managed; a value of 1 indicates the information for the associated network reentry control message shall be retained and managed. Bit 0: Retain M2M device service and operational information associated with AAI-SBC-REQ/RSP messages. Bit 1: Retain M2M device service and operational information associated with AAI-PKM-REQ/RSP messages. Bit 2: Retain M2M device service and operational information associated with AAI-REG-REQ/RSP messages. Bit 3: Retain M2M device service and operational information associated with network address. Bit 4: Retain M2M device state information. The information retained by setting bit 4 includes configuration of all Service Flows in the M2M device as set by successful AAI-DSA and AAI-DSC transactions. In particular it includes FIDs and related description (QoS descriptors and CS classifier information). |
| Reserved | 16 | Reserved. This field shall be set to Zero. |
| } |  |  |

----------------------------------------------------- End of Proposed Text -----------------------------------------------------

[Remedy 5: Add the following text on page 52, line 36 as follows:]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

| System | Name | Timer reference | Minimum value | Default value | Maximum value |
| --- | --- | --- | --- | --- | --- |
| M2M device | Abnormal Power Down Confirmation timer | Abnormal power down confirmation reception timeout following the transmission of an abnormal power down report | - | - | - |

----------------------------------------------------- End of Proposed Text -----------------------------------------------------

[Remedy 6: Modify the following text on page 42, line 59 as follows:]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

6.2.29.2 Abnormal power down reporting in idle state

The ABS, upon receiving the ranging preamble code, may include a CDMA Allocation A-MAP IE in the next frame identifying the M2M device and provide an allocation sufficiently large to allow the M2M device to transmit an AAI-RNG-REQ message including a Ranging Purpose Indication and the CMAC Tuple.

The M2M device shall not repeat sending of a ranging code if it does not receive an allocation from the ABS.

Upon receiving this allocation, the M2M device shall transmit an AAI-RNG-REQ message including a Ranging Purpose Indication with value 0b1110 (power outage) and a valid CMAC Tuple. The M2M device shall not repeat sending of a ranging code if it does not receive an allocation from the ABS. The M2M device shall start the T3 timer at the transmission of the AAI-RNG-REQ message. The abnormal power down reporting procedure is over after receiving the AAI-RNG-RSP message. If the M2M device has not received the AAI-RNG-RSP message by T3, it may restart the abnormal power down reporting procedure.

When the ABS decides to retain M2M device’s context, it shall set the corresponding bit of the Context Retain Information element in the AAI-RNG-RSP message to ‘1’. The corresponding context is kept in a network entity in order to expedite M2M device’s network reentry.

When the M2M device tries to (re)entry a network, the network (re)entry procedure is different depending on whether the M2M device's context is kept in a network entity. If the M2M device’s context is kept, the M2M device shall perform the same network reentry procedure as the M2M device in DCR mode. Otherwise, the M2M device shall perform an initial network entry procedure.

----------------------------------------------------- End of Proposed Text -----------------------------------------------------

[Remedy 7: Modify the following text on page 8, line 42 as follows:]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

Table 684—AAI-RNG-REQ message field descript

| Field | Size (bits) | Value | Condition |
| --- | --- | --- | --- |
| }//end of Ranging Purpose Indication else if (Ranging Purpose Indication == 0b1110){ |  | //Abnormal or involuntary power down |  |
| if (S-SFH Network Configuration bit == 0b1 or AMSID privacy is disabled) { |  |  |  |
| M2M device MAC address | 48 | M2M device’s real MAC address |  |
| } else{ |  |  |  |
| Deregistration Identifier (DID) | 18 | The ID that the M2M device is assigned for idle mode and currently maintains. | If the Localized_Idle_Mode flag is set to 1 in AAI-DREG-REQ/RSP message, DID shall not be included in this message. |
| Fixed M2M Deregistration ID (FMDID) | 16 | Used to indicate Fixed M2M Deregistration ID used to identify the fixed M2M device in idle mode 0..216-1 | Only present if the Localized_Idle_Mode flag is set to 1 in AAI-DREG-REQ/RSP message. |
| } |  |  |  |
| Paging Controller ID | 48 | The Paging Controller ID that the M2M device currently maintains in idle mode. | If the Localized_Idle_Mode flag is set to 1 in AAI-DREG-REQ/RSP message, Paging Controller ID shall not be included in this message. |
| }//end of Ranging Purpose Indication |  |  |  |

----------------------------------------------------- End of Proposed Text -----------------------------------------------------

[Remedy 8: Modify the following text on page 9, line 25 as follows:]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

Table 685—AAI-RNG-RSP message field descript

| Field | Size (bits) | Value | Condition |
| --- | --- | --- | --- |
| Unsolicited bandwidth grant indicator | 1 | 1: indicates that an unsolicited UL allocation will be provided without request from AMS during network entry or network re-entry, | Shall be included when AMS is attempting network entry Shall be included if AAI-RNG-RSP message is transmitted in response to AAI-RNG-REQ message that includes bandwidth request indicator during network reentry from idle mode. |
| Context Retain Information element | 5 | For each bit location, a value of 0 indicates the information for the associated network reentry control messages shall not be retained and managed; a value of 1 indicates the information for the associated network reentry control message shall be retained and managed. Bit 0: Retain M2M device service and operational information associated with AAI-SBC-REQ/RSP messages. Bit 1: Retain M2M device service and operational information associated with AAI-PKM-REQ/RSP messages. Bit 2: Retain M2M device service and operational information associated with AAI-REG-REQ/RSP messages. Bit 3: Retain M2M device service and operational information associated with network address. Bit 4: Retain M2M device state information. The information retained by setting bit 4 includes configuration of all Service Flows in the M2M device as set by successful AAI-DSA and AAI-DSC transactions. In particular it includes FIDs and related description (QoS descriptors and CS classifier information). | Shall be included if AAI-RNG-RSP message is transmitted in response to AAI-RNG-REQ message with Ranging Purpose Indication 0b1110. |

----------------------------------------------------- End of Proposed Text -----------------------------------------------------