| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Abnormal power down reporting in WirelessMAN Air Interface for Broadband Wireless Access Systems |  |
| Date Submitted | 2011-11-08 |  |
| Source(s) | Heejeong Cho, Youngsoo Yuk, Inuk Jung and Jin Sam Kwak LG Electronics | E-mail: heejeong.cho@lge.com |
| Re: | IEEE 802.16-11/0027, IEEE 802.16 Working Group Letter Ballot #33 |  |
| Abstract | This contribution provides the text for abnormal power down reporting. |  |
| Purpose | For discussion in 802.16p TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Abnormal power down reporting in WirelessMAN Air Interface
for Broadband Wireless Access Systems

Heejeong Cho, Youngsoo Yuk, Inuk Jung and Jin Sam Kwak

LG Electronics Inc.

# Introduction

There are ambiguous descriptions of the abnormal power down reporting in the current draft document [2]. In this contribution, the text for clarifications is suggested.

# Modification/Rationale

- Suggestion : Modify the sentence as suggested in remedy-1, remedy-2, remedy-3 and remedy-4.

  - Remove descriptions for the scheme (e.g., using RNG-REQ) of the abnormal power down reporting in the clause 6.3.35.

    - The descriptions have already been included in the sub- clause 6.3.35.1 and 6.3.35.2.

  - Define the timer to wait for BS’s response following the transmission of the abnormal power down report.

    - There are no descriptions about how long the M2M device waits for BS’s response.

    - T3 can be reused for this purpose.

# References

[1] IEEE P802.16Rev3/D1, “WirelessMAN Air Interface for Broadband Wireless Access Systems”

[2] IEEE P802.16p/D1, “Enhancements to Support Machine-to-Machine Applications”

# Proposed Text Change

Adopt the following Remedies in IEEE P802.16p/D1.

[Remedy 1: Modify the following text on page 15, line 1 as follows:]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

6.3.35 Abnormal Power Down

When an M2M device detects an abnormal power down event, it tries to send a RNG-REQ message with the Ranging Purpose Indication indicating that an abnormal or involuntary power down has occurred (bit 5 set to 1). When an abnormal or involuntary power down has occurred, an M2M device tries to report the abnormal power down event.

6.3.35.1 Abnormal power down reporting in normal operation

If the M2M device is in Nnormal Ooperation with uplink bandwidth already allocated and available, then it may use the available bandwidth to send this RNG-REQ message containing the Ranging Purpose Indication with value bit 5 set to 1.

If the M2M device is in normal mode but does not have available UL bandwidth, then it may use the procedure defined in 6.3.6 to request bandwidth. Upon receiving bandwidth allocation it may send the RNG-REQ message containing the Ranging pPurpose Indication with bit 5 set to 1.

The M2M device shall start the T3 timer at the transmission of the RNG-REQ message. The abnormal power down reporting procedure is over after receiving the RNG-RSP message. If the M2M device has not received the RNG-RSP message by T3, it may restart the abnormal power down reporting procedure.

----------------------------------------------------- End of Proposed Text -----------------------------------------------------

[Remedy 2: Modify the following text on page 15, line 39 as follows:]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

6.3.35.2 Abnormal power down reporting in idle mode

The BS, upon receiving the ranging code, may include a CDMA Allocation IE in the next frame identifying the M2M device and provide an allocation sufficiently large to allow the M2M device to transmit a RNG-REQ message including a Ranging Purpose Indication TLV and the CMAC/HMAC Tuple.

The M2M device shall not repeat sending of a ranging code if it does not receive an allocation from the BS.

Upon receiving this allocation, the M2M device shall transmit a RNG-REQ message including a Ranging Purpose Indication TLV with bit 5 set to 1 (power outage) and a valid HMAC/CMAC Tuple. The M2M device shall not repeat sending of a ranging code if it does not receive an allocation from the BS.

The M2M device shall start the T3 timer at the transmission of the RNG-REQ message. The abnormal power down reporting procedure is over after receiving the RNG-RSP message.

----------------------------------------------------- End of Proposed Text -----------------------------------------------------

[Remedy 3: Modify the following text on page 22, line 49 as follows:]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

10. Parameters and constants

10.1 Global values

| System | Name | Time reference | Minimum value | Default value | Maximum value |
| --- | --- | --- | --- | --- | --- |
| … | … |  |  |  |  |
| SS, MS | T3 | Ranging response reception timeout following the transmission of a ranging request. | — | OFDMA: 60 msec: RNG-RSP after CDMA ranging or RNG-REQ during initial or periodic ranging 50 msec: 1) RNG-RSP after RNG-REQ during HO to negotiated target BS 2) RNG-RSP after RNG-REQ during abnormal power down report 200 msec: RNG-RSP after RNG-REQ during HO to non-negotiated target BS 200 msec: RNG-RSP after RNG-REQ during location update or re-entry from idle mode 80 ms: RNG-RSP after CDMA ranging using Initial/HO Ranging Code set 1 or 2 during initial ranging/ handover/location update/ reentry from idle mode 100 ms: 1) RNG-RSP after RNG-REQ following usage of Initial Ranging Code set 1 during initial ranging, 2) RNG-RSP after RNG-REQ following usage of HO Ranging code set 1 or 2 during handover to negotiated target BS, or 3) RNG-RSP after RNG-REQ on the Primary Management Connection during initial ranging/handover to negotiated target BS /location update/reentry from idle mode. | 200 ms |

----------------------------------------------------- End of Proposed Text -----------------------------------------------------

[Remedy 4: Modify the following text on page 25, line 16 as follows:]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

Table 685—RNG-REQ message encodings

| NAME | Type | Length | Value | PHY Scope |
| --- | --- | --- | --- | --- |
| M2M Fast Bandwidth Request | 43 | 2 | Bit 0: M2M Bandwidth Request Indicator 0: No bandwidth request 1: bandwidth request Bits 1-15: BR size | OFDMA |
| Emergency Type | 44 | 1 | Bit 0-2: Emergency type for M2M abnormal power down report as follows: 000 – power outage 001~111 – Reserved Bit 3-7: Reserved | OFDMA |

----------------------------------------------------- End of Proposed Text -----------------------------------------------------