| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Abnormal power down reporting in WirelessMAN Air Interface for Broadband Wireless Access Systems |  |
| Date Submitted | 2011-11-10 |  |
| Source(s) | Heejeong Cho, Youngsoo Yuk, Inuk Jung and Jin Sam Kwak LG Electronics Jaesun Cha ETRI Tsung-Yu Tsai, Yi-Ting Lin Institute for Information Industry (III) | E-mail: heejeong.cho@lge.com |
| Re: | IEEE 802.16-11/0027, IEEE 802.16 Working Group Letter Ballot #33 |  |
| Abstract | This contribution provides the text for abnormal power down reporting. |  |
| Purpose | For discussion in 802.16p TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Abnormal power down reporting in WirelessMAN Air Interface
for Broadband Wireless Access Systems

Heejeong Cho, Youngsoo Yuk, Inuk Jung and Jin Sam Kwak

LG Electronics Inc.

Jaesun Cha

ETRI

Tsung-Yu Tsai, Yi-Ting Lin

Institute for Information Industry (III)

# Introduction

There are ambiguous descriptions of the abnormal power down reporting in the current draft document [2]. In this contribution, the text for clarifications is suggested.

# Modification/Rationale

- Comment #32 : Reflected in this contribution because there is no objection comment.

Modify the sentence as suggested in remedy-1 and remedy-2.

  - Change “Abnormal Power Down Indication” in Ranging Purpose Indication to “Abnormal Condition Indication”

- Comment #84 : Reflected in this contribution because Erik and Jaesun’s comments have been solved.

Suggestion 1 for the M2M device in connected state : Modify the sentence as suggested in remedy-3 and remedy-4

- Remove descriptions for the scheme (e.g., using RNG-REQ) of the abnormal power down reporting in the clause 6.3.35.

- Use signaling header instead of AAI-RNG-REQ/RSP MAC control message.

- Define the timer to wait for ABS’s response following the transmission of the abnormal power down report.

    - There are no descriptions about how long the M2M device waits for ABS’s response.

Suggestion 2 for the M2M device in idle state : Modify the sentence as suggested in remedy-5, remedy-6 and remedy-7

- Define the timer to wait for BS’s response following the transmission of the abnormal power down report.

    - There are no descriptions about how long the M2M device waits for BS’s response.

    - T3 can be reused for this purpose.

- Comment #85 : Not reflected in this contribution because Erik’s comment has not been solved.

- Comment #86 : Not reflected in this contribution because there are 2 objection comments.

# References

[1] IEEE P802.16Rev3/D1, “WirelessMAN Air Interface for Broadband Wireless Access Systems”

[2] IEEE P802.16p/D1, “Enhancements to Support Machine-to-Machine Applications”

# Proposed Text Change

Adopt the following Remedies in IEEE P802.16p/D1.

[Remedy 1- comment #32: Modify the following text on page 5, line 49 as follows:]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

Bit 5: Abnormal condition Power Down Indication. When this bit is set to 1, MS indicates that an abnormal condition or involuntary disconnection Power Down is occurring occurs.

----------------------------------------------------- End of Proposed Text -----------------------------------------------------

[Remedy 2- comment #32: Modify the following text on page 24, line 32 as follows:]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

Bit 5: Abnormal condition Power Down Indication. When this bit is set to 1, MS indicates that an abnormal condition or involuntary disconnection Power Down is occurring occurs.

----------------------------------------------------- End of Proposed Text -----------------------------------------------------

[Remedy 3 – comment #84: Modify the following text on page 15, line 1 as follows:]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

6.3.35 Abnormal Power Down

When an M2M device detects an abnormal power down event, it tries to send a RNG-REQ message with the Ranging Purpose Indication indicating that an abnormal or involuntary power down has occurred (bit 5 set to 1). When an abnormal or involuntary power down has occurred, an M2M device tries to report the abnormal power down event.

6.3.35.1 Abnormal power down reporting in normal operation

If the M2M device is in Nnormal Ooperation with uplink bandwidth already allocated and available, then it may use the available bandwidth to send this RNG-REQ message containing the Ranging Purpose Indication with value bit 5 set to 1. an M2M Abnormal Power Down Report header (as defined in 6.3.2.1.2.2.3.1).

If the M2M device is in normal mode but does not have available UL bandwidth, then it may use the procedure defined in 6.3.6 to request bandwidth. Upon receiving bandwidth allocation it may send the RNG-REQ message containing the Ranging purpose Indication with bit 5 set to 1. the M2M Abnormal Power Down Report header.

The M2M device shall start its Abnormal Power Down Confirmation timer at the transmission of the M2M Abnormal Power Down Report header. The abnormal power down reporting procedure is over after receiving the M2M Abnormal Power Down Confirmation header. If the M2M device has not received the M2M Abnormal Power Down Confirmation header until the Abnormal Power Down Confirmation timer expires, it may restart the abnormal power down reporting procedure.

----------------------------------------------------- End of Proposed Text -----------------------------------------------------

[Remedy 4 – comment #84: Modify the following text on page 4, line 61 as follows:]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

Table 4.—MAC header HT and EC fields encoding (continued)

| HT | ECa | MAC PDU type | Reference figure | Reference table |
| --- | --- | --- | --- | --- |
| 1 | 0 | DL: This encoding is not defined DL MAC signaling header type I. UL: MAC signaling header type I. MAC PDU without data payload, with a 3-bit type field, see Table 8 for type encoding definitions. | Figure 33, Figure 34 | Table 17 |

6.3.2.1.2.2 MAC signaling header type II

Table 16.—Type field encodings for MAC signaling header type II

| Type field | MAC header type (with HT/EC = 0b11) | Reference figure | Reference table |
| --- | --- | --- | --- |
| 0 | Feedback header, with another 4-bit type field; see Table 18 for its type encodings. | Figure 33, Figure 34 | Table 17 |
| 1 | Extended relay MAC Signaling Header Type II Extended M2M device MAC Signaling Header Type II | Figure 37 | Table 20 Table 22a |

6.3.2.1.2.2.3 Extended M2M device MAC signaling header type II

This type of MAC header is UL-specific. There is no payload following the MAC header. The Extended M2M device MAC signaling header type II is illustrated in Figure 37. Table 22a describes the encoding of the 3-bit extended type field following the type field.

Table 22a.—Extended Type field encodings for extended M2M device MAC signaling header type II

| Extended Type field | MAC header type | Reference table |
| --- | --- | --- |
| 0 | M2M abnormal power down report header | Table 23a |
| 1 ~ 7 | Reserved |  |

6.3.2.1.2.2.3.1 M2M abnormal power down report header

When an M2M device in connected state detects an abnormal power down event, it sends an M2M abnormal power down report signaling header indicating that an abnormal or involuntary power down has occurred. The M2M abnormal power down report signaling header is defined in Table 670b.

Table 23a – Description of fields in M2M abnormal power down report header

| Name | Length (bit) | Description |
| --- | --- | --- |
| CID | 16 | M2M device’s basic connection identifier. |
| Emergency Type | 2 | 0b00: power outage 0b01~0b11: Reserved |
| CMAC_KEY_COUNT | 16 | The M2M device’s current value of the CMAC_KEY_COU NT, which is used in the generation of the CMAC keys |
| HCS | 8 | Header Check Sequence (same usage as HCS entry in Table 5). |

6.3.2.1.3 DL MAC header without payload

This MAC header format is applicable to DL only. The MAC header is not followed by any MAC PDU payload and CRC.

6.3.2.1.2.1 DL MAC signaling header type I

For this MAC header format, there is no payload following the MAC header. The DL MAC signaling header type I is illustrated in Figure 24. Table 8a describes the encoding of the 3-bit Type field following the EC field.

Table 8a.— Type field encodings for DL MAC signaling header type I

| Type field (3 bits) | MAC header type (with HT/EC = 0b10) | Reference table |
| --- | --- | --- |
| 0 | M2M abnormal power down confirmation header | Table 9a |
| 1 ~ 7 | Reserved |  |

6.3.2.1.2.1.1 M2M abnormal power down confirmation header

An M2M abnormal power down confirmation signaling header shall be transmitted by the BS in response to a received abnormal power down report. The M2M abnormal power down confirmation signaling header is defined in Table 9a.

Table 9a – Description of fields in M2M abnormal power down confirmation header

| Name | Length (bit) | Description |
| --- | --- | --- |
| Type | 3 | The type of M2M abnormal power down confirmation header is defined in Table 8a. |
| Reserved | 35 | Set to 0. |
| HCS | 8 | Header Check Sequence (same usage as HCS entry in Table 5). |

----------------------------------------------------- End of Proposed Text -----------------------------------------------------

[Remedy 5 – comment #84: Modify the following text on page 15, line 39 as follows:]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

6.3.35.2 Abnormal power down reporting in idle mode

The BS, upon receiving the ranging code, may include a CDMA Allocation IE in the next frame identifying the M2M device and provide an allocation sufficiently large to allow the M2M device to transmit a RNG-REQ message including a Ranging Purpose Indication TLV and the CMAC/HMAC Tuple.

The M2M device shall not repeat sending of a ranging code if it does not receive an allocation from the BS.

Upon receiving this allocation, the M2M device shall transmit a RNG-REQ message including a Ranging Purpose Indication TLV with bit 5 set to 1 (power outage) and a valid HMAC/CMAC Tuple. The M2M device shall not repeat sending of a ranging code if it does not receive an allocation from the BS.

The M2M device shall start the T3 timer at the transmission of the RNG-REQ message. The abnormal power down reporting procedure is over after receiving the RNG-RSP message or after the expiration of T3.

----------------------------------------------------- End of Proposed Text -----------------------------------------------------

[Remedy 6 – comment #84: Modify the following text on page 22, line 49 as follows:]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

10. Parameters and constants

10.1 Global values

| System | Name | Time reference | Minimum value | Default value | Maximum value |
| --- | --- | --- | --- | --- | --- |
| … | … |  |  |  |  |
| SS, MS | T3 | Ranging response reception timeout following the transmission of a ranging request. | — | OFDMA: 60 msec: RNG-RSP after CDMA ranging or RNG-REQ during initial or periodic ranging 50 msec: 1) RNG-RSP after RNG-REQ during HO to negotiated target BS 2) RNG-RSP after RNG-REQ during abnormal power down report 200 msec: RNG-RSP after RNG-REQ during HO to non-negotiated target BS 200 msec: RNG-RSP after RNG-REQ during location update or re-entry from idle mode 80 ms: RNG-RSP after CDMA ranging using Initial/HO Ranging Code set 1 or 2 during initial ranging/ handover/location update/ reentry from idle mode 100 ms: 1) RNG-RSP after RNG-REQ following usage of Initial Ranging Code set 1 during initial ranging, 2) RNG-RSP after RNG-REQ following usage of HO Ranging code set 1 or 2 during handover to negotiated target BS, or 3) RNG-RSP after RNG-REQ on the Primary Management Connection during initial ranging/handover to negotiated target BS /location update/reentry from idle mode. | 200 ms |
| M2M device | Abnormal Power Down Confirmation timer | Abnormal power down confirmation reception timeout following the transmission of an abnormal power down report | — | 50 msec | — |

----------------------------------------------------- End of Proposed Text -----------------------------------------------------

[Remedy 7 – comment #84: Modify the following text on page 25, line 16 as follows:]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

Table 685—RNG-REQ message encodings

| NAME | Type | Length | Value | PHY Scope |
| --- | --- | --- | --- | --- |
| M2M Fast Bandwidth Request | 43 | 2 | Bit 0: M2M Bandwidth Request Indicator 0: No bandwidth request 1: bandwidth request Bits 1-15: BR size | OFDMA |
| Emergency Type | 44 | 1 | Bit 0-1: Emergency type for M2M abnormal power down report as follows: 00 – power outage 01~11 – Reserved Bit 2-7: Reserved | OFDMA |

----------------------------------------------------- End of Proposed Text -----------------------------------------------------