| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MGID, CID and Service flows |  |
| Date Submitted | 2011-11-04 |  |
| Source(s) | Anil Agiwal, Anshuman Nigam, Hyunjeong Kang, Rakesh Taori Samsung Electronics | E-mail: anilag@samsung.com, anshun@samsung.com, hyunjeong.kang@samsung.com |
| Re: | IEEE 802.16-11/0027, IEEE 802.16 Working Group Letter Ballot #33 |  |
| Abstract | This contribution proposes relationship between MGID, CID and Service flows |  |
| Purpose | To be discussed and adopted in latest version of IEEE 802.16p/D1 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MGID, CID and Service flows

Anil Agiwal, Anshuman Nigam, Hyunjeong Kang, Rakesh Taori

Samsung Electronics

# Introduction

The following MGID issues needs to be addressed in draft [1]. The current text in draft [1] does not clarify whether an M2M device is assigned one or multiple MGIDs. How are the MGIDs, CIDs and service flows related to each other?

# Proposal

A multicast service flow for an M2M device is assigned a distinct MGID. So an M2M device subscribed to multiple multicast service flows will be assigned multiple MGIDs. The multicast service flow for an M2M device is not assigned any CID in the DSx messages. The M2M multicast assignment IE (defined in 8.4.5.3.35) with MGID, indicates the resources for the PHY burst carrying the MAC PDU for a multicast service flow (identified by MGID). The CID field in the MAC header of the MAC PDU carrying the MAC SDUs for the multicast service flow is set to ‘0xFE9F’.  Note that the CID ‘0xFE9F’ is not reserved. It can be assigned to unicast service flows as usual even when it is used in the MAC header for MAC PDUs of multicast service flows for M2M device.

The advantages of this proposal are:

- When an MS moves from one BS to another BS in an M2M_GROUP_ZONE, there is no need to update CIDs for multicast service flows.

- Alternately, coordination amongst the BSs of M2M_GROUP_ZONE for CID assignment, in-order to reduce the CID updates, is not needed.

# Proposed text with change marks

Change 1: Modify Page 27, lines 47-56 as follows

11.13 Service flow management encodings

Add new subclause 11.13.46

11.13.46 MGID

The value of this field specifies MGID that is used for the associated flow. During connected mode, the MGID may be added by DSA-REQ message and may be changed by DSC-REQ message. If the MGID is included in the DSA-REQ and DSC-REQ messages then the CID shall not be included.

Change 2: Delete the field ‘CID’ from table 229a, on page 10

Change 3: Add a new paragraph after line 60, on page 4

To access the multicast service, the M2M device that is assigned an MGID shall decode the M2M multicast assignment IE (defined in 8.4.5.3.35). If the assigned MGID is included in the M2M multicast assignment IE, the M2M device shall obtain the multicast burst according to the instruction in the M2M multicast assignment IE. The CID field in the MAC header of the MAC PDU (in multicast burst) carrying the MAC SDUs for the multicast service flow is set to ‘0xFE9F’.

Change 4: Add the following subclause 8.4.5.3.2

8.4.5.3.2 DL-MAP Extended IE format

8.4.5.3.2.3 DL-MAP Extended-3 IE encoding format

Modify Table 409 as Indicated

Table 409—Extended-3 DIUC code assignment for Extended-2 DIUC = 15

| Extended-3 DIUC (hexadecimal) | Usage |
| --- | --- |
| 0x0 | Power Boosting IE |
| 0x1 | MR DL-MAP Monitor IE |
| 0x2 | DL_Burst_Transmit_IE |
| 0x3 | OFDMA Periodic_Channel_Measurement_IE |
| 0x4 | M2M multicast assignment IE |
| 0x45-0xF | Reserved |

Change 5: Add new subclause 8.4.5.3.35

Add new subclause 8.4.5.3.xx as indicated

8.4.5.3.35 M2M Multicast Assignment IE format

In the DL-MAP, a BS shall transmit the M2M Multicast Assignment MAP IE to indicate the DL allocation in which M2M Multicast data is sent.

Table xxx—M2M Multicast Assignment IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| M2M Multicast Assignment IE(){ | - | - |
| Extended-2 DIUC | 4 | M2M Multicast Assignment IE () = 0xF (Extended-3 DIUC) |
| Length | 8 | Length in bytes |
| Extended-3 DIUC | 4 | 0x04 |
| MGID | 15 |  |
| DIUC | 4 |  |
| OFDMA Symbol Offset | 8 | The offset of the OFDMA symbol measured in OFDMA symbols from beginning of the DL frame in which the DL-MAP is transmitted. Counting from the frame preamble and starting from 0 |
| Subchannel offset | 7 | The offset of the first OFDMA symbol of the MBS region measured in OFDMA symbols from beginning of this DL frame |
| No. Subchannels | 7 |  |
| NO. OFDMA symbols | 7 |  |
| Boosting | 3 | Refer to Table 321 |
| Repetition Coding Indication | 2 | 0b00—No repetition coding 0b01—Repetition coding of 2 used 0b10—Repetition coding of 4 used 0b11—Repetition coding of 6 used |
| if !(byte boundary) { |  |  |
| Padding Nibble | variable | Padding to reach byte boundary |
| } |  |  |
| } | - | - |

# References

[1] 80216p/D1: Enhancements to Support Machine-to-Machine Applications