| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Additional Paging Listening Interval for Group Paging |  |
| Date Submitted | 2011-11-04 |  |
| Source(s) | Ming-Hung Tao, Ying-Chuan Hsiao ITRI | E-mail: mhtao@itri.org.tw *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Category: LB33 / Area: Chapter 6.3.2.3 (MAC management messsages) |  |
| Abstract | This contribution proposes changes to the Section 6.3.2.3 of IEEE P802.16p/D1 |  |
| Purpose | To be discussed and adopted by TGp |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Additional Paging Listening Interval for Group Paging

Ming-Hung Tao, Ying-Chuan Hsiao

ITRI

# Introduction

In current draft, each M2M device has its own individual paging cycle and paging offset, and monitors the paging message at specific periods determined by the individual paging cycle and paging offset. For the MGID-based group paging, there is no additional paging cycle and paging offset defined. Therefore, the M2M device monitors the group paging instruction at the same periods as monitoring the individual paging instruction. The following introduces several overhead/complexity issues induced by the current paging operations.

- The group paging instructions have to be duplicated n times if there are n different paging listening intervals existing the among the M2M devices. If these M2M devices have no common paging cycle and paging offset configuration, the control message overhead will become too much.

- Whenever a paging controller is going to instruct an BS to page an MGID (for location update or network entry), it has to also inform the BS about the associated paging cycle and paging offset values, so that the BS can know when to broadcast the paging message. If the M2M devices belonging to the same MGID have many individual paging cycles and paging offsets, the backhaul overheads will increase seriously.

For the above reasons, we suggest that the M2M device shall be assigned an additional paging cycle and an additional paging offset for each MGID assigned to it. For the M2M devices subscribing the same MGID, their additional paging cycles and additional paging offsets shall be the same.

# Text Proposal

Adopt the following text modifications to IEEE P802.16p/D1:

--------------------------  Begin Text Proposal   -------------------------------

[Remedy #1: Insert the following blue text to page 9, line 21]

6.3.2.3.26 DREG-CMD (de/reregister command) message

When the DREG-CMD message is sent to an M2M device, the following TLVs may be included:

M2M device-specific Idle mode timer

Length of the maximum interval between two consecutive location updates while the M2M 			device is in idle mode

Localized_Idle_Mode flag

Bit 0: Indicator of the Localized Idle Mode for fixed M2M device

0b0: The M2M device enters the idle mode except the procedures in 6.3.22.11.2.

0b1: The M2M device enters the localized idle mode.

Bits 1-7: reserved

When Localized_Idle_Mode flag is not present in the DREG-CMD message, the M2M device enters the 	idle mode except the procedures in 6.3.22.11.2.

Max number of paging cycle

Max number of paging cycle for M2M device to wait for MOB_PAG-ADV with M2M report 			code. The unit is the duration of the paging cycle

<INS> MGID_Paging_Cycle

The additional paging cycle assigned to the M2M device for each assigned MGID

MGID_Paging_Offset

The additional paging offset assigned to the M2M device for each assigned MGID </INS>

[Remedy #2: Insert the following blue text from page 29, line 17]

11.14.1 M2M device-specific parameter

| Name | Type | Length | Value | Scope |
| --- | --- | --- | --- | --- |
| M2M device-specific Idle mode timer | 53 | 3 | Length of the maximum interval between two consecutive location updates while the M2M device is in idle mode | DREG-CMD |
| Transmission Type | 54 | 1 | Bit 0: Transmission type indicator of the UL data transmission 0b1: allowed to send data only after receiving a paging message with M2M report code Bits 1-7: Reserved. | DREG-CMD |
| Max number of paging cycle | 55 | 2 | This is max number of paging cycle for M2M device to wait for MOB_PAG-ADV with M2M report code. The unit is the duration of the paging cycle. | DREG-CMD |
| Localized_Idle_Mode_Accepted_Flag | 56 | 1 | Bit 0: Indicator of the Localized Idle Mode for fixed M2M device 0b0:The M2M device enters the idle mode except the procedures in 6.3.22.11.2 0b1:The M2M device enters the localized idle mode Bits 1-7: Reserved | DREG-REQ DREG-CMD |
| <INS> MGID_Paging_Cycle | 57 | 2 | Bits 0–15: Cycle in which the paging instruction for the assigned MGID is transmitted. | DREG-CMD |
| MGID_Paging_Offset | 58 | 2 | Bits 0–15: Determines the frame within the cycle from which the paging interval starts. Shall be smaller than MGID_Paging_Cycle value. | DREG-CMD </INS> |

-----------------------------  End Text Proposal   ----------------------------------

# Reference

[1] IEEE 802.16p-10/0004r3: IEEE 802.16p Machine to Machine (M2M) System Requirements Document (SRD).

[2] IEEE P802.16p/D1: Air Interface for Broadband Wireless Access Systems —Enhancements to Support Machine-to-Machine Applications.

[3] IEEE 802.16p-10/0005r1: Machine to Machine (M2M) Communications Technical Report.