| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of MGID field description |  |
| Date Submitted | 2011-10-31 |  |
| Source(s) | Namgi Kim Kyonggi University Soojung Jung, Jaesun Cha, Chulsik Yoon, Kwangjae Lim ETRI | Voice: +82-31-249-9662 E-mail: ngkim@kyonggi.ac.kr Voice: +82-42-860-5456 E-mail: sjjung@etri.re.kr |
| Re: | Comments on IEEE 802.16 Letter Ballot #33 (on P802.16p/D1) |  |
| Abstract | This contribution provides a text for clarification of MGID field description |  |
| Purpose | To be discussed and adopted by Working Group Letter Ballot |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of MGID field description

Namgi Kim

Kyonggi University

Soojung Jung, Jaesun Cha, Chulsik Yoon, Kwangjae Lim

ETRI

# Proposal

In the current IEEE 802.16p/D1 [1], there are a few unclear description related to MGID field. So, in this contribution, we proposed a clarified texts to fix them for IEEE 802.16p draft.

# References

[1] IEEE 802.16p/D1, “[Draft] Enhancements to Support Machine-to-Machine Applications”, Oct. 2011.

# Proposed Text

We differentiated any texts modified by this contribution by using a different color as follows.

- Texts added by this contribution: <INS>blue texts</INS>.

- Texts deleted by this contribution: <DEL>red texts</DEL>.

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

[Remedy 1: Modify section 6.3.2.3.6 as follows;]

### 6.3.2.3.6 RNG-RSP (ranging response) message

Add the following text at the end of the subclause 6.3.2.3.6 as indicated

The following TLV parameter may be included in a RNG-RSP message to update MGID in an M2M device.

MGID Update

Bits 0-<DEL>15</DEL><INS>14<INS>: current MGID value

Bits <DEL>16-31</DEL><INS>15-29</INS>: new MGID value

<INS>Bits 30-31: paddings, will be set to 0</INS>

[Remedy 2: Modify section 11.6 as follows;]

### 11.6 RNG-RSP management message encodings

Change the contents of Table 688 as indicated

Table 688 – RNG-RSP message encodings

| Name | Type(1byte) | Length | Value | PHY scope |
| --- | --- | --- | --- | --- |
| … | ... | … | ... | ... |
| M2M Short Data Burst Response | 45 | 1 | Bits 0-1: accept or reject Short Data Burst request 0b0: reject 0b1: accept Bits 2-3: action code, applied to Bits 0-1 = 0b00 only 0b00: network re-entry 0b01-0b11: Reserved Bits 4-7: Reserved | OFDMA |
| Temp CID Timer | 46 | 1 | Life time duration for the Basic CID assigned by BS | OFDMA |
| M2M Short Data Burst Confirmation | 47 | 1 | Bit 0: Short Data Burst confirmation b0 - NACK b1 -ACK Bits 1-7: Reserved | OFDMA |
| M2M Short Data Burst | 48 | Variable | M2M Short Data Burst message content up to 140 bytes<INS>.<INS> Padding bits to align boundary of byte. | OFDMA |
| MGID Update | 49 | 4 | Bits 0-<DEL>15</DEL> <INS>14</INS>: current MGID value Bits <DEL>16-31</DEL><INS>15-29</INS>: new MGID value <INS>Bit 30-31: paddings, will be set to 0</INS> | OFDMA |

[Remedy 3: Modify section 11.17.5 as follows;]

### 11.17.5 M2M device group paging parameter

…

The following TLV element shall appear in each M2M device group paging parameter TLV if Action code is set to 0b11 (which identifies Re-assignment of MGID).

| Name | Type | Length | Value |
| --- | --- | --- | --- |
| MGID Re-assignment for M2M | 156.4 | 2 | <DEL>New MGID (15bits)</DEL> <INS>Bits 0-14: New MGID value Bit 15: Padding, will be set to 0</INS> |

…

----------------- End of the text proposal ---------------------------------------------------------------------------------------