| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of MGMC message |  |
| Date Submitted | 2011-10-31 |  |
| Source(s) | Namgi Kim Kyonggi University Soojung Jung, Jaesun Cha, Chulsik Yoon, Kwangjae Lim ETRI | Voice: +82-31-249-9662 E-mail: ngkim@kyonggi.ac.kr Voice: +82-42-860-5456 E-mail: sjjung@etri.re.kr |
| Re: | Comments on IEEE 802.16 Letter Ballot #33 (on P802.16p/D1) |  |
| Abstract | This contribution provides a text for MGMC message clarification |  |
| Purpose | To be discussed and adopted by Working Group Letter Ballot |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of MGMC message

Namgi Kim

Kyonggi University

Soojung Jung, Jaesun Cha, Chulsik Yoon, Kwangjae Lim

ETRI

# Proposal

Currently, the connection type transmitting the MGMC message is TBD [1]. The connection type for the MGMC message may be broadcast or multicast. If the message is transmitted through broadcast connection, M2M devices even for multiple MGID groups get the message at the same time and change the MGIDs simultaneously. Therefore, we have to clarify the MGMC message to accommodate multiple MGID changes simultaneously. On the other hand, if the message is sent through multicast connection based on a MGID, only M2M devices with the designated MGID get the message. In this case, the MGID field in the message is not needed because a MGID group is already specified. Therefore, the MGMC message has to exclude MGID field in the message. Consequently, in this contribution, we suggest to choose one between two options.

# References

[1] IEEE 802.16p/D1, “[Draft] Enhancements to Support Machine-to-Machine Applications”, Oct. 2011.

# Proposed Text

We differentiated any texts modified by this contribution by using a different color as follows.

- Texts added by this contribution: <INS>blue texts</INS>.

- Texts deleted by this contribution: <DEL>red texts</DEL>.

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

[Choose one between option 1 and option 2]

[Option 1: Modify section 6.3.1, 6.3.2.3, and 6.3.2.3.99 as follow;]

### 6.3.1 Addressing and connections

The BS may use the M2M Group MAC Control (MGMC) message with the MGID<INS>s</INS> to send the <DEL>same</DEL> information to multiple M2M devices. The M2M device may respond to acknowledge this message with M2M ACK MAC Control (MAMC) message.

### 6.3.2.3 MAC management messages

Change the contents of Table <DEL>54</DEL><INS>53</INS> as indicated

Table <DEL>54<DEL><INS>53<INS> – MAC management messages

| Type | Message name | Message description | Connection |
| --- | --- | --- | --- |
| … |  |  |  |
| 110 | MOB_MTE-IND | Multicast transmission end indicator | TBD |
| 111 | MGMC | M2M Group MAC Control message | <DEL>TBD</DEL> <INS>Broadcast</INS> |
| 112 | MAMC | M2M ACK MAC Control message | Basic |
| 113100-255 |  | Reserved |  |

### 6.3.2.3.99 MGMC(M2M group MAC control) message

The MGMC message may be sent to a group of M2M devices that belong to the same M2M device group (defined by an MGID) to indicate parameters and/or instructions. The BS may send the MGMC message to M2M devices in normal operation mode.

Table 229b—MGMC message format

| Field | Size(bits) | Notes |
| --- | --- | --- |
| MGMC_Message_Format() { |  |  |
| Management Message Type = 111 | 8 |  |
| <INS>Num_MGID</INS> | <INS>[TBD]</INS> | <INS>Number of MGID</INS> |
| <INS>For(i=1; i<=Num_MGID; i++) {</INS> |  |  |
| MGID | 15 |  |
| Action Code | 2 | Use to identify the purpose if this message 0b00: re-assignment of MGID value 0b01-0b11: Reserved |
| If (Action Code == 0x00) { |  |  |
| New MGID | 15 | New MGID value to be assigned |
| } |  |  |
| <INS>}</INS> |  |  |
| <INS>}</INS> |  |  |

[Option 2: Modify section 6.3.2.3 and 6.3.2.3.99 as follow;]

### 6.3.2.3 MAC management messages

Change the contents of Table <DEL>54</DEL><INS>53</INS> as indicated

Table <DEL>54<DEL><INS>53<INS> – MAC management messages

| Type | Message name | Message description | Connection |
| --- | --- | --- | --- |
| … |  |  |  |
| 110 | MOB_MTE-IND | Multicast transmission end indicator | TBD |
| 111 | MGMC | M2M Group MAC Control message | <DEL>TBD</DEL> <INS>Multicast</INS> |
| 112 | MAMC | M2M ACK MAC Control message | Basic |
| 113100-255 |  | Reserved |  |

### 6.3.2.3.99 MGMC(M2M group MAC control) message

The MGMC message may be sent to a group of M2M devices that belong to the same M2M device group (defined by an MGID) to indicate parameters and/or instructions. The BS may send the MGMC message to M2M devices in normal operation mode.

Table 229b—MGMC message format

| Field | Size(bits) | Notes |
| --- | --- | --- |
| MGMC_Message_Format() { |  |  |
| Management Message Type = 111 | 8 |  |
| <DEL>MGID</DEL> | <DEL>15</DEL> |  |
| Action Code | 2 | Use to identify the purpose if this message 0b00: re-assignment of MGID value 0b01-0b11: Reserved |
| If (Action Code == 0x00) { |  |  |
| New MGID | 15 | New MGID value to be assigned |
| } |  |  |
| <INS>}</INS> |  |  |

----------------- End of the text proposal ---------------------------------------------------------------------------------------