| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of AAI-MGMC message |  |
| Date Submitted | 2011-11-10 |  |
| Source(s) | Namgi Kim Kyonggi University Soojung Jung, Jaesun Cha, Chulsik Yoon, Kwangjae Lim ETRI | Voice: +82-31-249-9662 E-mail: ngkim@kyonggi.ac.kr Voice: +82-42-860-5456 E-mail: sjjung@etri.re.kr |
| Re: | Comments on IEEE 802.16 Letter Ballot #34 (on P802.16p/11-0033) |  |
| Abstract | This contribution provides a clarification for AAI-MGMC message |  |
| Purpose | To be discussed and adopted by Working Group Letter Ballot |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of AAI-MGMC message

Namgi Kim

Kyonggi University

Soojung Jung, Jaesun Cha, Chulsik Yoon, Kwangjae Lim

ETRI

# Proposal

According to Table 683 of current pre-draft document [1], the AAI-MGMC message is transmitted through broadcast connection. So, the AAI-MGMC message may be sent to M2M devices for multiple MGID groups at the same time and change the MGIDs simultaneously. Therefore, we have to clarify the AAI-MGMC message to accommodate multiple MGID changes simultaneously.

# References

[1] IEEE 802.16p/11-0033, “[Pre-Draft] Enhancements to Support Machine-to-Machine Applications”, Oct. 2011.

# Proposed Text

We differentiated any texts modified by this contribution by using a different color as follows.

- Texts added by this contribution: blue texts

- Texts deleted by this contribution: red texts

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

[Modify section 6.2.1.3.1 and 6.2.3.65 as follow;]

### 6.2.1.3.1 M2M Group Identifier

The ABS may use the M2M device group MAC Control (MGMC) message with the MGIDs to send the same information to multiple M2M devices. The AMS may respond to acknowledge this message with AAI-MSG-ACK defined in 6.2.3.36.

### 6.2.3.65 AAI-MGMC(M2M device group MAC control) message

The AAI-MGMC message may be sent to a group of M2M devices that belong to the same M2M device group (defined by a MGID) to indicate parameters and/or instructions. The ABS may send the AAI-MGMC message to M2M devices in connect state.

Table 763b—AAI-MGMC message field description

| Field | Size(bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Action Code | 2 | Use to identify the purpose if this message 0b00: re-assignment of MGID value 0b01-0b11: Reserved |  |
| If (Action Code == 0x00) { |  |  |  |
| For(i=1; i<=Num_MGID; i++) { |  | Number of MGID to be updated[1..TBD]. | Presented if a MGID needs to be updated |
| Current MGID | 15 | Current MGID value to be assigned |  |
| New MGID | 15 | New MGID value to be assigned |  |
| } |  |  |  |
| } |  |  |  |

----------------- End of the text proposal ---------------------------------------------------------------------------------------