| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modification of AAI-PAG-ADV for Support of Backward Compatibility |  |
| Date Submitted | 2011-11-06 |  |
| Source(s) | Jaesun Cha, Soojung Jung, Chulsik Yoon, Kwangjae Lim ETRI | Email: jscha@etri.re.kr |
| Re: | WG Letter Ballot #34 |  |
| Abstract | This contribution proposes some modification of AAI-PAG-ADV message to support backward compatibility. |  |
| Purpose | For discussion in 802.16p TG and adoption in to IEEEP802.16p-11/0035 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modification of AAI-PAG-ADV Message for Support of Backward Compatibility

Jaesun Cha, Soojung Jung, Chulsik Yoon, Kwangjae Lim

ETRI

# Introduction

There is a backward compatibility problem in the current format of AAI-PAG-ADV message. There are three for-loops in AAI-PAG-ADV message. The first for-loop is defined for individual paging and shared by legacy stations and M2M devices. And, new parameters were added into the first for-loop to support M2M functionality. However, legacy stations can’t decode the newly added parameters. It means that the legacy stations can’t jump to the next iteration of the for-loop and may fail in decoding the AAI-PAG-ADV message.

In this contribution, we propose to define additional for-loop which can be used for individual paging for M2M devices only to support backward compatibility. All new parameters defined for M2M devices are moved to the proposed for-loop.

# Proposed Texts

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

[Modify Table 706 on page 17 as follows;]

Table 706 – AAI-PAG-ADV message format

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … | … | … | … |
| For (i=0; i<M; i++) { |  |  | M equals the number of bits in Paging_Group_IDs bitmap whose bit is set to 1. |
| For (j=0; j<Num_AMSs; j++) { |  | Num_AMSs indicates the number of paged AMSs in a corresponding paging group 1..32 |  |
| Deregistration Identifier | 18 | Used to indicate Deregistration ID for AMS or M2M device to be paged (Deregistration Identifier and Paging Cycle are used to identify each paged AMS) 0..218 - 1 | Present if the S-SFH Network Configuration bit == 0b0 |
| MAC Address Hash | 24 | Used to identify the AMS to be paged | Present if the S-SFH Network Configuration bit == 0b1 |
| Paging Cycle | 4 | Used to indicate Paging cycle for the AMS to be paged 0x00: 4 superframes 0x01: 8 superframes 0x02: 16 superframes 0x03: 32 superframes 0x04: 64 superframes 0x05: 128 superframes 0x06: 256 superframes 0x07: 512 superframes 0x08: 32768 superframes 0x09: 262144 superframes 0x10: 4194304 superframes 0x11080x08-0x15: Reserved | Present if the S-SFH Network Configuration bit == 0b0 Values 0x08-0x10 shall be applied to M2M devices only. |
| Action Code | 1 | Used to indicate the purpose of the AAI-PAG-ADV message 0b0: perform network reentry 0b1: perform ranging for location update |  |
| If (Action Code == 0b0) { |  |  |  |
| Initial ranging backoff start | 4 | Indicate the initial backoff window size for M2M devices |  |
| } |  |  |  |
| M2M network access type | 2 | Indicate the network re-entry type for M2M device; 0b00: Resource allocation (i.e., Assignment A-MAP offset) for AAI-RNG-REQ 0b01: dedicated ranging channel allocation in AAI-PAG-ADV 0b10: dedicated ranging channel allocation in broadcast assignment A-MAP IE 0b011: No dedicated ranging channel |  |
| If (M2M network access type == 0b00) { |  |  |  |
| Assignment A-MAP offset for AAI-RNG-REQ |  | Indicate offset that Assignment A-MAP IE for AAI-RNG-REQ message is transmitted. |  |
| } |  |  |  |
| M2M Report code | 1 | Indication for the M2M device to send the uplink report 0b0: reserved 0b1: Send uplink report | Present if M2M is supported |
| } // End of for (j=0;j<Num_AMSs;j++) |  |  |  |
| } // End of for (i=0;i<M;i++) |  |  |  |
| For (i=0; i<M; i++) { |  | M equals the number of bits in Paging_Group_IDs bitmap whose bit is set to 1. | Present only for M2M devices |
| For (j=0; j<Num_devices; j++) { |  | Num_devices indicates the number of paged M2M devices in a corresponding paging group 1..32 |  |
| Deregistration Identifier | 18 | Used to indicate Deregistration ID for M2M device to be paged (Deregistration Identifier and Paging Cycle are used to identify each paged M2M device) 0..218 - 1 | Present if the S-SFH Network Configuration bit == 0b0 |
| MAC Address Hash | 24 | Used to identify the M2M device to be paged | Present if the S-SFH Network Configuration bit == 0b1 |
| Paging Cycle | 4 | Used to indicate Paging cycle for the AMS to be paged 0x00: 4 superframes 0x01: 8 superframes 0x02: 16 superframes 0x03: 32 superframes 0x04: 64 superframes 0x05: 128 superframes 0x06: 256 superframes 0x07: 512 superframes 0x08: 32768 superframes 0x09: 262144 superframes 0x10: 4194304 superframes 0x1108-0x15: Reserved | Present if the S-SFH Network Configuration bit == 0b0 |
| Action Code | 1 | Used to indicate the purpose of the AAI-PAG-ADV message 0b0: perform network reentry 0b1: perform ranging for location update |  |
| M2M network access type | 2 | Indicate the network re-entry type for M2M device; 0b00: Resource allocation (i.e., Assignment A-MAP offset) for AAI-RNG-REQ 0b01: dedicated ranging channel allocation in AAI-PAG-ADV 0b10: dedicated ranging channel allocation in broadcast assignment A-MAP IE 0b011: No dedicated ranging channel |  |
| If (M2M network access type == 0b00) { |  |  |  |
| Assignment A-MAP offset for AAI-RNG-REQ |  | Indicate offset that Assignment A-MAP IE for AAI-RNG-REQ message is transmitted. |  |
| } |  |  |  |
| M2M Report code | 1 | Indication for the M2M device to send the uplink report 0b0: reserved 0b1: Send uplink report | Present if M2M is supported |
| } // End of for (j=0;j<Num_devices;j++) |  |  |  |
| } // End of for (i=0;i<M;i++) |  |  |  |
| Initial ranging backoff start | 4 | Indicate the initial backoff window size for M2M devices. This parameter is applied for all M2M devices which are instructed to perform network reentry or location update by this message. | May be present if there is at least one M2M device which is instructed to perform network reentry or location update by this message. |
| Ranging backoff window indicatior | 1 | 0b0: increasing the ranging backoff window size by a factor of 2 per every ranging entry 0b1: decreasing the ranging backoff window size by a factor of 2 per every ranging entry as described in 6.2.18.7.2 | If initial ranging backoff start field is present |
| For (i=0; i<Num_MGID; i++) { |  | Num_MGID indicates the number of MGIDs included in this paging message [0..63] | Shall be included if the ABS sends DL multicast data for M2M after transmission of the AAI-PAG-ADV message. |
| … | … | … | … |

----------------- End of the text proposal ---------------------------------------------------------------------------------------