| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modification of Individual Paging Parameters |  |
| Date Submitted | 2011-11-06 |  |
| Source(s) | Jaesun Cha, Soojung Jung, Chulsik Yoon, Kwangjae Lim ETRI | Email: jscha@etri.re.kr |
| Re: | WG Letter Ballot #33 |  |
| Abstract | This contribution proposes some changes of individual paging parameters. |  |
| Purpose | For discussion in 802.16p TG and adoption in to 16p draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modification of Individual Paging Parameters

Jaesun Cha, Soojung Jung, Chulsik Yoon, Kwangjae Lim

ETRI

# Introduction

This contribution proposes some changes of individual paging parameters.

Ranging backoff start TLV may be included in MOB_PAG-ADV message to indicate a new initial ranging backoff start which can be used for network reentry or location update. And, it is included in M2M paging parameter TLV as a sub-TLV, which means that a different ranging backff start can be assigned each M2M device. It’s not efficient to assign different ranging backoff start to each M2M device. So, this contribution proposes to define ranging backoff TLV as a separate TLV and assign the same ranging backoff TLV to all M2M devices which perform network reentry or location update in response to individual paging notification.

In addition, this contribution proposes some editorial changes such as correction of length of M2M network access type parameter.

# Proposed Texts

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

[Modify texts in section 11.17.7 on page 31 as follows;]

11.17.7 M2M paging parameter

Following M2M paging parameter TLV may be included in MOB_PAG-ADV message.

| Name | Type | Length | Value | Scope |
| --- | --- | --- | --- | --- |
| M2M paging parameter | 158 | Variable | Compound TLV to be used in M2M individual paging operation | MOB_PAG-ADV |

Following TLV element may be included in MOB_PAG-ADV message.

| Name | Type | Length | Value |
| --- | --- | --- | --- |
| Raging backoff start | 158.1 | 1 | Initial backoff window size for ranging contention during network reentry or location update, expressed as a power of 2. Range: 0 ~ 15 |

Following TLV element may be included in MOB_PAG-ADV message when the Action code is set to 0b01 (i.e., Perform ranging to establish location and acknowledge message) or 0b10 (i.e., Enter network).

| Name | Type | Length | Value |
| --- | --- | --- | --- |
| M2M network access type | 158.21 | Variable | Indicate the network re-entryaccess type for M2M device; 0b00: Resource allocation (i.e., UL MAP IE offset) for RNG-REQ 0b01: dedicated ranging channel allocation in MOB_PAG-ADV 0b10: dedicated ranging channel allocation in UL-MAP Extended IE 0b11: No dedicated ranging channel Bits 0-7: Num_MAC_Address_Hash, Number of M2M devices For (i=0;i< Num_MAC_Address_Hashs; i++) { 32-bit M2M network access type } |

Following TLV element may be included in MOB_PAG-ADV message when the M2M network access type is set to 0b00.

| Name | Type | Length | Value |
| --- | --- | --- | --- |
| UL-MAP offset for RNG-REQ | 158.32 | Variable | Indicate offset that UL-MAP IE containing Fast Ranging IE for RNG-REQ message is transmitted. Bits 0-7: Num_MAC_Address_Hash, Number of M2M devices For (i=0;i< Num_MAC_Address_Hashs; i++) { RNG-REQ offset } |

11.17.8 Ranging backoff start

The ranging backoff start field TLV may be included in MOB_PAG-ADV message to indicate a new initial backoff start to be used for network reentry or location update. If this TLV is included in MOB_PAG-ADV message, all M2M devices which perform network reentry or location update in response to individual paging notification shall use this ranging backoff start TLV for initial ranging.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 153 | 1 | Initial backoff window size for ranging contention during network reentry or location update, expressed as a power of 2. Range: 0 ~ 15 | OFDMA |

----------------- End of the text proposal ---------------------------------------------------------------------------------------