| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Reliable MGID Update |  |
| Date Submitted | 2011-11-06 |  |
| Source(s) | Jaesun Cha, Soojung Jung, Chulsik Yoon, Kwangjae Lim ETRI | Email: jscha@etri.re.kr |
| Re: | WG Letter Ballot #33 |  |
| Abstract | This contribution proposes a configuration change count for reliable MGID update. |  |
| Purpose | For discussion in 802.16p TG and adoption in to 16p draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Reliable MGID Update

Jaesun Cha, Soojung Jung, Chulsik Yoon, Kwangjae Lim

ETRI

# Introduction

MGID may be updated when a network entity which manages MGID configuration changes the MGID configuration in certain M2M DEVICE GROUP ZONE. In this case, BS may instruct M2M devices to update MGID by using MGMC message or MOB_PAG-ADV message. However, it’s very difficult for BS to determine when the updated MGIDs will be used because some M2M devices may miss the broadcast messages. Therefore, BS has to transmit the broadcast message continuously until it confirms that all M2M devices receive the broadcast messages successfully and then it will use the update MGIDs. This approach is not efficient and requires much air interface resources.

In this contribution, we propose to define M2M device group zone change count to indicate the current version of MGID configuration. M2M devices shall check M2M DEVICE GROUP ZONE ID together with M2M device group zone change count. If M2M devices detect that M2M DEVICE GROUP ZONE ID is same but M2M device group zone change count is different, then they regard it as MGID update notification and request MGID information to serving BS or preferred BS.

# Proposed Texts

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

[Add the following row at the end of Table 678 – DCD channel encodings as follows;]

11.4 DCD management message encodings

11.4.1 DCD channel encodings

Add a new parameter at the end of Table 678 as indicated

Table 678 – DCD channel encodings

| Name | Type (1 byte) | Length | Value (variable length) | PHY scope |
| --- | --- | --- | --- | --- |
| M2M device group zone change count | 158 | 1 | The value is increased whenever configuration of M2M DEVICE GROUP ZONE ID is changed. The value rolls over from 0 to 8. | OFDMA |

----------------- End of the text proposal ---------------------------------------------------------------------------------------