| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Distinguish AMS and M2M device by MAC version |  |
| Date Submitted | 2011-11-6 |  |
| Source(s) | Yi-Ting Lin, Tsung-Yu Tsai Institute for Information Industry (III) | E-mail: eating@iii.org.tw |
| Re: | Change Request on Pre-Draft IEEE P802.16p-11/0033 |  |
| Abstract | This contribution proposes distinguishing scheme between AMS and M2M devices |  |
| Purpose | To be discussed and adopted by Machine-to-Machine(M2M) Task Group for IEEE P802.16p-11/0033 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Distinguish AMS and M2M device by MAC version

Yi-Ting Lin, Tsung-Yu Tsai

Institute for Information Industry (III)

# Introduction

Since the M2M devices may perform initial network entry in the normal ranging resource, MAC version, which is mandatory in MOB_RNG-REQ message, can be used for identifying the type of subscriber. Such that it is no necessary to carry additional M2M capability indicator in the following procedure, such as SBC or registration. In this contribution, new MAC version encoding is proposed for Standard 802.16p conformance with IEEE 802.16.1.

# Reference

[1] IEEE P802.16.1/D2, “IEEE Draft Standard for WirelessMAN-Advanced for Broadband Wireless Access Systems,” Oct. 2011.

[2] IEEE P802.16p-11/0033, “DRAFT Amendment to IEEE Standard for WirelessMAN-Advanced Air Interface for Broadband Wireless Access Systems – Enhancements to Support Machine-to-Machine Applications,” Oct. 2011.

- Text proposal for inclusion in IEEE P802.16p-11/0033

[Modify the following text into the IEEE P802.16p-11/0033]

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

11.1.3 MAC version encoding

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 148 | 1 | Maximum version number supported by the device sending the message. 0: Reserved 1–7: Indicates conformance with an earlier and/or obsolete version of IEEE Std 802.16 8: Indicates conformance with IEEE Std 802.16-2009 9: Indicates conformance with IEEE Std 802.16-2009 as amended by IEEE Std 802.16j-2009 10: Indicates conformance with IEEE Std 802.16-2009, as amended by IEEE Std 802.16j-2009, IEEE Std 802.16h-2010 and IEEE Std 802.16m-2011 12: Indicates conformance with IEEE Std 802.16.1, as amended by IEEE Std 802.16p 1113–255: Reserved | PMP: DCD, RNG-REQ |

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------