| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text related to idle mode optimization for fixed M2M devices |  |
| Date Submitted | 2011-11-6 |  |
| Source(s) | Soojung Jung, Jaesun Cha, Chulsik Yoon, Kwangjae Lim ETRI | Voice: +82-42-860-5928 E-mail: {sjjung}@etri.re.kr |
| Re: | IEEE 802.16 Working Group Letter Ballot #33 |  |
| Abstract | This contribution proposes a text of idle mode optimization for fixed M2M devices to be included in the 802.16p amendment. |  |
| Purpose | To be discussed and adopted by TGp for the 802.16p amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text related to idle mode optimization for fixed M2M devices

Soojung Jung, Jaesun Cha, Chulsik Yoon, Kwangjae Lim

ETRI

# Introduction

Functionalities of fixed M2M devices related to optimization of idle mode (e.g., BS-initiated localized idle mode entry and polling for periodic uplink non-real time data transmission) are optional. So, to support these functionalities of fixed M2M devices, the M2M device informs the BS of its capability during the registration process. After negotiation, these functionalities can be supported.

# References

[1] IEEE P802.16p/D1, “Draft amendment to IEEE standard for Air Interface for Broadband Wireless Access Systems: Enhancements to support Machine –to-Machine Applications,” October. 2011.

[2] IEEE P802.16Rev3/D2, “IEEE Draft Standard for Air Interface for Broadband Wireless Access Systems,” October. 2011

# Proposed text

We differentiated any texts modified by this contribution by using a different color as follows.

- Texts added by this contribution: blue underlined texts

- Texts deleted by this contribution: red strikeout texts

----------------- Start of the text proposal -------------------------------------------------------------------------------------

Add following text at the end of 6.3.2.3.7

6.3.2.3.7 REG-REQ (registration request) message

For a M2M device, the REG-REQ may contain the following TLV:

Mobility parameters support (11.7.13)

Add following text at the end of 6.3.2.3.8

6.3.2.3.8 REG-RSP (registration response) message

For a M2M device, the REG-RSP may contain the following TLV:

Mobility parameters support (11.7.13)

Modify the following text as indicated

11.7.13.1 Mobility features supported

The mobility features supported field indicates whether the MS supports mobility HO, sleep mode, and idle mode. A bit value of 0 indicates “not supported” while 1 indicates “supported.”

For M2M devices, the mobility features supported field also indicates whether the M2M device supports localized idle mode, and polling for periodic uplink data transmission. A bit value of 0 indicates “not supported” while 1 indicates “supported”.

| Type | Length | Value | Scopre |
| --- | --- | --- | --- |
| 31 | 1 | Bit 0: Mobility(HO)support Bit 1: Sleep mode support Bit 2: Idle mode support Bit 3: localized idle mode support of M2M device Bit 4:polling for periodic uplink data transmission of M2M device | REG-REQ, REG-RSP |

----------------- End of the text proposal ----------------------------------------------------------------------------------------