| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text related to localized idle mode for fixed M2M devices |  |
| Date Submitted | 2011-11-8 |  |
| Source(s) | Soojung Jung, Jaesun Cha, Chulsik Yoon, Kwangjae Lim ETRI Giwon Park LG Electronics Shantidev Mohanty Intel Corporation | Voice: +82-42-860-5928 E-mail: {sjjung}@etri.re.kr |
| Re: | IEEE 802.16 Working Group Letter Ballot #33 |  |
| Abstract | This contribution proposes a text of localized idle mode for fixed M2M devices to be included in the 802.16p amendment. |  |
| Purpose | To be discussed and adopted by TGp for the 802.16p amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text related to localized idle mode for fixed M2M devices

Soojung Jung, Jaesun Cha, Chulsik Yoon, Kwangjae Lim

ETRI

Giwon Park

LG Electronics

Shantidev Mohanty

Intel Corporation

# Introduction

For localized idle mode, only M2M device- initiated idle mode entry is included in the current AWD. But BS- initiated idle mode entry should also be included. In addition, as ’Localized_Idle_Mode_flag’ is used not only in the DREG-REQ message but also in the DREG-CMD message, it is appropriate to delete “request to” in the description of Localized_Idle_Mode_flag in the DREG-REQ message.

# References

[1] IEEE P802.16p/D1, “Draft amendment to IEEE standard for Air Interface for Broadband Wireless Access Systems: Enhancements to support Machine –to-Machine Applications,” October. 2011.

[2] IEEE P802.16Rev3/D2, “IEEE Draft Standard for Air Interface for Broadband Wireless Access Systems,” October. 2011

# Proposed text

We differentiated any texts modified by this contribution by using a different color as follows.

- Texts added by this contribution: blue underlined texts

- Texts deleted by this contribution: red strikeout texts

----------------- Start of the text proposal -------------------------------------------------------------------------------------

6.3.22.11.2 Idle mode optimizations for fixed M2M devices

Modify the following texts as indicated

Localized idle mode operation is used for fixed M2M devices in idle mode that do not need to perform the BS paging group based paging operation and location update operation. To eliminate the need for allocating the unnecessary paging information (i.e., Paging Group ID, Paging Controller ID), a fixed M2M device may set the Localized_Idle_Mode_Flag to 1 in the DREG-REQ message.

Localized idle mode for the fixed M2M device is initiated either by the M2M device or by its serving BS.

In case of M2M device-initiated localized idle mode entry, a fixed M2M device may include Localized_Idle_Mode_flag set to 1 in the DREG-REQ message.

When a BS receives a DREG-REQ message with the Localized_Idle_Mode_flag_set to 1 from an M2M device, it sends DREG-CMD with Localized_Idle_Mode_flag set to 1 or 0. Localized_Idle_Mode_flag set to 1 indicates that the BS accepts the M2M device’s localized idle mode operation request. In this case, the BS shall not include Paging Controller TLV in the DREG-CMD message and the M2M devices shall use the current serving BS as its preferred BS. Then the M2M device transitions to idle mode and does not perform BS paging group based paging operation and location update operation.

If Localized_Idle_Mode_flag is set to 0 in the DREG-CMD message, the M2M device enters into the idle mode except the procedures in 6.3.22.11.2.

Using BS-initiated localized idle mode entry, a BS may signal for a fixed M2M device to begin localized idle mode by sending a DREG-CMD message in unsolicited manner. This unsolicited DREG-CDM may include Localized_Idle_Mode_flag set to 1. When a fixed M2M device receives an unsolicited DREG-CMD with Localized_Idle_Mode_flag set to 1, the fixed M2M device may start the idle mode initiation procedures by sending DREG-REQ message with Localized_Idle_Mode_flag set to 1 or 0 in response to the unsolicited DREG-CMD message. Localized_Idle_Mode_flag set to 1in the DREG-REQ indicates that the M2M device accepts the BS’s localized idle mode operation request. If Localized_Idle_Mode_

flag is set to 0 in the DREG-REQ, it indicates that the M2M device does not accept the BS’s localized idle mode operation request. The detail initiation procedure according to the action code follows the mechanism defined in 6.3.22.1.

6.3.2.3.37 DREG-REQ (SS deregistration request) message

Modify the following text in page 9, line 32 as indicated

When the DREG-REQ message is sent to a BS, the following TLV may be included:

Localized_Idle_Mode_flag

Bit 0: Indicator of the Localized Idle Mode for fixed M2M device

0b0: The M2M device requests to enters the idle mode except the procedure in 6.3.22.11.2

0b1: The M2M device requests to enters the localized idle mode

Bit 1-7: reserved

----------------- End of the text proposal ----------------------------------------------------------------------------------------