| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text related to location update trigger conditions for M2M devices |  |
| Date Submitted | 2011-11-10 |  |
| Source(s) | Soojung Jung, Jaesun Cha, Chulsik Yoon, Kwangjae Lim ETRI Eunjong Lee, Youngsoo Yuk, Inuk Jung, Jinsam Kwak LG electronics | Voice: +82-42-860-5928 E-mail: {sjjung}@etri.re.kr |
| Re: | IEEE 802.16 Working Group Letter Ballot #33 |  |
| Abstract | This contribution proposes a text of location update procedure for M2M devices to be included in the 802.16p amendment. |  |
| Purpose | To be discussed and adopted by TGp for the 802.16p amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text related to location update conditions for M2M devices

Soojung Jung, Jaesun Cha, Chulsik Yoon, Kwangjae Lim

ETRI

Eunjong Lee, Youngsoo Yuk, Inuk Jung, Jinsam Kwak

LG electronics

# Introduction

There are five location update evaluation conditions: paging group based update, timer based update, power down update, MAC hash skip threshold and MBS update [2].

According to the current AWD [1], M2M DEVICE GROUP ZONE ID may be broadcasted in the DSD message. Therefore an M2M device may perform location update process if it detects that M2M Device Group Zone of selected preferred BS is different from previous one.

# References

[1] IEEE P802.16p/D1, “Draft amendment to IEEE standard for Air Interface for Broadband Wireless Access Systems: Enhancements to support Machine –to-Machine Applications,” October. 2011.

[2] IEEE P802.16Rev3/D2, “IEEE Draft Standard for Air Interface for Broadband Wireless Access Systems,” October. 2011

# Proposed text

We differentiated any texts modified by this contribution by using a different color as follows.

- Texts added by this contribution: blue underlined texts

- Texts deleted by this contribution: red strikeout texts

----------------- Start of the text proposal -------------------------------------------------------------------------------------

Add new sub-clause 6.3.22.11.3 as indicated (page 14, line 20)

6.3.22.11.3 Location update for M2M devices

6.3.22.11.3.1 Location update conditions

An M2M device in idle mode may also perform location update when it detects a change in M2M DEVICE GROUP ZONE ID.

6.2.18.7.6.1.1 M2M Device Group Zone based update

An M2M device shall perform a Location Update process when an M2M device detects that the selected preferred BS does not support its currently assigned M2M GROUP ZONE ID. The M2M device shall detect that by monitoring M2M DEVICE GROUP ZONE ID in the DCD message, which is transmitted by the preferred BS. During location update, M2M device’s M2MCIDs may be updated.

11.6 RNG-RSP management message encodings

Add the following parameter at the end of Table 688 as indicated

Table 688 – RNG-RSP message encodings

| NAME | Type | Length | Value | PHY Scope |
| --- | --- | --- | --- | --- |
| ….. | ….. | ….. | ….. | ….. |
| MGID M2MCID Update | 49 | 4 12 | Bits 0-15: current MGID M2MCID value Bits 16-31: new MGID M2MCID value Bits 32-95 : new MGSS | OFDMA |

----------------- End of the text proposal ----------------------------------------------------------------------------------------