| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text related to idle mode optimization for fixed M2M devices |  |
| Date Submitted | 2011-11-6 |  |
| Source(s) | Soojung Jung, Jaesun Cha, Chulsik Yoon, Kwangjae Lim ETRI | Voice: +82-42-860-5928 E-mail: {sjjung}@etri.re.kr |
| Re: | IEEE 802.16 Working Group Letter Ballot #34 |  |
| Abstract | This contribution proposes a text of idle mode optimization for fixed M2M devices to be included in the 802.16.1b amendment. |  |
| Purpose | To be discussed and adopted by TGp for the 802.16.1b amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text related to idle mode optimization for fixed M2M devices

Soojung Jung, Jaesun Cha, Chulsik Yoon, Kwangjae Lim

ETRI

# Introduction

Functionalities of fixed M2M devices related to optimization of idle mode (e.g., BS-initiated localized idle mode entry and polling for periodic uplink non-real time data transmission) are optional. So, in order to support these functionalities of fixed M2M devices, the M2M device informs the ABS of its capability during the registration process. After negotiation, these functionalities can be supported.

# References

[1] IEEE 802.16p-11/0033, “Draft amendment to IEEE standard for WirelessMAN-Advanced Air Interface for Broadband Wireless Access Systems: Enhancements to support Machine –to-Machine Applications,” October. 2011.

[2]IEEE P802.16.1/D2, “IEEE Draft for WirelessMAN-Advanced Air Interface for Broadband Wireless Access Systems,” October. 2011

# Proposed text

We differentiated any texts modified by this contribution by using a different color as follows.

- Texts added by this contribution: blue underlined texts

- Texts deleted by this contribution: red strikeout texts

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

6.2.3.8 AAI-REG-REQ

Modify Table 34 as indicated

Table 34- AAI-REG-REQ message field description

| Field | Size(bit) | Value/description | condition |
| --- | --- | --- | --- |
| …. | ….. |  |  |
| Mobility parameters support | 3 | Indicates the mobility parameters of M2M device Bit 0: mobility(HO) support 0b0: not support (M2M device is fixed) 0b1: support (M2M device is mobile) Bit 1: localized idle mode support 0b0: not support 0b1: support Bit 2: polling for periodic uplink data transmission support 0b0: not support 0b1: support | Present if need |

6.2.3.9 AAI-REG-RSP

Modify Table 35 692 as indicated

Table 35 692- AAI-REG-RSP message field description

| Field | Size(bit) | Value/description | condition |
| --- | --- | --- | --- |
| …. | ….. |  |  |
| Mobility parameters support | 3 | Indicates the mobility parameters of M2M device Bit 0: mobility(HO) support 0b0: not support (M2M device is fixed) 0b1: support (M2M device is mobile) Bit 1: localized idle mode support 0b0: not support 0b1: support Bit 2: polling for periodic uplink data transmission support 0b0: not support 0b1: support | Present if needed |

6.2.15.6 Registration

Add the following text at the end of section 6.2.15.6

A M2M device may inform of its capability related to the mobility in AAI-REG-REQ message. Upon receiving the AAI-REG-REQ with mobility parameter support, an ABS should include mobility parameter support in AAI-REG-RSP message in response to the AAI-REG-REQ message. If the M2M device indicates that it supports localized idle mode or polling for periodic uplink data transmission, the ABS may initiate localized idle mode entry or set polling procedure for periodic uplink non-real time data transmission during idle mode initiation. If the AMS does not indicate its capability related to the mobility, the ABS considers that the M2M device does not support these operations.

----------------- End of the text proposal ----------------------------------------------------------------------------------------