| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text related to location update trigger conditions for M2M devices |  |
| Date Submitted | 2011-11-8 |  |
| Source(s) | Soojung Jung, Jaesun Cha, Chulsik Yoon, Kwangjae Lim ETRI Eunjong Lee, Youngsoo Yuk, Inuk Jung, Jinsam Kwak LG electronics | Voice: +82-42-860-5928 E-mail: {sjjung}@etri.re.kr |
| Re: | IEEE 802.16 Working Group Letter Ballot #34 |  |
| Abstract | This contribution proposes a text of location update procedure to be included in the 802.16.1b amendment. |  |
| Purpose | To be discussed and adopted by TGp for the 802.16.1b amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text related to location update trigger conditions for M2M devices

Soojung Jung, Jaesun Cha, Chulsik Yoon, Kwangjae Lim

ETRI

Eunjong Lee, Youngsoo Yuk, Inuk Jung, Jinsam Kwak

LG electronics

# Introduction

There are four location update trigger conditions: paging group based update, timer based update, power down update, and MBS update [2].

According to the current AWD [1], M2M Group Zone ID may be broadcasted in the AAI-SCD message. Therefore an M2M device may perform location update process if it detects that M2M Group zone supported by selected preferred ABS is different from previous one.

In addition, this contribution proposes some editorial changes such as correction of table number.

# References

[1] IEEE 802.16p-11/0033, “Draft amendment to IEEE standard for WirelessMAN-Advanced Air Interface for Broadband Wireless Access Systems: Enhancements to support Machine –to-Machine Applications,” October. 2011.

[2]IEEE P802.16.1/D2, “IEEE Draft for WirelessMAN-Advanced Air Interface for Broadband Wireless Access Systems,” October. 2011

# Proposed text

We differentiated any texts modified by this contribution by using a different color as follows.

- Texts added by this contribution: blue underlined texts

- Texts deleted by this contribution: red strikeout texts

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

Add new sub-clause 6.2.18.7.6 as indicated (on page 40, line 37)

6.2.18.7.6 Location update for M2M devices

6.2.18.7.6.1 Location update trigger conditions

An M2M device in idle state may also perform location update when it detects that a M2M Group Zone ID of the selected preferred ABS is changed.

6.2.18.7.6.1.1 M2M Group Zone based update

An M2M device shall perform a Location Update process when an M2M device detects that the selected preferred ABS does not support its currently assigned M2M GROUP ZONE ID. The M2M device shall detect that by monitoring M2M GROUP ZONE ID which is transmitted in the AAI-SCD message. During location update, M2M device’s MGID(s) may be updated.

6.2.3.2 AAI-RNG-RSP

Modify Table 685 28 as indicated

Table 685 28 – AAI-RNG-RSP message field description

| Field | Size(bits) | Value | Condition |
| --- | --- | --- | --- |
| …… |  |  |  |
| New M2M Group Zone ID | TBD | New M2M Group Zone ID of the M2M device | Shall be included when the M2M Group Zone ID is updated. |
| For(i=0;i<Num_MGID;i++){ |  | Number of MGID and FID(Num_MGID) to update in the T-ABS[1..TBD]. Mapping of current MGID and FID and new MGID and FID to be updated. | Present if the mapping between MGID and FID needs to be updated. |
| Current MGID | 15 |  |  |
| Current FID | 4 |  |  |
| New MGID | 15 |  |  |
| New FID | 4 |  |  |
| New MGSS | 64 |  | Shall be included when the MGID is updated. |
| } |  |  |  |
| … |  |  |  |
| Location Update Response | 4 | 0x0 = Success of Location Update 0x1 = Failure of Location Update 0x2 = Reserved 0x3 = Success of Location Update and DL traffic pending 0x4 = Allow AMS’s DCR mode initiation request of DCR mode extension request 0x5 = Reject AMS’s DCR mode initiation request or DCR mode extension request 0x6 ~0xF: Reserved | Shall be included when this message is sent in response to an AAI-RNG-REQ message used to perform location update or DCR mode initiation from Idle Mode or DCR mode extension |
| If(Location Update Response == 0x0){ |  |  |  |
| ……. |  |  |  |
| New Fixed M2M Deregistration ID | 16 | New FMDID that the fixed M2M device shall maintain in idle mode | Only present if the Localized_Idle_Mode_flag is set to 1 in AAI-DREG-REQ/RSP message |
| }// end of If(Location Update Response ==0x0) |  |  |  |
| ……. |  |  |  |

----------------- End of the text proposal ----------------------------------------------------------------------------------------