| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text related to location update trigger conditions for M2M devices |  |
| Date Submitted | 2011-11-9 |  |
| Source(s) | Soojung Jung, Jaesun Cha, Chulsik Yoon, Kwangjae Lim ETRI Eunjong Lee, Youngsoo Yuk, Inuk Jung, Jinsam Kwak LG electronics Anshuman Nigam Samsung Electronics Ming-Hung Tao ITRI | Voice: +82-42-860-5928 E-mail: {sjjung}@etri.re.kr |
| Re: | IEEE 802.16 Working Group Letter Ballot #34 |  |
| Abstract | This contribution proposes a text of location update procedure to be included in the 802.16.1b amendment. |  |
| Purpose | To be discussed and adopted by TGp for the 802.16.1b amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text related to location update trigger conditions for M2M devices

Soojung Jung, Jaesun Cha, Chulsik Yoon, Kwangjae Lim

ETRI

Eunjong Lee, Youngsoo Yuk, Inuk Jung, Jinsam Kwak

LG electronics

Anshuman Nigam

Samsung Electronics

Ming-Hung Tao

ITRI

# Introduction

There are four location update trigger conditions: paging group based update, timer based update, power down update, and MBS update [2].

According to the current AWD [1], M2M Group Zone ID may be broadcasted in the AAI-SCD message. Therefore an M2M device may perform location update process if it detects that M2M Group zone supported by selected preferred ABS is different from previous one.

In addition, this contribution proposes some editorial changes such as correction of table number.

# References

[1] IEEE 802.16p-11/0033, “Draft amendment to IEEE standard for WirelessMAN-Advanced Air Interface for Broadband Wireless Access Systems: Enhancements to support Machine –to-Machine Applications,” October. 2011.

[2]IEEE P802.16.1/D2, “IEEE Draft for WirelessMAN-Advanced Air Interface for Broadband Wireless Access Systems,” October. 2011

# Proposed text

We differentiated any texts modified by this contribution by using a different color as follows.

- Texts added by this contribution: blue underlined texts

- Texts deleted by this contribution: red strikeout texts

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

Add new sub-clause 6.2.18.7.6 as indicated (on page 40, line 37)

6.2.18.7.6 Location update for M2M devices

6.2.18.7.6.1 Location update trigger conditions

An M2M device in idle state may also perform location update when it detects that an M2M GROUP ZONE ID of the selected preferred ABS is changed.

6.2.18.7.6.1.1 M2M Group Zone based update

An M2M device shall perform a Location Update process when an M2M device detects that the selected preferred ABS does not support its currently assigned M2M GROUP ZONE ID. The M2M device shall detect that by monitoring M2M GROUP ZONE ID. During location update, M2M device’s MGID(s) may be updated.

Modify the text on page 4, line 47 as indicated

6.2.1.3.1 M2M Group Identifier (MGID)

A 15-bit value that uniquely identifies an M2M device group in the domain of the network entity that assigns MGID that one or more M2M devices belong to. The domain of the network entity is identified by M2M GROUP ZONE ID. M2M GROUP ZONE ID(s) may be broadcasted in the AAI-SCD message or the PGID-Info message.

Add the section in line 55of page 22 as follows

6.2.3.24 PGID-Info (paging group information) message

Table 50—PGID-Info message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ……. |  |  |  |
| For(i=0; i<N-M2M-GROUP-ZONE; i++) { |  | N-M2M-GROUP-ZONE is the number of M2M GROUP ZONE ID(s) which is assigned to the ABS [1..4]. | Shall be included if one or more M2M GROUP ZONE ID(s) are assigned to the ABS. |
| M2M GROUP ZONE ID | 12 | M2M GROUP ZONE ID which is assigned to the ABS. |  |
| } // End of for (i=0; i<N-M2M-GROUP-ZONE-ID; i++) |  |  |  |

Add the contents in Table 57 of line 33 of page 24 as follows

6.2.3.31 AAI-System Configuration Descriptor (SCD) message

Change the contents of Table 714 57 as indicated

Table 714 57— AAI-SCD message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ……. |  |  |  |
| For(i=0; i<N-M2M-GROUP-ZONE; i++) { |  | N-M2M-GROUP-ZONE is the number of M2M GROUP ZONE ID(s) which is assigned to the ABS [1..4]. | Shall be included if one or more M2M GROUP ZONE ID(s) are assigned to the ABS. |
| M2M GROUP ZONE ID | 12 | M2M GROUP ZONE ID which is assigned to the ABS. |  |
| } // End of for (i=0; i<N-M2M-GROUP-ZONE-ID; i++) |  |  |  |

----------------- End of the text proposal ----------------------------------------------------------------------------------------