| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text related to paging operation of the fixed M2M devices |  |
| Date Submitted | 2011-11-6 |  |
| Source(s) | Soojung Jung, Jaesun Cha, Chulsik Yoon, Kwangjae Lim ETRI | Voice: +82-42-860-5928 E-mail: {sjjung}@etri.re.kr |
| Re: | IEEE 802.16 Working Group Letter Ballot #34 |  |
| Abstract | This contribution proposes a text of paging for fixed M2M devices to be included in the 802.16.1b amendment. |  |
| Purpose | To be discussed and adopted by TGp for the 802.16.1b amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text related to paging operation of the fixed M2M devices

Soojung Jung, Jaesun Cha, Chulsik Yoon, Kwangjae Lim

ETRI

# Introduction

There are 3 types of identifies for M2M devices in idle mode, i.e. DID, MGID and FMDID, which may be included in the AAI-PAG-ADV message. However, there is no description related to paging operation of the fixed M2M devices assigned FMDID.

This contribution proposes some texts related to paging operation of the fixed M2M devices assigned FMDID.

# References

[1] IEEE 802.16p-11/0033, “Draft amendment to IEEE standard for WirelessMAN-Advanced Air Interface for Broadband Wireless Access Systems: Enhancements to support Machine –to-Machine Applications,” October. 2011.

[2]IEEE P802.16.1/D2, “IEEE Draft for WirelessMAN-Advanced Air Interface for Broadband Wireless Access Systems,” October. 2011

# Proposed text

We differentiated any texts modified by this contribution by using a different color as follows.

- Texts added by this contribution: blue underlined texts

- Texts deleted by this contribution: red strikeout texts

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

6.2.18.7.3.1 Paging operation

Add the following texts at the end of 6.2.18.7.3.1

For fixed M2M device, Fixed M2M device de-registration identifier (FMDID) may be included in a paging message instead of DID to identify the fixed M2M device in localized idle mode. The M2M device with FMDID shall monitor a pre-determined frame for paging message. The pre-determined frame N paging frame for the fixed M2M device is implicitly determined as follows:

N paging frame= Fixed M2M de-registaration ID (FMDID) mod m

Where

m=1 or 2 or 3 or 4

m is indicated by an ABS using the PGID-Info message.

----------------- End of the text proposal ----------------------------------------------------------------------------------------