| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text related to group paging for the M2M devices |  |
| Date Submitted | 2011-11-6 |  |
| Source(s) | Soojung Jung, Jaesun Cha, Chulsik Yoon, Kwangjae Lim ETRI | Voice: +82-42-860-5928 E-mail: {sjjung}@etri.re.kr |
| Re: | IEEE 802.16 Working Group Letter Ballot #34 |  |
| Abstract | This contribution proposes a text of group paging to be included in the 802.16.1b amendment. |  |
| Purpose | To be discussed and adopted by TGp for the 802.16.1b amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text related to group paging for the M2M devices

Soojung Jung, Jaesun Cha, Chulsik Yoon, Kwangjae Lim

ETRI

# Introduction

The superframe which a M2M device’s paging listening interval start is determined by the M2M paging cycle and the M2M paging offset field. According to the participants’ consensus during ad-hoc CC, a M2M device should have to monitor only the paging interval/frame that it is supposed to monitor to receive its individual paging messages.

If an ABS belongs to more than one Paging Group (PG) and a M2M Group Zone, the M2M devices of a M2M group within the ABS will monitor different paging listening intervals according to paging offset in spite of same paging cycle.

Fig.1. Example of a M2M group Zone

Fig.2 Example of a paging listening interval

In order to avoid this kind of ABS’s overhead, interaction and coordination between a Paging controller and a Network entity which controls the M2M Group Zone are required.

Therefore, to page all M2M devices of a M2M group within a M2M Group Zone without increasing ABS’s overhead, the same paging cycle and paging offset should be assigned to all member of a M2M group regardless of assigned PG(s). Even though the same paging cycle and paging offset are assigned to all M2M devices of a M2M group, the pre-determined frame for monitoring paging message within the paging listening interval may be different according to the individual identifier for idle mode (i.e., DID).

# References

[1] IEEE 802.16p-11/0033, “Draft amendment to IEEE standard for WirelessMAN-Advanced Air Interface for Broadband Wireless Access Systems: Enhancements to support Machine –to-Machine Applications,” October. 2011.

[2]IEEE P802.16.1/D2, “IEEE Draft for WirelessMAN-Advanced Air Interface for Broadband Wireless Access Systems,” October. 2011

# Proposed text

We differentiated any texts modified by this contribution by using a different color as follows.

- Texts added by this contribution: blue underlined texts

- Texts deleted by this contribution: red strikeout texts

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

Modify the texts on page 35, line 22 as follows

6.2.18.7.1 Paging operation

Group Paging may be used for M2M devices. For this, M2M Group Identifier (MGID) defined in 6.2.1.3.1 may be included in a paging message instead of an individual identifier to identify the group of M2M devices. For group paging message, A M2M device monitors only the paging interval/frame that it is supposed to monitor to receive its individual paging message. And the same paging cycle and paging offset should be assigned to all M2M devices with same MGID within a M2M Group Zone regardless of assigned PG(s).

----------------- End of the text proposal ----------------------------------------------------------------------------------------