| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | M2M device group zone-based location update in 16p |  |
| Date Submitted | 2011-11-07 |  |
| Source(s) | Eunjong Lee, Youngsoo Yuk, Inuk Jung and Jinsam Kwak LG electronics | E-mail: eunjong.lee@lge.com |
| Re: | IEEE 802.16-11/0027, IEEE 802.16 Working Group Letter Ballot #33 |  |
| Abstract | This contribution proposes to define an additional location update condition for an M2M device |  |
| Purpose | To be discussed and adopted by Machine-to-Machine(M2M) Task Group for IEEE P802.16p AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

M2M device group zone-based location update in 16p

Eunjong Lee, Youngsoo Yuk, Inuk Jung and Jinsam Kwak

LG electronics

# Introduction

In the last meeting, we defined that the domain of the network entity that assigns MGID is identified by M2M DEVICE GROUP ZONE ID transmitted in the DCD message. According to the current texts, if an M2M device moves to another M2M device group zone, the current MGID should be updated because the assigned MGID is only available in the M2M device group zone. During connected state, the target BS can reassign the new MGID for the M2M device, based on the MS mobility information after handover. However, for an idle mode M2M device, the change of M2M group zone can be only detected by the MS because the selected preferred BS only knows the location of an MS based on the paging group. Therefore, in order to update the MGID, the M2M device should notify its location to the preferred BS when detecting the zone change. Then, the preferred BS can reassign the new MGID based on the MGID information of the M2M device which is received from the paging controller. So, this contribution proposes to trigger location update when an M2M device detects that the selected preferred BS does not support its currently assigned MGID. The M2M device can detect the change of M2M DEVICE GROUP ZONE by monitoring the M2M DEVICE GROUP ZONE ID in the DCD message which is transmitted by the preferred BS.

In addition, the M2M DEVICE GROUP ZONE ID is not defined in the current DCD message. So, we propose to define the M2M DEVICE GROUP ZONE ID in the DCD message.

# References

[1] IEEE P802.16p/D1, “Enhancements to Support Machine-to-Machine Applications” October 2011.

# Text proposal for inclusion in the 802.16p AWD

[Remedy #1: Add the sections in line 27 of page 12 as follows]

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

6.3.22.8.1 Location update conditions

An MS in idle mode shall perform a location update process operation if any location update condition is met. There are five location update evaluation conditions: paging group update, timer update, power down update, MAC hash skip threshold update and MBS update. In case of an M2M device, there is an additional location update trigger condition: M2M device group zone based update. MS may also perform location update process at will. When an MS performs location update, the MS may include Paging Cycle Change (see 11.5) in RNGREQ to change Paging Cycle. A BS may also change MS’s Paging Cycle by requesting MS to perform location update via MOB_PAG-ADV with Action code = 0b01 (i.e., Perform ranging to establish location). Whether MS has requested or BS has initiated, the BS shall include appropriate Paging Information (see 11.1.8.3) in RNG-RSP, in response to RNG-REQ message including Paging_Cycle_Change TLV sent by MS during Location Update.

6.3.22.8.1.6 M2M device group zone-based update

An M2M device shall perform a Location Update process when an M2M device detects that the selected preferred BS does not support its currently assigned MGID. The M2M device shall detect that by monitoring M2M DEVICE GROUP ZONE ID in the DCD message, which is transmitted by the preferred BS. After performing Location Update, the BS may reassign new MGID to the M2M device through the AAI-RNG-RSP message based on the mobility information of the M2M device.

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------

[Remedy #2: Add the section in line 1 of page 24 as follows]

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

11.4 DCD management message encodings

11.4.1 DCD channel encodings

Table 678.—DCD channel encodings

| Name | Type | Length | Value | PHY scope |
| --- | --- | --- | --- | --- |
| M2M DEVICE GROUP ZONE ID | TBD | variable | This parameter shall include M2M DEVICE GROUP ZONE ID(s) (i.e., nｘM2M DEVICE GROUP ZONE ID) for M2M zone(s) that BS belongs to. A BS may belong to more than one M2M zone(s). An M2M DEVICE GROUP ZONE ID is TBD byte long. | OFDMA |

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------