| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | M2M GROUP ZONE related issues for an idle mode M2M device in 16.1 |  |
| Date Submitted | 2011-11-09 |  |
| Source(s) | Eunjong Lee, Youngsoo Yuk, Inuk Jung and Jinsam Kwak LG electronics | E-mail: eunjong.lee@lge.com |
| Re: | IEEE 802.16-11/0028, IEEE 802.16 Working Group Letter Ballot #34 |  |
| Abstract | This contribution proposes to clarify the M2M GROUP ZONE related issues for an idle mode M2M devices in 16.1 |  |
| Purpose | To be discussed and adopted by Machine-to-Machine(M2M) Task Group for IEEE P802.16.1 AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

M2M GROUP ZONE related issues for an idle mode M2M device in 16.1

Eunjong Lee, Youngsoo Yuk, Inuk Jung and Jinsam Kwak

LG electronics

# Introduction

In the last meeting, we defined that the domain of the network entity that assigns MGID is identified by M2M GROUP ZONE ID and this ID is transmitted in the AAI-SCD message. However, there are still some issues for the idle mode M2M devices. In this contribution, therefore, we’d like to address the remaining issues and propose the methods to solve the problems.

# Discussions

Issue 1. M2M GROUP ZONE ID in the AAI-SCD message

If an M2M device moves to another M2M GROUP ZONE, the MGID should be updated. During connected state, the target ABS can re-assign the new MGID for the M2M device, based on the AMS mobility information after handover. However, during idle state, in order to detect the change of M2M GROUP ZONE, an M2M device should receive the AAI-SCD message whenever moving to another cell. We think it is very inefficient because the idle mode AMS doesn’t know when the ABS will transmit the AAI-SCD message. So, the idle mode M2M device should monitor all the sub-frames until receiving the AAI-SCD message. In addition, the decoding should be performed in all cells that the M2M device is moving. It leads to high power consumption for the idle mode AMSs.

Therefore, in order to reduce power consumption, we propose to transmit the list of BSIDs belonging to the M2M GROUP ZONE in the AAI-SCD message. Based on the list information, the M2M device would try to receive the AAI-SCD message only when it is moving to an ABS not belonging to the list.

Issue 2. MGID reassignment

The MGID should be updated if an M2M device is moving and detects the change of M2M GROUP ZONE ID through the AAI-SCD message. During idle state, the change of M2M GROUP ZONE can be only detected by the M2M device because the selected preferred ABS only knows the location of an AMS based on the paging group. Therefore, the M2M device shall notify its location to the preferred ABS when detecting the zone change for MGID re-assignment.

So, we propose to trigger a location update process when an M2M device detects that the selected preferred ABS does not support its currently assigned MGID. The M2M device can detect the change of M2M GROUP ZONE by monitoring the AAI-SCD message.

# References

[1] IEEE 802.16p-11/0033, “Enhancements to Support Machine-to-Machine Applications,” October 2011.

- Text proposal for inclusion in the 802.16p AWD

[Remedy #1: Add the section in line 55of page 22 as follows]

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

6.2.3.24 PGID-Info (paging group information) message

Table 50—PGID-Info message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| For(i=0; i<N-M2M-GROUP-ZONE; i++) { |  | N-M2M-GROUP-ZONE is the number of M2M GROUP ZONE ID(s) which is assigned to the ABS [1..4]. | Shall be included if an M2M GROUP ZONE ID is assigned to the ABS. |
| M2M GROUP ZONE ID | 12 | M2M GROUP ZONE ID which is assigned to the ABS. |  |
| } // End of for (i=0; i<N-M2M-GROUP-ZONE-ID; i++) |  |  |  |

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------

[Remedy #2: Add the contents in Table714 of line 33 of page 24 as follows]

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

6.2.3.31 AAI-System Configuration Descriptor (SCD) message

Table 714 57— AAI-SCD message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| For(i=0; i<N-M2M-GROUP-ZONE; i++) { |  | N-M2M-GROUP-ZONE is the number of M2M GROUP ZONE ID(s) which is assigned to the ABS [1..4]. | Shall be included if an M2M GROUP ZONE ID is assigned to the ABS. |
| M2M GROUP ZONE ID | 12 | M2M GROUP ZONE ID which is assigned to the ABS. |  |
| } // End of for (i=0; i<N-M2M-GROUP-ZONE-ID; i++) |  |  |  |

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------

[Remedy #3: Modify the texts in line 47 of page 4 as follows]

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

6.2.1.3.1 M2M Group Identifier (MGID)

A 15-bit value that uniquely identifies an M2M device group in the domain of the network entity that assigns MGID that one or more M2M devices belong to. The domain of the network entity is identified by M2M GROUP ZONE ID. M2M GROUP ZONE ID(s) may be broadcasted in the AAI-SCD message or the PGID-Info message.

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------