| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | De-allocation method of long cycle persistent allocation |  |
| Date Submitted | 2011-11-07 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, Inuk Jung, Jinsam Kwak LG Electronics Honggang Li Intel | Email: jeongki.kim@lge.com |
| Re: | IEEE 802.16-11/0028, IEEE 802.16 Working Group Letter Ballot #34 |  |
| Abstract | This contribution proposes the text related to de-allocation method in long cycle persistent scheduling |  |
| Purpose | For discussion in 802.16p TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

De-allocation method in long cycle persistent scheduling

Jeongki Kim, Youngsoo Yuk, Inuk Jung, Jinsam Kwak

LG Electronics

Honggang Li

Intel

# Introduction

At the last meeting, the long cycle persistent scheduling was defined to periodically allocate the resource for the periodic small packet transmission (e.g., smart metering, vending machine, health care, etc.). Most of these periodic traffics will have lower priority than human type traffics (e.g., VoIP, streaming, video call, other real-time traffics, etc.).

In this scenario, if BS wants to use the long cycle persistent allocation for sending other real-time bursts due to limited resource, the BS will de-allocate the persistent resource by sending UL M2M persistent allocation A-MAP IE with allocation period = 0b000 and at the next period the BS will re-allocate the persistent resource by sending UL M2M persistent allocation A-MAP IE with same period as previous allocation period at next period. Figure 1 shows an example of this situation.

Figure 1

When the BS de-allocates the persistently allocated resource, if the BS informs M2M devices that the de-allocation happens one-time or n times, BS doesn’t need to send the UL M2M PA A-MAP IE for re-allocation as shown in figure 2.

Figure 2

# Text proposal

---- Start of proposed text ----

[Remedy #1: Modify the Table 867b as follows:]

6.3.5.5.2.4.16 UL M2M Persistent allocation A-MAP IE

The UL M2M persistent allocation A-MAP IE is specified in Table 867b.

Table 867b—UL M2M Persistent Allocation A-MAP IE

| Syntax | Size (bits) | Value/Description |
| --- | --- | --- |
| UL M2M Persistent Allocation A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | UL M2M Persistent Allocation A-MAP_IE |
| Allocation Period | 4 | Period of persistent allocation for M2M: Bit 0~2: 0b000: deallocation 0b001: 2 frames 0b010: 4 frames 0b011: 6 frames 0b100: 5 superframes 0b101: 10 superframes 0b110: 25 superframes 0b111: 50 superframes Bit 3: reserved |
| if (Allocation Period==0b000){ | - | - |
| Resource Index | 11 | Confirmation of the resource index for a previously assigned persistent resource that has been deallocated 512 FFT size: 0 in first 2 MSB bits + 9 bits for resource index 1024 FFT size: 11 bits for resource index 2048 FFT size: 11 bits for resource index Resource index includes location and allocation size |
| TTI and Relevance | 2 | Indicates the TTI type and the location of UL subframe relevant to this A-MAP. 0b00: long TTI 0b01: default TTI, the first UL subframe relevant to this A-MAP 0b10: default TTI, the second UL subframe relevant to this A-MAP 0b11: default TTI, the third UL subframe relevant to this A-MAP |
| HFA | 6 | Explicit Index for HARQ Feedback Allocation to acknowledge receipt of deallocation A-MAP IE |
| De-allocation type | 1 | 0b0: Permanent de-allocation. The resource indicated by resource index is de-allocated at UL subframe referenced by this A-MAP IE and the related persistent allocation is terminated. 0b1: Temporal de-allocation Indicate that the de-allocation happens temporally. |
| If (De-allocation type = 0b1) { |  |  |
| Num_de-allocation | 2 | The number that the consecutive de-allocations happen 0b00: 1 0b01: 2 0b10: 3 0b11: 4 |
| } |  |  |
| Reserved | 13 | Reserved bits |
| } else if (Allocation Period != 0b000){ | - | - |
| IsizeOffset | 5 | Offset used to compute burst size index |
| Resource Index | 11 | Confirmation of the resource index for a previously assigned persistent resource that has been deallocated 512 FFT size: 0 in first 2 MSB bits + 9 bits for resource index 1024 FFT size: 11 bits for resource index 2048 FFT size: 11 bits for resource index Resource index includes location and allocation size |
| TTI and Relevance | 2 | Indicates the TTI type and the location of UL subframe relevant to this A-MAP. 0b00: long TTI 0b01: default TTI, the first UL subframe relevant to this A-MAP 0b10: default TTI, the second UL subframe relevant to this A-MAP 0b11: default TTI, the third UL subframe relevant to this A-MAP |
| HFA | 3 | HARQ Feedback Allocation |
| N_ACID | 2 | Number of ACIDs for implicit cycling of HARQ channel identifier. 0b00: 1 0b01: 2 0b10: 3 0b11: 4 |
| Initial_ACID | 4 | Initial value of HARQ channel identifier for implicit cycling of HARQ channel identifiers. |
| Reserved | 5 | Reserved bits |
| } |  |  |
| } |  |  |

[Remedy #: Add the subclause 6.2.7.2 in P802.16p-11/0035, on page 33, line32 :]

6.2.7.2 De-allocation mechanism

Add the following text at the end of 6.2.7.2

In case of de-allocation of long cycle persistent scheduling by UL M2M persistent allocation A-MAP IE, if de-allocation type is set to 0b0 (i.e., Permanent de-allocation), the persistent resource assigned by the UL M2M persistent allocation A-MAP IE shall be de-allocated in the reference UL subframe and the ABS and M2M device shall terminate the persistent allocation. Otherwise (i.e., Temporal de-allocation), the persistent resource assigned by the UL M2M persistent allocation A-MAP IE shall consecutively be de-allocated at N times (as indicated by UL M2M PA A-MAP IE) and the ABS and M2M device shall use the previously assigned persistent allocation from the same subframe of the next period after the last de-allocation.

---- End of proposed text ----