| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarifications on Paging Operation in M2M |  |
| Date Submitted | 2011-11-04 |  |
| Source(s) | Anshuman Nigam, Anil Agiwal, Hyunjeong Kang, Rakesh Taori Samsung Electronics | E-mail: anshun@samsung.com, hyunjeong.kang@samsung.com |
| Re: | IEEE 802.16-11/0027, IEEE 802.16 Working Group Letter Ballot #33 |  |
| Abstract | This contribution proposes clarifications on paging operation in M2M |  |
| Purpose | To be adopted in the updated version of IEEE P802.16p/D1 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarifications on Paging Operation in M2M

Anshuman Nigam, Anil Agiwal, Hyunjeong Kang, Rakesh Taori

Samsung Electronics

# Introduction

The following issues need to be clarified in [1].

- Monitoring of both individual and group paging

- Paging frame determination

# Discussion

Issue: Monitoring of both individual and group paging

An M2M device does not know whether both individual paging and multicast paging comes up in the same paging listening interval. But the individual paging shall be read as default and with priority than multicast paging. If the multicast paging presents ahead of the individual paging, then the M2M device shall read both the paging. Otherwise, the M2M device does not have to wait for reading multicast paging once it read the individual paging indicator because if there is multicast data for the individually paged M2M device, then the M2M device will receive multicast data after reentering the network or during network reentry by the individual paging.

So, the clarified operation is that individually paged M2M device shall not follow the multicast operation in idle state.

Furthermore, it should be clearly mentioned that there is no separate paging cycle for group paging.

# Proposal

Section 6.3.22.5 Page23 Line 12 of IEEE P802.16p/D1

Add the following text at the end of subclause 6.3.22.6

Group paging may be used for M2M devices. For this, M2M Group Identifier (MGID) defined in 6.3.1 may be included in a paging message instead of an individual identifier to identify the group of M2M devices. The M2M device follows the paging cycle as per section 6.3.22.5

# References

[1] 80216p/D1: Enhancements to Support M2M Application for WirelessMAN System