| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Issues arising due to overlapping M2M zones |  |
| Date Submitted | 2011-11-04 |  |
| Source(s) | Anshuman Nigam, Anil Agiwal, Hyunjeong Kang, Rakesh Taori Samsung Electronics | E-mail: anshun@samsung.com, hyunjeong.kang@samsung.com |
| Re: | IEEE 802.16-11/0027, IEEE 802.16 Working Group Letter Ballot #33 |  |
| Abstract | This contribution proposes solutions for issues arising due to overlapping M2M zones |  |
| Purpose | To be adopted in the updated version of IEEE P802.16p/D1 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Issues arising due to overlapping M2M zones

Anshuman Nigam, Anil Agiwal, Hyunjeong Kang, Rakesh Taori

Samsung Electronics

# Introduction

The following issues arise due to overlapping M2M zones

- Collision of MGIDs between different M2M devices communicating with a BS belonging to multiple M2M group zones.

  - Uniquely addressing an M2M device in MAP

  - Uniquely addressing an M2M device in paging message

# Discussion

In order to avoid frequent update of MGID at the boundary of M2M device group zones because of M2M device mobility (Ping Pong), the BSs at the border of M2M device group zones are being considered to be part of multiple M2M device group zones. This resolves the frequent updates of MGID at the boundary of M2M device group zones. However, it leads to collisions of MGID between multiple M2M devices communicating with a BS but belonging to different M2M device group zones. As a result signaling message transmitted using MGID meant for M2M devices of one group will also reach another group which has the same MGID but belongs to different M2M device group zone.

For e.g. M2M device 1 is assigned MGID 1 in M2M Device group zone 1 and is communicating with BS1. M2M device 2 is assigned MGID 1 in M2M Device group zone 2 and is communicating with BS2. Consider that an M2M device 1 moves from BS1 to BS2 wherein BS1 is part of M2M device group zone 1 and BS2 is part of M2M device group zone 1 and M2M device group zone 2. When the M2M Device 1 moves to BS2, BS2 has to communicate with two M2M devices having the same MGID. MGID is not updated for M2M device 1 as BS2 is also part of M2M group zone 1 to which M2M device 1 belongs.

In order to uniquely identify an M2M device group, the MGID can be coupled with M2M_GROUP_ZONE_ID it belongs to. But since the M2M_GROUP_ZONE_ID can be a large in size, this can lead to significant overhead. Furthermore, an M2M device should be able to identify itself as early as possible in order to have greater power savings.

The ABS may broadcast the M2M_GROUP_ZONE_IDs it belongs to (already in section 6.3.1). In order to reduce the overhead an M2M device can derive a Zone Index corresponding to the M2M_GROUP_ZONE_ID according to implicit ordering of the M2M_GROUP_ZONE_IDs in the broadcast message. The maximum number of M2M_GROUP_ZONEs that an ABS can belong to is broadcasted or specified. The ABS can include Zone Index corresponding to the M2M_GROUP_ZONE along with the MGID to uniquely identify an M2M device group.

Idle mode operation:-

The MOB_PAG-ADV message includes the Zone Index along with the MGIDs to uniquely page an M2M device group.

Connected mode operation:-

A new MAP IE is defined and the new MAP IE contains the MGID and the Zone Index of the included MGID.

MGID Assignment/Update:-

Use MGID along with Zone ID. In order to avoid delay in reading broadcast message, it is suggested to use Zone ID instead of Zone Index.

# Proposal

Change 1

Section 6.3.1, page 4, line 17

A 15-bit value that uniquely identifies an M2M device group in the domain of the network entity that assigns MGID that one or more M2M devices belong to. The domain of the network entity is identified by M2M GROUP ZONE ID. M2M GROUP ZONE ID may be broadcasted in the AAI-SCD message. An M2M device derives the Zone Index corresponding to an M2M GROUP ZONE ID based on the implicit ordering of the M2M GROUP ZONE IDs in the broadcasted message where the Zone Index of the first included M2M GROUP ZONE_ID is assigned to be 0 and increases sequentially till (MAX_M2M_Group_Zone – 1) where MAX_M2M_Group_Zone is defined to be 4. Hence when the ABS is part of only one M2M Group Zone, it broadcasts only one M2M GROUP ZONE_ID and the M2M device derives the corresponding Zone Index as 0x00. An M2M device group is addressed using the MGID and the corresponding Zone Index.

Change 2

Section 6.3.22.5 Page23 Line 12 of IEEE P802.16p/D1

Add the following text at the end of subclause 6.3.22.6

Group paging may be used for M2M devices. For this, M2M Group Identifier (MGID) defined in 6.2.1.3.1along with the Zone Index of the associated M2M group zone may be included in a paging message instead of an individual identifier to identify the group of M2M devices. The M2M device follows the paging cycle as per section 6.3.22.5

Change 3

Section 11.17.5 page 29 line 44

| Name | Type | Length | Value |
| --- | --- | --- | --- |
| MGID. | 156.1. | 2. | Bits 0-14: Indicates M2M device group ID for which the multicast traffic is scheduled Bit 15: Padding, Will be set to 0 |
| Zone Index | 156.2 | 2 |  |
| Action Code. | 156.23. | 1. | Bits 0-1: Indicates Action code for the M2M device group ID 0b00 - Performing network reentry 0b01- Performing location update 0b10 - Receiving multicast traffic without requiring network reentry 0b11- Re-assignment of MGID Bits 2-7: Padding, Will be set to 0. |

Change 4

Section 6.3.2.3.6 page 6 line 35

MGID Update

Bits 0-15: current MGID value

Bits 16-31: new MGID value

Bits       : M2M GROUP ZONE ID of new MGID (Present if ABS is part of more than one M2M group zone)

Change 5

Section 6.3.2.3.10 page 8 line 5

MGID (see 11.13.46)

MGID associated with the SF

M2M_GROUP_ZONE_ID of the MGID (Present if ABS is part of more than one M2M group zone)

Change 6

Section 6.3.2.3.13 page 8 line 45

MGID (see 11.13.46)

new MGID, replacing a previously associated MGID.

M2M_GROUP_ZONE_ID of the new MGID (Present if ABS is part of more than one M2M group zone)

Change 7

Section 6.3.2.3.99 page 10 line 45, 55

| Syntax | Size | Notes |
| --- | --- | --- |
| MGID. | 15 |  |
| Zone Index |  | Zone Index of the MGID |
| Action Code | 2 | Use to indentify the purpose if this message 0b00: re-assignment of MGID value 0b01-0b11: Reserved. |
| If(Action Cde == 0x00) { |  |  |
| New MGID | 15 | New MGID value to be assigned. |
| M2M_GROUP_ZONE_ID |  | Zone ID of new MGID (Present if ABS is part of more than one M2M group zone) |
| } |  |  |

Change 8

Table 229c Section 6.3.2.3.100 page 11 line 21

| Syntax | Size | Notes |
| --- | --- | --- |
| MAMC_Message_Format() {. |  |  |
| Management Message Type = 112. | 8 |  |
| MGID. | 15 |  |
| Zone Index |  | Zone Index of the MGID |
| Action Code | 2 | Action Value code to acknowledge.. |

Change 9

Table 688 Section 11.6 page 25 line 60

| Name | Type | Length | Value | PHY Scope |
| --- | --- | --- | --- | --- |
| MGID Update. | 49 | 4 | Bits 0-15: current MGID value Bits 16-31: new MGID value Bits :M2M_GROUP_ZONE_ID of new MGID (Present if ABS is part of more than one M2M group zone). |  |

Change 10

Section 11.17.5 page 30 line 21

| Name | Type | Length | Value | PHY Scope |
| --- | --- | --- | --- | --- |
| MGID Re-assignment for M2M.. | 156.4 | 2 | New MGID (15 bits) M2M_GROUP_ZONE_ID (Present if ABS is part of more than one M2M group zone). |  |

Change 11

Section 11.13.46 page 27 line 64

This TLVe value of this field specifies MGID that is used for the associated flow. During connected mode, the MGID may be added by DSA-REQ message and may be changed by DSC-REQ message.

| Name | Type | Length | Value | Scope |
| --- | --- | --- | --- | --- |
| MGID.. | [145/146].57 | 2 | Bits 0-14: Indicates MGID; Bit 15: Padding, Will be set to 0.. | DSA-REQ DSC-REQ. |
| M2M_GROUP_ZONE_ID |  |  | Zone ID of included MGID (Present if ABS is part of more than one M2M group zone) |  |

Change 12

Add new subclause 8.4.5.3.xx as indicated

8.4.5.3.35 M2M Multicast Assignment IE format

In the DL-MAP, a BS shall transmit the M2M Multicast Assignment MAP IE to indicate the DL allocation in which M2M Multicast data is sent.

Table xxx—M2M Multicast Assignment IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| M2M Multicast Assignment IE(){ | - | - |
| Extended-2 DIUC | 4 | M2M Multicast Assignment IE () = 0xF (Extended-3 DIUC) |
| Length | 8 | Length in bytes |
| Extended-3 DIUC | 4 | 0x04 |
| Num_MGID | 6 | Number of included MGIDs |
| For(i=0;i<Num_MGID;i++) { |  |  |
| MGID | 15 |  |
| Zone Index | 2 | Zone Index of the included MGID |
| } |  |  |
| DIUC | 4 |  |
| OFDMA Symbol Offset | 8 | The offset of the OFDMA symbol measured in OFDMA symbols from beginning of the DL frame in which the DL-MAP is transmitted. Counting from the frame preamble and starting from 0 |
| Subchannel offset | 7 | The offset of the first OFDMA symbol of the MBS region measured in OFDMA symbols from beginning of this DL frame |
| No. Subchannels | 7 |  |
| NO. OFDMA symbols | 7 |  |
| Boosting | 3 | Refer to Table 321 |
| Repetition Coding Indication | 2 | 0b00—No repetition coding 0b01—Repetition coding of 2 used 0b10—Repetition coding of 4 used 0b11—Repetition coding of 6 used |
| if !(byte boundary) { |  |  |
| Padding Nibble | variable | Padding to reach byte boundary |
| } |  |  |
| } | - | - |

# References

[1] P802.16p/D1: WirelessMAN - Air Interface for Broadband Wireless Access Systems