| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of BS initiated localized idle mode operation |  |
| Date Submitted | Nov 06, 2011 |  |
| Source(s) | Giwon Park, Youngsoo Yuk, Inuk Jung, JinSam Kwak LG Electronics | giwon.park.@lge.com |
| Re: | IEEE 802.16-11/0027, IEEE 802.16 Working Group Letter Ballot #33 |  |
| Abstract | This is to clarify the BS initiated localized idle mode. |  |
| Purpose | To be discussed and adopted by 16p TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of BS initiated localized idle mode operation

Giwon Park, Youngsoo Yuk, Inuk Jung, JinSam Kwak

LG Electronics

# Introduction

Although current draft [1] defines MS initiated localized idle mode operation, it does not describe the BS initiated localized idle mode operation. Thus, we proposed the BS initiated localized idle mode.

# Proposed Text Change

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

[Remedy 1. Add the text in section 6.3.22.11.2 of page 14, line 21 as follows:]

6.3.22.11.2 Idle mode optimizations for fixed M2M devices

Using BS-initiated localized idle mode entry, an BS may signal for a fixed M2M device to begin localized idle mode by sending an DREG-CMD message in unsolicited manner. This unsolicited DREG-CMD message may include Localized_Idle_Mode flag set to 1. When a fixed M2M device receives an unsolicited DREG-CMD message with Localized_Idle_Mode flag set to 1, the fixed M2M device shall immediately start the idle mode initiation procedures by sending DREG-REQ message with Localized_Idle_Mode flag set to 1 or 0 in response to the unsolicited DREG-CMD message.

[Remedy 2. Add the section 11.7.13 and 11.7.13.3 in page 26, line 11 as follows:]

11.7.13 Mobility parameters support

11.7.13.1 Mobility features supported

The Mobility Features Supported field indicates whether the MS supports mobility HO, sleep mode, idle mode and Localized Idle Mode. A bit value of 0 indicates “not supported” while 1 indicates “supported.”

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 31 | 1 | Bit 0: Mobility (HO) support Bit 1: Sleep mode support Bit 2: Idle mode support Bit 3: Localized Idle Mode support | REG-REQ, REG-RSP |

11.7.13.3 Mobility type

The Mobility type field indicates mobility type of device. A bit value of 0 indicates “fixed device” while 1 indicates “mobility enabled device.”

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| TBD | 1 | Bit 0: Mobility type 0b0: Fixed device 0b1: Mobility enabled device Bits 1~7: Reserved | REG-REQ |

# References

[1] P802.16p/D1