| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Ranging resource allocation for idle mode fixed M2M device in WirelessMAN IMT-Advanced system |  |
| Date Submitted | Nov 06, 2011 |  |
| Source(s) | Giwon Park, Youngsoo Yuk, Inuk Jung, JinSam Kwak LG Electronics | giwon.park.@lge.com |
| Re: | IEEE 802.16-11/0028, IEEE 802.16 Working Group Letter Ballot #34 |  |
| Abstract | This is to propose the ranging resource allocation for idle mode fixed M2M device in WirelessMAN IMT-Advanced system. |  |
| Purpose | To be discussed and adopted by 16p TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Ranging resource allocation for idle mode fixed M2M device in WirelessMAN IMT-Advanced system

Giwon Park, Youngsoo Yuk, Inuk Jung, JinSam Kwak

LG Electronics

# Introduction

Current draft [1] describes dedicated channel can be allocated to M2M devices when the ABS transmits the AAI-PAG-ADV message. However, there is no description how long will this allocated channel valid. Thus, we clarify it.

# Proposed Text Change

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

[Remedy 1. Add the new text in 6.2.18.7.2 (page 36, line 42) as follows:]

6.2.18.7.2 Network re-entry from idle mode for M2M devices

If the dedicated ranging channel (i.e., M2M ranging opportunity subframe index, Periodicity of the M2M ranging) which is allocated through AAI-PAG-ADV message is different with the dedicated ranging channel which is allocated via AAI-SCD message, Dedicated Channel Allocation Timer is included in AAI-PAG-ADV message and the allocated dedicated ranging channel is valid within the Dedicated Channel Allocation Timer.

If the M2M device receives the individual paging message (i.e., AAI-PAG-ADV with DID or FMDID) and the M2M network access type is set to 0b11, M2M device performs the normal ranging using the ranging resources defined in Table 839 in 6.3.5.5.1.2.

[Remedy 2. Modify the table 706 in page 20, line 25 as follows:]

Table 706 – AAI-PAG-ADV message field description

| Field | Size | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| for (i=0; j<Num_MGID; i++ { |  | Num_MGID indicates the number of MGIDs included in this paging message [0..63] | Shall be included if the ABS sends DL multicast data for M2M after transmission of the AAI-PAG-ADV message. |
| … |  |  |  |
| if (M2M network access type == 0b01 ¦ 0b10) { |  |  |  |
| M2M ranging opportunity subframe index | 3 | Indicates the subframe index of the allocated ranging opportunity dedicated for M2M devices. | Shall be included if the ABS allocates dedicated ranging channels in addition to the ranging channels allocated by the SCD message to the M2M devices paged through the AAI-PAG-ADV message. |
| Periodicity of the M2M ranging | 3 | Indicates the periodicity of the ranging dedicated for M2M devices. 0b000: transmission in every frame 0b001: transmission in the first frame in every superframe 0b010: transmission in the first frame in every even numbered superframe, i.e., mod(superframe number, 2) = 0 0b011: transmission in the first frame in every 4th superframe, i.e., mod(superframe number, 4) = 0 [0b100~0b111: Reserved] | Shall be included if the ABS allocates dedicated ranging channels in addition to the ranging channels allocated by the SCD message to the M2M devices paged through the AAI-PAG-ADV message. |
| Dedicated Channel Allocation Timer | TBD | Time duration in which dedicated channel allocation (i.e., M2M ranging opportunity subframe index, Periodicity of the M2M ranging) is valid. | Shall be included if the ABS allocates dedicated ranging channels in addition to the ranging channels allocated by the SCD to the M2M devices paged through the AAI-PAG-ADV message. |
| } |  |  |  |
| … |  |  |  |
| } // End of for (k=0; k<Num_FMDID; k++) { |  |  |  |
| M2M ranging opportunity subframe index | 3 | Indicates the subframe index of the allocated ranging opportunity dedicated for M2M devices. | Optional, This parameter shall be present if the M2M network access type of individually paged M2M devices is set to 0b01. |
| Periodicity of the M2M ranging | 3 | Indicates the periodicity of the ranging dedicated for M2M devices. 0b000: transmission in every frame 0b001: transmission in the first frame in every superframe 0b010: transmission in the first frame in every even numbered superframe, i.e., mod(superframe number, 2) = 0 0b011: transmission in the first frame in every 4th superframe, i.e., mod(superframe number, 4) = 0 [0b100~0b111: Reserved] | Optional, This parameter shall be present if the M2M network access type of individually paged M2M devices is set to 0b01. |
| Dedicated Channel Allocation Timer | TBD | Time duration in which dedicated channel allocation (i.e., M2M ranging opportunity subframe index, Periodicity of the M2M ranging) is valid. | Shall be included if the ABS allocates dedicated ranging channels in addition to the ranging channels allocated by the SCD to the M2M devices paged through the AAI-PAG-ADV message and if the M2M network access type is set to 0b01 (i.e., dedicated ranging channel allocation in AAI-PAG-ADV). |
| … |  |  |  |

# References

[1] P802.16p/11-0033