| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for network access entry for a large number of M2M devices |  |
| Date Submitted | 2011-11-09 |  |
| Source(s) | Lei Zhou, Hai Wang, Xufeng Zheng China Samsung Telecom R&D Center Hyunjeong Kang, Chiwoo Lim,Kaushik Josiam, Jaeweon Cho, Rakesh Taori Samsung Electronics Co., Ltd. Jinsoo Choi, Inuk Jung, Jin Sam Kwak LGE Ming-Hung Tao, Ying-Chuan Hsiao ITRI Erik Colban Huawei | l.zhou@samsung.com +86-10-59253333 |
| Re: | Call for contributions for 802.16 WG Letter Ballot (#33)(IEEE 802.16p) |  |
| Abstract | This contribution proposes to network access entry for a large number of M2M devices |  |
| Purpose | To be discussed and adopted by 802.16p for 802.16p amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for network access entry for a large number of M2M devices

Lei Zhou, Hai Wang, Xufeng Zheng

China Samsung Telecom R&D Center

Hyunjeong Kang, Chiwoo Lim,Kaushik Josiam, Jaeweon Cho, Rakesh Taori

Samsung Electronics Co., Ltd.

Jinsoo Choi, Inuk Jung, Jin Sam Kwak

LGE

Ming-Hung Tao, Ying-Chuan Hsiao

ITRI

Erik Colban

Huawei

- Introduction

Last meeting, network reentry procedure of M2M group based on group delegate in 802.16m was accepted. So this contribution proposes to use network reentry procedure of M2M group based on group delegate in 802.16e.

- The Proposed Text in AWD

------------------------------- Text Proposal Start ---------------------------------------------------

------------------------------- Text Proposal 1 Start ---------------------------------------------------

[Remedy1: Add proposed text from line# 20 Page 14 in IEEE 802.16p/D1 with the followings:]

6.3.22.11.3 Network reentry for M2M group

In order to reduce network congestion produced by a large number of M2M devices, network reentry may be initiated for a group of M2M devices as follows. An M2M device in an M2M group is called as group member (GM). An M2M device in a group is authorized to act as representative for this M2M group and is expected to initiate the first ranging access for this M2M group. Such M2M device that initiates ranging for an M2M group is called a group delegate (GD). The BS assigns a dedicated ranging code set?? to the GD. When an M2M group is expected to report its data, group delegates of the group send a ranging code based on the M2M group ID (MGID) from the ranging code set to the BS. On receiving this ranging code, the BS sends a RNG-RSP message in response, which includes one of three ranging statuses (success, abort and continue) for group. All group members listen for confirmation in the RNG-RSP message for the dedicated ranging code transmitted by the group GD. After all group members get “success” information, they will use common or dedicated ranging resource for network access.

Upon a detection of an event specific for a group the GMs of this group shall start the T32 timer defined in TBD. Within the T32 timer the GMs of this group await the RNG-RSP message, which is expected to be a response to the ranging code sent by this group’s GD. If the GMs receive the RNG-RSP, which is in response to the ranging code transmitted by the GD, every GM initiates network re-entry. Otherwise, if the GMs do not receive such RNG-RSP message within the duration of T32, the GMs perform voluntary network re-entry upon expiry of T32. Selection of the group delegate is TBD. Ranging resource allocation to be used by the GD is TBD.

If ranging status is “success”, all GMs in this group start its network reentry procedure by transmitting a ranging code.

If ranging status is “abort”, all GM in this group shall start the ranging abort timer and abort the ranging process until the ranging abort timer expires. After abort timer expires, this group shall restart the ranging procedure based on GD.

  - If ranging status is “continue”, the GD in this group shall adjust its parameters accordingly and continue the ranging process.

------------------------------- Text Proposal1 End ---------------------------------------------------

------------------------------- Text Proposal End ---------------------------------------------------