| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for network access entry for a large number of M2M devices |  |
| Date Submitted | 2011-11-09 |  |
| Source(s) | Lei Zhou, Hai Wang, Xufeng Zheng China Samsung Telecom R&D Center Hyunjeong Kang, Chiwoo Lim,Kaushik Josiam, Jaeweon Cho, Rakesh Taori Samsung Electronics Co., Ltd. Honggang Li Intel Jinsoo Choi, Inuk Jung, Jin Sam Kwak LGE Ming-Hung Tao, Ying-Chuan Hsiao ITRI Erik Colban Huawei Inbar Anson Bratspiess Sequans | l.zhou@samsung.com +86-10-59253333 |
| Re: | Call for contributions for 802.16 WG Letter Ballot (#33)(IEEE 802.16p) |  |
| Abstract | This contribution proposes to network access entry for a large number of M2M devices |  |
| Purpose | To be discussed and adopted by 802.16p for 802.16p amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for network access entry for a large number of M2M devices

Lei Zhou, Hai Wang, Xufeng Zheng

China Samsung Telecom R&D Center

Hyunjeong Kang, Chiwoo Lim,Kaushik Josiam, Jaeweon Cho, Rakesh Taori

Samsung Electronics Co., Ltd.

Hongang Li

Intel

Jinsoo Choi, Inuk Jung, Jin Sam Kwak

LGE

Ming-Hung Tao, Ying-Chuan Hsiao

ITRI

Erik Colban

Huawei

Inbar Anson Bratspiess

Sequans

- Introduction

Last meeting, network reentry procedure of M2M group based on group delegate in 802.16m was accepted. So this contribution proposes to use network reentry procedure of M2M group based on group delegate in 802.16e.

- The Proposed Text in AWD

------------------------------- Text Proposal Start ---------------------------------------------------

------------------------------- Text Proposal 1 Start ---------------------------------------------------

[Remedy1: Add proposed text from line# 20 Page 14 in IEEE 802.16p/D1 with the followings:]

6.3.22.11.3 Network reentry for M2M group

In order to reduce network congestion produced by a large number of M2M devices initiating network reentry simultaneously, network reentry of a group may be controlled as follows.

An M2M device in an M2M group is called as Group Member (GM). An M2M device from the group is authorized to act as a representative for this M2M group and is expected to initiate the first ranging access for this M2M group. This M2M device is called a Group Delegate (GD). The BS assigns a dedicated ranging code from a set of CDMA ranging codes to each group.

When an M2M group is expected to transmit UL data, whether periodically or following an event, the group delegates perform ranging using the CDMA ranging code assigned to the group, and all GM’s start timer T32. On receiving this ranging code, the BS shall respond with a RNG-RSP message or with an anonymous UL grant. The RNG-RSP includes one of three ranging statuses (success, continue or abort).

All GM’s listen for a response from the BS identified by the group’s ranging code. If the BS transmits an anonymous UL grant, or a RNG-RSP message with the status set to “continue” or “successful”, the GM’s shall stop timer T32, and may perform ranging and network reentry. If the BS transmits a RNG-RSP message with the status set to “abort”, all GM’s shall stop the T32 timer, abort the ranging process and shall start the ranging abort timer. When the ranging abort timer expires, this group shall restart the ranging procedure based on GD.

If the GM’s do not receive the BS’s response within the duration of T32, the GMs may perform individual network re-entry upon the expiry of T32.

Selection of the group delegate is beyond the scope of this standard.

Ranging resource allocation to be used by the GD is TBD.

------------------------------- Text Proposal1 End ---------------------------------------------------

------------------------------- Text Proposal End ---------------------------------------------------