| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for network access entry for a large number of M2M devices in 16m |  |
| Date Submitted | 2011-11-04 |  |
| Source(s) | Lei Zhou, Hai Wang, Xufeng Zheng China Samsung Telecom R&D Center Hyunjeong Kang, Chiwoo Lim,Kaushik Josiam, Jaeweon Cho, Rakesh Taori Samsung Electronics Co., Ltd. | l.zhou@samsung.com +86-10-59253333 |
| Re: | Call for contributions for 802.16 WG Letter Ballot (#34)(IEEE 802.16p) |  |
| Abstract | This contribution proposes to network access entry for a large number of M2M devices |  |
| Purpose | To be discussed and adopted by 802.16p for 802.16p amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for network access entry for a large number of M2M devices in 16m

Lei Zhou, Hai Wang, Xufeng Zheng

China Samsung Telecom R&D Center

Hyunjeong Kang, Chiwoo Lim,Kaushik Josiam, Jaeweon Cho, Rakesh Taori

Samsung Electronics Co., Ltd.

- Introduction

- This contribution proposes to modify AAI-RNG-ACK message to support network reentry for M2M group based on group delegate and support group delegate for both MS initialization and Network initialization.

- The Proposed Text in AWD

------------------------------- Text Proposal Start ---------------------------------------------------

------------------------------- Text Proposal 1 Start ---------------------------------------------------

[Remedy1: Add proposed text from line# 28 Page 38 in IEEE 802.16p-11/0033 with the followings:]

6.2.18.7.4 Network reentry from idle mode for M2M group

In order to reduce network congestion produced by a large number of M2M devices, network reentry may be initiated for a group of M2M devices. M2M device in a M2M group is called as group member (GM). An M2M device in a group is authorized to act as representative for this M2M group and is expected to initiate the first ranging access for this M2M group. Such M2M device that initiates ranging for an M2M group is called as a group delegate (GD). BS assigns a dedicated ranging code to a GD. When a M2M group is expected to report their data or is paged by network, group delegates of the group send a ranging code based on M2M group ID (MGID) from a ranging code set to BS. On receiving this ranging code, the BS sends AAI-RNG-ACK in response, which includes three ranging status (success, abort and continue) for group. All group members listen to confirmation for the dedicated ranging code transmitted by the group GD in AAI-RNG-ACK message. After all group members get “success” information, they will use common or dedicated ranging resource for network access.

------------------------------- Text Proposal 1 End ---------------------------------------------------

------------------------------- Text Proposal 2 Start ---------------------------------------------------

Remedy2: Add proposed text from line# 59 Page 9 in IEEE 802.16p-11/0033 with the followings:]

6.2.3.3 AAI-RNG-ACK

Modify Table 29 as indicated

Table 29—AAI-RNG-ACK message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … | … | … | … |
| For (j=0; j<N_Received_Codes for M2M group; j++) { |  | The number of ranging preamble code indices(i.e. N_Received_Codes for M2M groups)[1..32] detected in this corresponding ranging opportunity. |  |
| Ranging Status | 2 | Indicate whether ranging preamble code or UL burst is received within acceptable limits by ABS. 0b00 = success for M2M Group 0b01 = abort for M2M Group 0b10 = continue |  |
| If (Ranging Status == 0b01) { |  |  |  |
| Ranging Abort Timer | 16 | Timer defined by an ABS to prohibit the AMS from attempting network entry at this ABS, for a specific time duration. Value: 0 (Do not try ranging again at the ABS.) Value: 1-65535, In units of seconds |  |
| }else if (Ranging Status ==0b10) { |  |  |  |
| Adjustment parameters indication (API) | 3 | Bit#0: Time offset adjustment indication. Bit#1: Power level adjustment indication Bit#2: Frequency offset adjustment Indication |  |
| If( API Bit#0==0b1 ) { |  |  |  |
| Timing offset adjustment | 15 |  |  |
| } |  |  |  |
| If( API Bit#1==0b1 ) { |  |  |  |
| Power level adjustment | 4 |  |  |
| } |  |  |  |
| If (API Bit#2==0b1) { |  |  |  |
| Frequency offset adjustment | 9 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  | ... |

------------------------------- Text Proposal 2 End ---------------------------------------------------

------------------------------- Text Proposal Start ---------------------------------------------------