| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | M2M group delegate assignment |  |
| Date Submitted | 2011-11-08 |  |
| Source(s) | Rui Huang, Honggang Li, Shantidev Mohanty Intel Corporation Lei Zhou, Hai Wang, Xufeng Zheng China Samsung Telecom R&D Center | E-mail: rui.huang@intel.com l.zhou@samsung.com |
| Re: | IEEE 802.16-11/0028 for IEEE 802.16 Letter Ballot #34 on P802.16p/11-0033 |  |
| Abstract | This contribution proposes the assignment of M2M group delegate in Idle mode. |  |
| Purpose | For review and adoption into P802.16p/11-0033. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

M2M group delegate assignment

Rui Huang, Honggang Li, Shantidev Mohanty

Intel Corporation

Lei Zhou

China Samsung Telecom R&D Center

# Introduction

In [1], a M2M group delegate will be used to reduce network random access congestion due to a large number of M2M devices in a M2M group. However, some of key issues for this mechanism were not investigated yet, e.g. M2M group topology including the delegate, the assignment of M2M delegate. Therefore, in this contribution, we will and present some consideration on these open issues.

# M2M group topologies

There are two typical M2M group topologies when M2M delegates existing:

- No direct connection between M2M delegates and M2M members in a M2M group

- Direct connection between M2M delegates and M2M members in a M2M group

But regarding to the legacy support issue, Topology A should be considered in 16p with higher priority.

Figure 1. M2M Group Topologies

# M2M group delegate assignment in Idle mode

Before M2M group delegate initiates the ranging access for all M2M devices in this group, BS and other upper level entities will assign this delegate firstly. However, how to assign such a MG delegate (GD) is not defined yet. In the following figure, we described a method to assign M2M group delegate without any dedicate messages .

Figure 2.M2M delegate assignment by PAG_ADV

- Step1.  BSs in a M2M group zone will group multiple M2M devices as one M2M group according to some criteria, e.g. the geographic situations. And MGID information will be delivered to these devices by corresponding signaling, e.g. DSA in 16p.

- Step 2. BS will select one of M2M devices as the group delegate(GD) by some predefined rule.

- Step 3. When BS broadcast the group-based paging message to a M2M group, BS will assign the delegate to this M2M group by this paging message

- Step 4. M2M device in the group which was assigned as GD will precede the following network reentry procures as well as the normal MS does. Otherwise, the M2M devices which are not served as the delegate will reentry the network by reusing the resource which was requested by the delegate.

# Conclusion

In this contribution, we proposed an efficient control signaling mechanism for M2M group delegate assignment which needs any other dedicated signaling.

# Text Proposal

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

Proposal #1 : Insert the following text in Section6.2.18.7. in the latest 16p letter ballot

6.2.18.7.4.3 M2M group delegate assignment

M2M group delegate (GD) could be assigned by the broadcast paging message in Idle mode. The assigned M2M group delegates will respond the corresponding paging messages and initiate the network reentry immediately . The other M2M group members will precede their network reentry with the resource granted by ABS to GD after they received AAI-RNG-ACK confirmation for this group.

Proposal #2 : Revised the following text in Section6.2.3.24  in the latest 16p letter ballot

6.2.3.23 AAI-PAG-ADV Message

Table 706—AAI-PAG-ADV message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ... | ... | ... |  |
| MAC Address Hash | 24 |  |  |
| ... |  |  |  |
| For(i=0;i<Num_MGID;i++){ |  |  |  |
| MGID |  |  |  |
| .... |  |  |  |
| If (Action Code== 0b00 ) { |  |  |  |
| ... |  |  |  |
| GD_Indicator | 1 | 0b0:networkentry based on GD 0b1:GD firstly access network Used to indicate the paged devices is the delegate of M2M group |  |
| } |  |  |  |
| } |  |  |  |
| ... | ... | ... |  |

---------------------------End Text Proposal----------------------------------

# References

- IEEE P802.16p-11/0033

- IEEE 802.16e2009

- IEEE 802.16mD12